| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KOR, 20-28 January, 2011 | Document: JCTVC-D372 WG11 Number: m19148 |
| --- | --- |

| Title: | CE3: Cross check of Toshiba's luma interpolation filter (JCTVC-D154) |  |
| --- | --- | --- |
| Status: | Input Document to JCT-VC |  |
| Purpose: | Cross check report |  |
| Author(s) or Contact(s): | Haricharan Lakshman, | Tel: +49-30-31002 232 Email: haricharan.lakshman@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |

_____________________________

# Abstract

This contribution confirms the test results reported by Toshiba for the interpolation filter proposal in CE3.

# Introduction

Toshiba proposed non-uniform tap length filters (NTLF) in JCTVC-C078.

This method uses shorter tap length of asymmetrical filter for inner quarter-pixel potions. HHI was the cross checker for this proposal. The version we tested uses DCT-IF filter coefficients as in TMuC 0.9 software. For the LC profile 8/4-tap DCT-IF is used and for the HE profile 12/8-tap DCT-IF is used.

# Cross check results

The RD performance as reported by Toshiba matched exactly with our results. The encoding and decoding times were found to be similar. All experiments for all sequences based on the common conditions, JCTVC-B302r2 [1] have been conducted. The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The test summary is given below:

|  | 12/8-tap DCT-IF |  |  | 8/4-tap DCT-IF |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | 0.0 | 0.1 | 1.1 | 0.9 |
| Class B | -0.2 | 0.0 | 0.0 | -1.1 | 1.1 | 1.3 |
| Class C | 0.1 | 0.0 | 0.0 | -4.0 | -0.4 | -0.6 |
| Class D | 0.2 | 0.3 | 0.3 | -8.8 | -3.3 | -3.3 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.1 | -3.7 | -0.5 | -0.5 |
| Enc Time[%] | 98% |  |  | 104% |  |  |
| Dec Time[%] | 94% |  |  | 103% |  |  |
|  | 12/8-tap DCT-IF |  |  | 8/4-tap DCT-IF |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.6 | -0.2 | -0.3 | -2.8 | -0.9 | -1.1 |
| Class C | -0.3 | 0.0 | 0.1 | -4.5 | -3.0 | -3.3 |
| Class D | -0.2 | 0.0 | 0.5 | -8.6 | -7.0 | -7.5 |
| Class E | -0.8 | -0.3 | -0.2 | -2.8 | -1.5 | -1.0 |
| All | -0.5 | -0.1 | 0.0 | -4.6 | -3.1 | -3.2 |
| Enc Time[%] | 99% |  |  | 104% |  |  |
| Dec Time[%] | 95% |  |  | 106% |  |  |

# Reference

[1] T. Chujoh, “CE3: Interpolation for MC (Luma),” Joint Collaborative Team on Video Coding, JCTVC-C503r5, Guangzhou, Oct. 2010.