| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KOR, 20-28 January, 2011 | Document: JCTVC-D373 WG11 Number: m19149 |
| --- | --- |

| Title: | CE4: Cross check of Nokia's chroma interpolation filter (JCTVC-D324) |  |
| --- | --- | --- |
| Status: | Input Document to JCT-VC |  |
| Purpose: | Cross check report |  |
| Author(s) or Contact(s): | Haricharan Lakshman, | Tel: +49-30-31002 232 Email: haricharan.lakshman@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |

_____________________________

# Abstract

This contribution confirms the test results reported by Nokia for the 2 variants of the interpolation filters proposed in JCTVC-D324 in the context of CE4.

This scheme utilizes 1D directional filters for obtaining chroma samples where the chroma sample position is horizontally/vertically or diagonally aligned with integer positions and uses bilinear filter for other positions.

# Cross check results

The RD performance as reported by Nokia matched exactly with our results. All experiments for all sequences based on the common conditions of CE4 have been conducted. The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The test summary is given below:

Variant1:

|  | Random access |  |  |  | Random access LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | 0.0 | -0.6 | -0.7 | -0.2 | 0.0 | -0.5 | -0.6 | -0.2 |
| Class B | 0.0 | -1.2 | -1.8 | -0.5 | 0.0 | -1.6 | -1.8 | -0.5 |
| Class C | 0.0 | -1.2 | -1.1 | -0.4 | 0.0 | -2.7 | -2.5 | -0.9 |
| Class D | 0.0 | -1.0 | -1.2 | -0.4 | 0.0 | -2.9 | -3.6 | -0.9 |
| Class E |  |  |  |  |  |  |  |  |
| All | 0.0 | -1.1 | -1.3 | -0.4 | 0.0 | -2.1 | -2.3 | -0.7 |

|  | Low delay |  |  |  | Low delay LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1 | -0.8 | -1.3 | -0.4 | -0.2 | -2.0 | -3.0 | -0.8 |
| Class C | -0.1 | -1.0 | -1.1 | -0.5 | -0.3 | -2.2 | -2.0 | -1.0 |
| Class D | -0.1 | -1.3 | -0.6 | -0.4 | -0.4 | -2.3 | -2.8 | -1.0 |
| Class E | -0.1 | -1.1 | -1.2 | -0.4 | -0.1 | -0.3 | -0.8 | -0.2 |
| All | -0.1 | -1.0 | -1.1 | -0.4 | -0.3 | -1.8 | -2.3 | -0.8 |

Variant2:

Added 1/8 positions that can be implemented using a single 1D directional filter.

The enc/dec times are not very reliable as the tests were run on a cluster of computers.

# Reference

[1] E. Alshina, “CE4: Interpolation for MC (Chroma),” Joint Collaborative Team on Video Coding, JCTVC-C504, Guangzhou, Oct. 2010.