| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D375 WG11 Number: m19152 |
| --- | --- |

| Title: | LCEC coded block flag coding under residual quadtree |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianglin Wang, Wei-Jung Chen Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-651-5135 xianglin@qualcomm.com wchien@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal presents an improved coding scheme for coded block flag under residual quadtree structure. Based on the proposal, the current joint coding of Y, U and V coded block flag at CU root level is extended in two ways: 1) it further includes transform split flag if available; 2) The joint coding is not only performed at CU root level, but also extended to higher transform depth level so that the benefit of such joint coding can be maximized. Simulation results show that such extension of the joint coding can further improve coding efficiency, especially when there are multiple levels of transform tree decomposition.

- Introduction

According to the current TMuC low complexity entropy coding (LCEC), coded block flags are coded both at coding unit root level and at higher transform depth level. At coding unit root level, coded block flag of Y, U and V components are coded jointly. A coded block flag at this level indicates if a corresponding video component of the coding unit contains any non-zero coefficient. If a coded block flag at coding unit root level is not zero and the transform block size is smaller than the coding unit size, coded block flags at higher transform depth level are further coded. In this case, four coded block flags at a same transform depth level are grouped and coded jointly.

One thing to mention is that the current implementation of CBP coding under LCEC works well with a maximum TU depth level 1 and 2. When the maximum depth level is 3, the performance is found not very well. We did some investigation into the issue after release of TMuC 0.9 and found that it is not because of the CBP signaling mechanism, but the RD not quite matching to CBP signaling. With slight changes in the RD function, coding performance of the current CBP signaling under LCEC works well too for a maximum depth level of 3 or higher.

# Proposed Approach

Based on this proposal, to further improve coding efficiency the joint coding of Y, U and V coded block flag at CU root level is further extended in two ways: 1) transform split flag, when available, is also jointly coded together with the three coded block flags;  2) the joint coding is not only performed at CU root level, but also used at higher transform depth level. If there is only one luma coded block flag left for a block, which always happen for a 4x4 block, the flag is combined with three other luma coded block flags from neighboring blocks and coded jointly.

# Simulation Results

To test the proposed scheme, we used the three low complexity configurations.  In addition to the default low complexity configurations, we also tested the proposed scheme using a maximum TU depth of 3 in the low complexity configurations. The implementation is on TMuC 0.9.

The results with the current TMuC 0.9 and default low complexity configurations are used as anchor. In this case, the maximum TU depth is 1 for intra and 2 for inter.

Three tests are conducted. Under each test, two cases are checked. In case one, the maximum TU depth is 1 for intra and 2 for inter block. In case two, a maximum TU depth of 3 is used for both intra and inter block. In test 1, we disable the two macros LCEC_CBP_YUV_ROOT and QC_BLK_CBP. In this case, the CBP coding falls back to the intial design for residual quadtree. In test 2, we enable the RD fix for the current CBP signaling. In test 3, the proposed CBP coding scheme is used. Detailed results can be found in the associated excel data sheet.

Based on the results, it can be seen that the current CBP signaling with RD fix clearly outperforms the initial residual quadtree CBP signaling. When using the proposed CBP signaling scheme, coding performance can be further improved for the case with a maximum TU depth of 3.

# Conclusion

This contribution presents experimental results on the current CBP coding and a proposed new CBP coding scheme. Based on the results, we recommend that the RD fix for the current CBP coding be put into HM. Meanwhile, we recommend to further test the proposed new CBP scheme in the next LCEC Core Experiment.

# References

[1] X. Wang, M. Karczewicz, W.-J. Chien, “TE4: Report of VLC for coded block flag”, JCTVC-C262, Guangzhou, China, Oct. 2010.

[2] T. Wiegand, H. Schwarz, B. Bross, A. Fuldseth, X. Wang, W.-J. Han, “BoG report: residual quadtree structure”, JCTVC-C319, Guangzhou, China, Oct. 2010.

# Patent rights declaration(s)

Qualcomm Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).