| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D376 |
| --- | --- |

| Title: | CE3: Qualcomm’s proposal for luma interpolation filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | In Suk Chong, Liwei Guo, and Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: 1-858-658-5673 Email: martak@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In this contribution, Qualcomm proposes two variations of mixed tap filters (i.e., 12/8 tap filters) for high efficienct configuration. Simulation results are presented to demonstrate that proposed methods decrease the complexity of the 12 tap filters with similar or even better performance. (0.2% for low delay cases -0.5% for random access cases).

- Introduction

In the CE3 (luma interpolation core experiments), we described our proposal as follows.

“JCTVC-A121 proposes single pass switched interpolation filters with offsets (single pass SIFO) to interpolate the reference frame for the luma component. The basic idea behind switched interpolation filters is that for each of the 15 fractional pixel positions, the interpolation filter can be chosen from a set of interpolation filters associated with that fractional pixel position. For each fractional pixel position, the choice of the interpolation filter as well as offset is signaled at the slice level. Also that proposal mentioned the possibility of using interpolation filters with different tap sizes, based on the fractional pixel location will also be tested. This is useful for reducing the complexity by using longer filters for fractional pixel positions that require 1-D filtering and shorter filters for positions that require 2-D filtering”.

At the Guangzhou meeting, there were many concerns about the high complexity of SIFO. Therefore, efforts were made to reduce the complexity SIFO while keeping reasonable performance. Our first approach is a frame (or slice) level switching DIF/DCT-IF to reduce the complexity of switching operations, which requires one-pass encoder decision using the previous frame. It should be noted that Nokia and Qualcomm have independently arrived to a very similar design except small differences in the encoder algorithm. Therefore that is presented jointly with Nokia (JCTVC-D323).

On the other hand, we also wanted to analyze the possibility of “using longer filters for fractional pixel positions that require 1-D filtering and shorter filters for positions that require 2-D filtering”. Note that Toshiba also introduced one proposal on that direction. Therefore, we want to provide our own proposal which include more methods to explore various possiblities.

# Algorithm description

We first provide a method which is using longer filters for fractional pixel positions that require 1-D filtering and shorter filters for positions that require 2-D filtering (Variation 2). And the second method is the same as Variation 2 except that longer filtering is used for (1/2,1/2) position (Variation 1). Note that Toshiba’s approach can be considered as an extended variation of the second method (Variation 1 + applying longer filtering in more positions that require 2-D filtering). In this proposal, we provide results for high efficiency cases using 12tap DCT-IF filters as a longer filter and 8tap DCT-IF filter as a shorter filter, which is the same as Toshiba’s proposal. Furthermore different types of DCT-IF filters are tested for Variation 1 (i.e., new 12 tap DCT-IF filters and new 8 tap DCT-IF filters) are tested in the framework of Variation 1 (Variation 1_1).

# Complexity Analysis

Compared to HE anchor (DCT-IF 12 taps) complexity numbers are measured for both Variations.

Variation 2

| Operations | Worst case / pixel | 81% |
| --- | --- | --- |
|  | Average of operations / pixel | 77% |
| Instructions | Worst case / block | 82% |
|  | Average instructions / block | 77% |

Variation 1

| Operations | Worst case / pixel | 81% |
| --- | --- | --- |
|  | Average of operations / pixel | 79% |
| Instructions | Worst case / block | 81% |
|  | Average instructions / block | 80% |

Toshiba proposal

| Operations | Worst case / pixel | 81% |
| --- | --- | --- |
|  | Average of operations / pixel | 83% |
| Instructions | Worst case / block | 81% |
|  | Average instructions / block | 83% |

Variation 1_1

| Operations | Worst case / pixel | 85% |
| --- | --- | --- |
|  | Average of operations / pixel | 79% |
| Instructions | Worst case / block | 86% |
|  | Average instructions / block | 79% |

# Results

Results are obtained with new interpolation proposals implemented in TM 0.9 reference software. All results are referenced to anchor in TM 0.9, and are obtained with the recommended simulation common conditions in JCTVC-C500.

## 12/8 tap interpolation filters variation 1

|  | 12/8-tap DCT-IF |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Random access |  |  | Low delay |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | 0.1 |  |  |  |
| Class B | -0.2 | -0.1 | -0.2 | -0.7 | -0.3 | -0.3 |
| Class C | 0.3 | 0.0 | 0.1 | -0.2 | 0.2 | 0.3 |
| Class D | 0.6 | 0.7 | 0.6 | -0.1 | 0.0 | 0.7 |
| Class E |  |  |  | -0.9 | -0.9 | -0.6 |
| All | 0.2 | 0.2 | 0.1 | -0.5 | -0.2 | 0.0 |
| Enc Time[%] | 98% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 95% |  |  |

Compared to Toshiba’s proposal, only negligible loss (avg. 0.05%) is observed.

## 12/8 tap interpolation filters variation 2

|  | 12/8-tap DCT-IF |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Random access |  |  | Low delay |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | 0.1 |  |  |  |
| Class B | -0.2 | -0.1 | -0.2 | -0.6 | -0.1 | -0.4 |
| Class C | 0.4 | 0.2 | 0.1 | 0.0 | 0.4 | 0.3 |
| Class D | 0.8 | 0.6 | 0.6 | 0.1 | 0.4 | 0.8 |
| Class E |  |  |  | -0.7 | -0.4 | -0.1 |
| All | 0.2 | 0.2 | 0.1 | -0.3 | 0.1 | 0.1 |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 94% |  |  |

Compared to Toshiba’s proposal, only negligible loss (avg. 0.2%) is observed.

## Toshiba’s  proposal

|  | 12/8-tap DCT-IF |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | 0.0 |  |  |  |
| Class B | -0.2 | 0.0 | 0.0 | -0.6 | -0.2 | -0.3 |
| Class C | 0.1 | 0.0 | 0.0 | -0.3 | 0.0 | 0.1 |
| Class D | 0.2 | 0.3 | 0.3 | -0.2 | 0.0 | 0.5 |
| Class E |  |  |  | -0.8 | -0.3 | -0.2 |
| All | 0.0 | 0.1 | 0.1 | -0.5 | -0.1 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 94% |  |  | 94% |  |  |

## 12/8 tap interpolation filters variation 1_1

|  | 12/8-tap DCT-IF |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Random access |  |  | Low delay |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.1 |  |  |  |
| Class B | -0.2 | -0.1 | -0.2 | -0.8 | -0.2 | -0.1 |
| Class C | 0.3 | 0.2 | 0.2 | -0.5 | 0.2 | 0.1 |
| Class D | 0.8 | 1.0 | 1.1 | -0.9 | 0.3 | 1.2 |
| Class E |  |  |  | -0.8 | -0.7 | -0.7 |
| All | 0.2 | 0.3 | 0.3 | -0.8 | -0.1 | 0.2 |
| Enc Time[%] | 102% |  |  | 105% |  |  |
| Dec Time[%] | 103% |  |  | 98% |  |  |

# Conclusions

In this contribution, we propose two variations of mixed tap filters (i.e., 12/8 tap filters for high efficienct configuration). Variation 2 is using longer filters for fractional pixel positions that require 1-D filtering and shorter filters for positions that require 2-D filtering. Variation 1 is the same as Variation 2 except using longer filters for (1/2,1/2) position. Simulation results are presented to demonstrate that proposed methods lower the complexity of the 12 tap filters with similar or even better performance. (-0.5%/-0.3% for low delay cases 0.1%/0.2% for random access cases). If Samsung’s new filters are used, coding gain is increased by -0.15%.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).