| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D377 WG11 Number: m19154 |
| --- | --- |

| Title: | Development of HEVC deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin, Kenneth Andersson, Rickard Sjöberg Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes a deblocking filter developed from the current HEVC deblocking filter. The proposed filter outperforms the TMuC0.9 anchor for all six common test configurations with the average BD-rate reduction of 1.2%. The decoding time is 99.8% of TMuC0.9. The subjective performance of the proposal is similar to that of the anchor.

# Introduction

The HEVC deblocking filter shows good subjective and objective performance and is computationally simpler than the H.264 deblocking filter. Despite its merits, the HEVC deblocking filter has limited capacity of reducing ringing artifacts as it modifies the first pixel by delta and the second pixel by delta/2. Therefore, small deviations cannot be efficiently smoothed out. Moreover, the same offsets are used for modifying second pixels from both sides of the block border although the structure may be different. The HEVC deblocking filter always modifies at least two pixels from the block border if the deblocking is turned on by the block based filter decision and the clipping value of the delta is not equal to zero. Moreover, the same number of pixels from the block border is always filtered although HEVC has transform sizes varying from 4 to 32 and prediction unit sizes that vary from 4x4 to 64x64. The proposal  addresses the stated problems without changing the filtering decisions of the current HEVC deblocking filter. The tc and β thresholds have been adapted to the new filter.

Simulation results show that the proposal decreases that average BD-rate by 1.2% for all six configurations Error: Reference source not found. The computational complexity of the decoder is 99.8% of the TMuC0.9.

# Algorithm description

The proposed algorithm consists of a few modifications to the HEVC deblocking filter. These modifications are: a modified deblocking filter, the filter length decisions and the introduction of block size adaptation. The tc and β thresholds have also been adjusted.

## Filter description

The proposal filters input pixels according to the following formulas.

 = (9*(q0-p0)-3*(q1-p1))/16

p0’ = p0 + 

q0’ = q0 - 

p = (p0 - 2*p1+ p2 +2*)/4;

p1’ = p1 + p

q = (q0 - 2*q1+ q2 -2*)/4;

q1’ = q1 + q

The proposed filter is a stronger low-pass filter than HEVC and attenuates small ripples better than the HEVC filter. When applied to a signal in the form of a slope (10 20 30 40 50 60), the filter would not modify the signal, since , p and q will all be equal to zero. When the proposed filter is applied to an unfiltered input signal that has a form close to that of a step function (a deblocking artefact), the filter will smooth it. These are desirable features of the HEVC deblocking filter that is kept in this proposal.

## Filtering decisions

The filter uses additional conditions to control the filter. The pixels p1 and q1 (at the second position from the block border) are modified if abs() < (tc*8). The delta is equal to zero if the signal on the sides of the block border forms a line and increases with the size of a blocking artifact. This condition helps to differentiate blocking artifacts from natural edges. It also reduces computational complexity of the filter as p and q are calculated less often.

Another filter decision limits the filtering strength if there are no blocking artifacts. If abs() < thr1 then the second pixel from the block border is not filtered. This condition limits the amount of filtering applied to the second pixel if there are no blocking artifacts (the first delta is close to zero). The value of 2 is used for thr1. This condition limits the amount of low-pass filtering that is applied to the frames over time.

## Block size adaptation

The proposed filter modifies different number of pixels depending on the block sizes of the adjacent blocks. The smaller block size of the blocks adjacent to the block border is used for the decision. The block size is determined as the minimum of the TU and PU sizes. When the block size is equal to 4, only 1 pixel from the block border is modified. For blocks of size 8, up to two pixels from the block border are modified. The blocks of size 16 and larger can be filtered with up to 3 pixels from the block border if the strong filter is used.

## Beta table adjustment

Beta table has been adjusted for the new filter. The old and the new tables are compared in Figure 1.

.

Figure 1 Beta table adjustments.

# Compression performance

The compression performance of the proposed method can be found in the table below. One should notice that the simulation time is wrong as the simulations have been done on a Linux cluster. The decoding time on a single computer is given in the next section. The proposed scheme has been compared against the TMuC0.9 anchor. The comparison results are given in Table 1. One can see that the proposal shows an average BD-rate [2] reduction of 1.2%.

Table 1 Comparison of proposed deblocking vs TMuC0.9 anchor

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.8 | -0.5 | -0.5 | -1.2 | -0.5 | -0.4 |
| Class B | -1.2 | -0.6 | -0.5 | -0.9 | -0.6 | -0.5 |
| Class C | -1.0 | -0.7 | -0.7 | -0.9 | -0.7 | -0.6 |
| Class D | -0.9 | -0.7 | -0.7 | -0.8 | -0.6 | -0.6 |
| Class E | -1.2 | -0.1 | -0.1 | -0.8 | 0.0 | 0.0 |
| All | -1.1 | -0.5 | -0.5 | -0.9 | -0.5 | -0.5 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 108% |  |  | 109% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.8 | -0.4 | -0.3 | -1.4 | -0.5 | -0.4 |
| Class B | -1.1 | -0.5 | -0.6 | -1.1 | -0.5 | -0.6 |
| Class C | -0.9 | -0.5 | -0.5 | -0.8 | -0.5 | -0.5 |
| Class D | -0.7 | -0.3 | -0.3 | -0.6 | -0.7 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -1.0 | -0.5 | -0.5 | -0.9 | -0.5 | -0.4 |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 116% |  |  | 117% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.6 | -0.5 | -0.7 | -1.7 | -0.3 | -0.3 |
| Class C | -1.4 | -0.1 | -0.2 | -1.3 | -0.1 | -0.1 |
| Class D | -0.9 | -0.4 | -0.3 | -1.2 | -0.1 | -0.1 |
| Class E | -3.7 | -0.4 | -0.5 | -2.8 | -0.6 | -0.7 |
| All | -1.8 | -0.4 | -0.4 | -1.7 | -0.2 | -0.3 |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 113% |  |  | 112% |  |  |

# Complexity considerations

In order to evaluate the complexity of the proposed deblocking, the decoding time on a single machine has been measured for both the proposal and the anchor. The time has been obtained without writing decoded pictures on the disk in order to minimize the influence of the I/O time.

Table 2 Decoding time of the proposal vs TMuC0.9 decoding time

| Configuration | Dec. time TMuC0.9 [s] | Dec time proposal [s] | Dec. time. proposal [%] |
| --- | --- | --- | --- |
| Random access | 2625.72 | 2619.48 | 99.8% |
| Low delay | 2946.05 | 2908.19 | 98.7% |
| All intra | 5835.03 | 5846.96 | 100.2% |
| Random access loco | 845.41 | 848.01 | 100.3% |
| Low delay loco | 947.26 | 946.62 | 99.9% |
| All intra loco | 2077.34 | 2079.52 | 100.1% |
| Average |  |  | 99.8% |

Table 1 shows that the proposal has a lower complexity compared with the TMuC anchor.

# Conclusions

The contribution describes a deblocking filtering method that is a further development of the HEVC deblocking filter. The main differences compared with the current HEVC deblocking are that the filter itself is improved and that a new filter criteria for modification of the second pixel from the block border is introduced and that the number of modified pixels are adapted to the block sizes. The filter achieves a BD-rate reduction of 1.2% on average for the 6 test configurations while at the same time reducing the decoder complexity slightly. The subjective performance of the proposal is similar to TMuC0.9. We suggest adoption of this proposal to the next version of HM.

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, JCT-VC Meeting, Guangzhou, Oct. 2010.

- Gisle Bjontegaard, “Calculation of average PSNR differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).