| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D378 |
| --- | --- |

| Title: | Generalized slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Michael Horowitz 1450-1055 West Hastings Street Vancouver, BC Canada V6E 2E9 Shilin Xu 1450-1055 West Hastings Street Vancouver, BC Canada V6E 2E9 | Tel: Email: Tel: Email: | +1 778.331.3432 x323 michael@ebriskvideo.com +1 778.331.3432 shilin@ebriskvideo.com |
| Source: | eBrisk Video, Inc. |  |  |

_____________________________

# Abstract

This contribution presents generalized slices (GS). GS is a video coding construct that provides more options for partitioning a video picture compared to standards with less flexible picture partitioning (e.g., H.264/AVC with the number of slice groups equal to zero).  Specifically, GS introduces vertical slice boundaries that partition a picture into columns in a manner reminiscent of Rectangular Slices (H.263 Annex K). Introduction of vertical slice boundaries enables an encoder to encode slices that have a lower normalized moment of inertia (i.e., they have a more square shape) which typically results in greater homogeneity of content within a slice and increases the ratio of slice area to slice boundary leading to improved coding efficiency. Coding units are processed in raster scan order within a column. Results are presented demonstrating that GS provides an average 2.0%, 1.3%, and 1.6% Y BD-rate advantage for Random Access, Low Delay, and Intra cases respectively compared with an identically configured encoder using the same number of scan-order slices.

# Introduction

Partitioning a picture into slices as part of the encoding process is known to negatively impact coding efficiency particularly when the slices are designed to be independently decodable. Many applications and implementations however require the partitioning of a picture. Some examples are described in the bullet items below.

- Parallel processing: some implementations such as those executed on modern multi-core CPUs, partition a source picture into slices and send each slice to a separate core to be encoded in parallel. High quality real-time encoding of high definition video (e.g., 1280x720 and larger) would not be possible today on a general-purpose multi-core CPU without partitioning and parallel encoding. In addition, to reduce costly information sharing between cores during the encoding/decoding process, it is typically advantageous for slices to be coded independently.

- MTU size matching: when transporting a coded bitstream on a packet switched network (e.g., an IP network), packets are subject to a maximum transmission unit (MTU) size.  If a packet contains many fewer bits than the MTU size, then packet header bit overhead can significantly affect coding efficiency. On the other hand, if a packet contains more bits than the MTU size, the network will fragment the packet. A lost packet fragment results in an error resilience problem since the entire packet is unrecoverable if one fragment is lost. One way to avoid packet fragmentation is to send bits from large slices in separate packets each smaller than the MTU size. This scheme has the unfortunate disadvantage that if one packet is lost, the large slice is undecodable and is therefore lost in its entirety. In contrast, matching an independently decodable slice to the MTU size of the network has the advantage that all received packets can be decoded – a property that has great practical significance in real-world systems.

- Error resilience: some applications partition pictures into independently decodable slices and apply unequal error protection techniques to protect slices deemed more important.

Given the prevalence of applications requiring picture partitioning with independently decodable slices, it is natural to consider techniques that provide the required functionality while minimizing coding efficiency loss.

In this contribution, it is shown that an encoder using generalized slices (GS) will in general achieve higher coding efficiency compared with an identically configured encoder using scan-order slices. GS introduces vertical slice boundaries that partition a picture into columns in a manner reminiscent of Rectangular Slices (H.263 Annex K) [1]. Coded Tree Blocks (CTBs) are processed in raster scan order within a column (see Figure 1). Introduction of vertical slice boundaries enables an encoder to encode slices that have a lower normalized moment of inertia (i.e., a more square shape) which can result in greater homogeneity of content within a slice and increases the ratio of slice area to slice boundary leading to improved coding efficiency.

Figure 1. An example of generalized slice partitioning using three columns.

Earlier techniques such as slice groups [2] in H.264/AVC [3] and Rectangular Slices in H.263 may also be used to reduce efficiency losses due to partitioning.  While slice groups can provide similar functionality to GS, GS achieves the desired functionality in a much simpler way significantly reducing implementation complexity. Compared with Rectangular Slices, GS relaxes the rectangular shape requirement allowing the encoder to produce packets that are closer to the MTU size than it could if it were constrained to produce a rectangular slice. The ideal packet payload size is such that the payload plus packet header is exactly equal to the MTU size. One byte more and the packet will be fragmented, significantly less and header overhead becomes a problem. In practice, packet termination does not typically occur at a pre-defined slice boundary, rather encoders designed for video transport on packet switched networks terminate a packet at the Nth coding unit (e.g., the Nth macroblock or CTB) if the bits used to encode coding unit N+1 would cause the packet size to exceed the MTU. In addition, for many applications column boundaries are unlikely to change from picture to picture so indicating the boundaries in the PPS avoids the bit rate overhead associated with sending widths for every slice (as is done in Rectangular Slices).

This contribution is organized as follows. Section 2 contains a description of GS syntax, semantics, and decoding process.  In Section 3, the experiment is described.  Section 4 contains a summary of experimental results. The conclusion is contained in Section 5 and references in Section 6.

# GS syntax, semantics, and decoding process

To implement GS, three decoder modifications were made to the TMuC 0.9-AhG slices [4] source code:

- Addition of the functionality to decode columns widths and compute column boundary locations,

- Modification of the decoding order from raster scan within a picture to raster scan within a column,

- Modification of CU availability to recognize that CUs in a different column are not available.

Each of these modifications is discussed in the sub-sections that follow. Analogous changes were made to the encoder source code.

## Column widths and boundaries

The picture parameter set syntax was modified to convey vertical column boundaries.  Specifically, a syntax element indicating the number of columns minus 1, num_columns_minus1, was added followed by the indication of num_columns_minus1 column widths as shown below. This is the only syntax modification required to implement GS.

### GS Syntax

The following syntax was added to the picture parameter set.  A fixed length code was used for columnWidth[] (12 bits) and num_columns_minus1 (8 bits).  Ultimately, the code length will be level dependent.  Further, we envisage a normative level-dependent minimum column width specification to prevent vertically elongated slices.

num_columns_minus1

for (i=0; i < num_columns_minus1; i++)

columnWidth[i]

### GS Semantics

The syntax element num_columns_minus1 indicates the number of column boundaries (i.e., the number of columns minus one) in the picture.  The syntax element columnWidth[i] specifies the width of the ith column in units of CTBs.  The last column width equals the number of CTBs between the right-most column boundary and the right picture edge. The decoder can use the columnWidth[] values to derive the column boundaries as shown below.

if (num_column_minus1 > 0) {

columnBoundary[1] = columnWidth[0];

for (i=1; i < num_columns_minus1; i++) {

columnBoundary[i+1] = columnBoundary[i] + columnWidth[i];

}

}

## Decoding order

The decoding order for GS was modified so that CTBs are decoded in raster scan order within columns rather than raster scan order within the picture as is done in TMuC 0.9-AhG slices. In addition, rather than encoding all slices in column 1 followed, by all slices in column 2, and so on, after the last CTB of a slice has been encoded, the encoder will start encoding the slice whose first CTB address is smallest of all unencoded CTBs. This encoding order can, in certain architectures, reduce encoding latency.

Figure 2.  CTB decoding order for a picture encoded with 3 columns.

## CU availability

CU availability for GS is the same as that for AhG slices except that CUs in a different column are not available.  Note that column boundaries are slice boundaries.

# Description of the experiment

The introduction discussed several motivations for introducing slices.  In this experiment we focus on CPU high-level parallelization. To simulate CPU load balancing (to a zero-order approximation), we have chosen slices that contain approximately the same number of CTBs. The common conditions, slice partitioning, and equipment used to conduct the experiment are detailed in the sub-section that follow.

## Common conditions

The reference software TMuC 0.9-AhG Slices revision 413 was used to produce results for the reference in the experiment.  In addition, the encoder and decoder source code from the reference codebase was modified to introduce GS functionality. The common conditions and encoding configurations [5] were used without modification except:

- Generalized slices and associated parameters were introduced,

- Roughly one third of the frames were encoded due to a lack of time (the exact number of frames encoded for each sequence is included in the spreadsheet accompanying this document), and

- Class D was not tested since we believe it unlikely that pictures of this size would require more than one slice to encode in practice.

## Slice partitioning

The slice sizes for each sequence class except Class C were chosen so as to contain roughly the same number of pixels as one picture in a Class D sequence. To introduce a degree of variability to the experiment, the slices in Class C were chosen to be approximately twice the size of slices in the other classes. As mentioned earlier, Class D pictures were not included in the experiments since for most applications pictures of that size would likely be coded in a single slice. In the reference case, CTBs were assigned to each slice in raster scan order within the picture.  The number of CTBs per slice was kept constant when possible. In accordance with the common conditions, each CTB contained 64x64 pixels. A detailed description of the slice partitioning for each picture class is presented in the following sub-sections and in Figure 3.

### Class A (2560x1600)

Pictures in Class A were partitioned into 40 slices with 25 CTBs in each slice.  In the GS case, each picture was partitioned into 8 columns and 5 rows and each slice had dimensions of 5-by-5 CTBs. In the AhG slice case, each slice contained 25 CTBs in raster scan order.

### Class B (1920x1080)

Pictures in Class B were partitioned into 18 slices. In the GS case, each picture was partitioned into 6 columns and 3 rows. Twelve slices had dimensions of 6-by-5 CTBs and six slices were 5-by-5 CTBs. In the AhG slice case, seventeen slices contained 28 CTBs and one slice contained 34 CTBs in raster scan order.

### Class C (832x480)

Pictures in Class C were partitioned into 2 slices. In the GS case, each picture was partitioned into 2 columns. One slice had dimensions of 6-by-8 CTBs and the other 7-by-8. In the AhG slice case, both slices contained 52 CTBs in raster scan order.

### Class E (1280x720)

Pictures in Class E were partitioned into 8 slices with 30 CTBs in each slice.  In the GS case, each picture was partitioned into 4 columns and 2 rows and each slice had dimensions of 5-by-6 CTBs. In the AhG slice case, each slice contained 30 CTBs in raster scan order.

Figure 3. GS slice partitions with column boundaries indicated as thick vertical lines.

## Equipment

The GS simulations were executed on an Acer PC with the following system configuration:

CPU: Intel Core i7 870 @ 2.93GHz x 4 cores
RAM: 4 GB
OS:  Windows 7 Home Premium.

The AhG slice simulations were executed on a Hewlett Packard PC with the following system configuration:

CPU: Intel Core i7 870 @ 2.93GHz x 4 cores
RAM: 4 GB
OS:  Windows 7 Home Premium.

# Results

The results of the experiment comparing GS to AhG slices are summarized in Table 1 and presented in more detail in the accompanying spreadsheet. GS provides coding gains for all sequences in all test cases.

Table 1. Experimental results summary*.

As is apparent from Table 1, the highest gains on average were found in the Random Access test case.  Similar, but slightly lower gains were found in the Low Delay and Intra cases.  High efficiency and Low Complexity results were on average very similar to one another across the Random Access, Low Delay, and Intra test conditions.  Class B showed the highest gains and the largest efficiency gain was due to Class B’s BaskeballDrive with a Y BD-Rate [6] gain at 5.4% in the High Efficiency Random Access case. Interestingly, despite the fact that Class C had a single slice boundary (horizontal for scan order slices and vertical for GS), it showed small but consistent gains. The GS and AhG slice simulations were executed on different hardware consequently encode and decode times are not directly comparable. That said, the two machines have similar capabilities.  A spot check run of GS slices on the same hardware used to generate the AhG slice results suggested that the encode and decode time differences in our results can for the most part be attributed to small differences in processing capabilities. In any case, the measured encode/decode times for GS and AhG are very similar.

We hypothesize that the coding efficiency gains for GS result in part from the fact that the AhG slice reference has more slice boundaries (hence more broken prediction mechanisms) compared with GS. Table 2 shows the AhG:GS slice boundary ratios for each of the sequence classes in the experiment. Except for Class A, a higher AhG:GS ratio correlates to a larger GS gain. The lower gain for Class A suggests that the content of the sequence also plays a role in determining GS gains.  Finally, we note that in our experiments the GS slice boundaries were determined somewhat arbitrarily and an encoder designed to more optimally partition pictures based on content would likely achieve superior GS gains.

|  | Class A | Class B | Class C | Class E |
| --- | --- | --- | --- | --- |
| AhG:GS Slice Boundary Ratio | 2.7 | 2.8 | 1.2 | 2.1 |

Table 2. AhG:GS slice boundary ratios for each sequence class.

# Conclusion and recommendation

Given the prevalence of applications requiring picture partitioning with independently decodable slices, it is natural to consider techniques that provide the required functionality while minimizing coding efficiency loss. In all experimental cases, GS demonstrated higher coding efficiency compared with an identically configured encoder using the same number of scan-order slices. In addition, GS adds no computational complexity and is simple to implement. Further, we believe GS is complementary to the fine granularity slices [7] and entropy slices [8] ideas that have been proposed in earlier meetings and the combination of tools might prove advantageous for some applications.

We recommend that the JCT-VC continue the evaluation of Generalized Slices for possible adoption into the HM. Specifically, we recommend verification of this experiment and harmonization with other slice related tools.

# References

[1] ITU-T Recommendation H.263 version 2 (H.263+), "Video coding for low bit rate communication” (1998).

[2] S. Wenger, M. Horowitz, “Flexible Macroblock ordering (FMO)”, JVT-C089, May 2002, available from ftp://ftp.imtc-files.org/jvt-experts/2002_05_Fairfax/JVT-C089.doc.

[3] ITU-T Recommendation H.264 and ISO/IEC 14496-10, Advanced Video Coding for Generic Audiovisual Services (03/2010).

[4] JCT-VC, “Test Model under Consideration Software 0.9-ahg-slices”,

https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/branches/0.9-ahg-slices/

[5]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-B300, Geneva, July 2010.

[6] G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

[7] R. Sjoberg, P. Wennersten, “Fine granularity slices”, JCT-VC Document, JCTVC-C154, Guangzhou, October 2010.

[8] K. Misra, J. Zhao, and A. Segall, “Entropy slices for parallel entropy coding”, JCT-VC Document, JCTVC-C256, Guangzhou, October 2010.

# Patent rights declaration(s)

eBrisk Video, Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The decoder cannot in general take advantage of high-level parallelization unless the encoder is constrained to use parallelization since the decoder must be capable of decoding any conformant bitstream it receives. While outside the scope of the HEVC specification and the standards that develop from it, it is possible to constrain encoders to facilitate high-level parallelization at the decoder.

*Encoding and decoding for the generalized slice and AhG slice reference were performed on different (but very similar) machines.