| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D379r1 |
| --- | --- |

| Title: | CE8 Subset1: Cross verification of Mediatek’s deblocking filter by Ericsson |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross verification |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Andrey Norkin, Rickard Sjöberg Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 7141538 kenneth r andersson @ericsson com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution is a cross-verification of Mediatek’s deblocking filter proposed in document C142 [1] according to CE8 Sub-test 1 [3]. The source-code has been studied and found to have been re-designed compared to Guangzhou meeting C142 but corresponds to D163 [6]. The decoding time and the coding efficiency numbers match those of Mediatek except for PSNR for some sequences and QPs. Different mismatches than in Toshibas cross verification in D203. The pure deblocking time for all intra is increased with about 30% compared to TMuC 0.9 measured on the same platform. Visual inspections at low bitrates for random access and low-delay configurations show more blocking artifacts than TMuC 0.9 [5]. Visual inspections for all intra coding configurations show similar performance as TMuC 0.9.

# Objective results

The results were obtained according to common test conditions [2] and conditions defined in sub-test1 of by CE8: In-loop filtering [3].

## Encoding and decoding performance on linux cluster

The encoder and decoder were compiled on Linux using gcc 4.3.3. To be sucessful it was needed to modify some lines of code in TComLoopFilter.h, e.g. removing “TComLoopFilter::” from line 131-137. The simulations were performed on a Linux cluster using processors with different capabilities and the encoding and decoding time measures are therefore not reliable. The simulations have been performed for all 6 configurations defined in [2] and the additional configuration defined in CE8 Sub-test 1 [3].

The PSNR, bitrate and BD-rate match those of Mediatek except for PSNR numbers for some sequences and QPs. The results do not either match perfectly to Toshibas cross verification in [7].

Table I Results of cross-verification of Mediatek’s loop filter compared to TMuC 0.9 [5]

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1,7 | -0,4 | -0,4 | -1,2 | -0,8 | -0,7 |
| Class B | -1,1 | -0,7 | -0,9 | -0,9 | -1,0 | -1,0 |
| Class C | -0,9 | -0,8 | -0,8 | -0,7 | -1,1 | -1,1 |
| Class D | -0,7 | -0,5 | -0,4 | -0,5 | -1,1 | -0,7 |
| Class E |  |  |  |  |  |  |
| All | -1,0 | -0,6 | -0,6 | -0,8 | -1,0 | -0,9 |
| Enc Time[%] | 106% |  |  | 105% |  |  |
| Dec Time[%] | 134% |  |  | 133% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1,5 | -0,3 | -0,2 | -1,6 | -0,3 | -0,3 |
| Class C | -1,3 | 0,0 | -0,1 | -1,3 | -0,4 | -0,1 |
| Class D | -0,9 | -0,6 | 0,1 | -1,2 | 0,0 | -0,1 |
| Class E | -2,9 | -0,6 | -0,1 | -2,3 | -0,1 | -0,5 |
| All | -1,6 | -0,4 | -0,1 | -1,6 | -0,2 | -0,2 |
| Enc Time[%] | 106% |  |  | 104% |  |  |
| Dec Time[%] | 135% |  |  | 130% |  |  |
|  | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2,2 | -0,7 | -0,6 | -2,1 | -0,5 | -0,4 |
| Class B | -1,4 | -0,9 | -0,6 | -1,5 | -0,9 | -0,7 |
| Class C | -1,1 | -1,1 | -0,9 | -1,3 | -0,9 | -0,7 |
| Class D | -1,0 | -1,0 | -0,9 | -1,2 | -0,7 | -0,5 |
| Class E | -1,5 | 0,2 | 0,1 | -1,7 | 0,5 | 0,5 |
| All | -1,3 | -0,7 | -0,6 | -1,5 | -0,6 | -0,4 |
| Enc Time[%] | 102% |  |  | 104% |  |  |
| Dec Time[%] | 118% |  |  | 120% |  |  |

## Decoding time on same platform

The average decoding time match Mediatek’s reported number. In order to compare the complexity of TMuC deblocking and Mediatek’s deblocking, the following experiment has been done. The decoder without deblocking function has been obtained by commenting out call for deblocking function in the TDecGop.cpp file.

// m_pcLoopFilter->loopFilterPic( rpcPic );

Then, the decoding time of TMuC 0.9 decoder and decoding time of Mediatek’s proposal has been compared to the decoding times obtained with decoders without deblocking on the TMuC bitstreams and Mediatek’s bitstreams respectively. The difference between the decoding time of the TMuC (or Mediatek’s) decoder and the deblocking time of the decoder without deblocking on the same bitstream shows the pure time spent on deblocking.

The results have been obtained on the computer with Intel T8100 2.1 GHz processor and with writing of reconstructed frames turned off. The results indicate that Mediatek has 25% more deblocking time than TMuC 0.9. This is larger than Mediatek reports.

Table II Comparison of deblocking time of Mediatek and TMuC for coding conditions in [2]

| Configurations | TMuC deblocking time (s) | Mediatek’s deblocking time (s) | Deblocking time (%) |
| --- | --- | --- | --- |
| All_Intra loco | 406,27 | 541,59 | 33,31 % |
| All_Intra | 413,1 | 522,99 | 26,60 % |
| Low delay loco | 192,02 | 197,7 | 2,96 % |
| Low delay | 151,76 | 223,06 | 46,98 % |
| Random access loco | 162,13 | 170,05 | 4,88 % |
| Random access | 123,72 | 177,79 | 43,70 % |
| Average | 244 | 306 | 25 % |

# Subjective results

Visual inspection at reconstructions at low bitrates found the proposal to have more blocking artifacts than TMuC 0.9 for random access and low-delay coding configurations. For all intra coding the visual  quality was found to be similar as for TMuC 0.9. The monitor used for the subjective inspection was HP LP2475w.

# Source code evaluation

The source-code has been studied and found to have been re-designed compared to previous versions. The source code corresponds to [6].

## Luma filtering

For luma filtering block based filter decision is applied for block borders not smaller that 8x8 similar as for TMuC but with different criteria for the block based decision than TMuC [5]. The thresholds used for the filter decision are different if one of the block at the block boundary is intra compared to when both are inter. In total four tables with thresholds dependent on QP are used for this.

In addition to TMuC the intra modes are analyzed when both blocks are intra. From the possible 34 modes a quantization into 9 modes is done similar as for MDDT in TMuC 0.7. If both blocks have DC mode a MDDF type of 18 is assigned this is also the case if one of the blocks is inter. If both blocks have the same mode but the mode is parallell to the block border the MDDF type is set to 20. If both blocks have the same mode but not DC or a mode parallell to the block border the MDDF type is set to the intra mode for horisontal edges and intra mode + 9 for vertical edges. If the intra modes are different the MDDF type is set to 19. If both blocks are inter the MDDF type is 21.

If the block based decision is positive and at least one of the blocks at the block border is intra each line of the block border is checked for strong or weak filtering. The decision criteria for strong or weak filter  is different from TMuC. The strong filter is same as in TMuC. If both blocks are inter the strong filter is not used at all. This is different from TMuC.

The weak filter is different from TMuC and for MDDF type 0-18 and 21 it also have two additional criterias to decide to filter two pixels or one pixel on respective side of the block boundary such checks are not used in TMuC since TMuC always filter two pixels on each side of the block boundary. To obtain a filtered value the same principle as TMuC is used e.g. using delta and delta/2.  The delta is computed with one multiplication less and two additions less than for TMuC. For MDDF type 19-20 only one pixel is filtered on each side of the block boundary. The clipping table is different from TMuC and the clipping threshold depends not only on QP but also on MDDF type.

In case of MDDF type between 0 and 17 and when weak filter is decided by strong/weak filter decision deblocking filtering in the direction of the intra mode is performed. First two tap interpolation to obtain pixel values in positions according to the intra mode and centered on the pixel closest to the block border on one side and then filter the interpolated values to obtain a delta. The delta is used to obtain deblocked value for that pixel and the second pixel on the same side of the block boundary. Next two tap interpolation centered on the pixel closest to the block border on the other side and then filter the interpolated values to obtain a new delta that is used for obtaining filtered values on this side of the block boundary. The directional filtering is not only cumbersome in number of operations for interpolation and filtering but also introduce dependency across the horizontal block boundary when filtering vertical block boundary.

The filtering is performed according to the MDDF type for respective 4x4 patch of the 8x8 block borders so that 4x4 intra modes can be exploited to some extent.

## Chroma filtering

The chroma filter kernel is changed and another clipping table is used compared to TMuC 0.9.

# Conclusion

Visual inspection of reconstructed video indicate more blocking artifacts for the Mediatek’s deblocking filter than for TMuC 0.9 for low bitrates for both random access and low delay configurations. The complexity when both blocks are intra and fulfills criterions for directional filtering is quite heavy. The pure deblocking time is increased by about 30% compared to TMuC 0.9 for all intra.  In addition directional filtering introduces dependencies across the horizontal block boundary when filtering vertical block boundary.

# References

- Y.-W. Huang, C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y. Gao, J. An, K. Zhang, S. Lei, “In-loop adaptive restoration”, JCTVC-C142, Guangzhou, China, October 2010.

- F. Bossen, “Common test conditions and software reference configurations” JCTVC-C500, Guangzhou, China, October 2010.

- T. Yamakage, T. Chujoh, S. Lei, K. Chono, M. Karczewicz, A. Segall, T. Yamamoto, B. Jeon, J. Lim, J. Xu, “Description of CE8: In-loop filtering”, JCTVC-C508, Guangzhou, China, October 2010.

- G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- “Test Model under Consideration”, ITU-T SG16 WP3 document, JCTVC-B205

- Y.-W. Huang, C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y. Gao, J. An, K. Zhang, S. Lei, “In-loop adaptive restoration”, JCTVC-D163, Daegu, Korea, January 2011.

- T. Yamakage, “CE8 Subset1: Verification results of MediaTek's Proposal JCTVC-D163”, JCTVC-D203, Daegu, Korea, January 2011.