| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D381 |
| --- | --- |

| Title: | CE5: Cross-check of Qualcomm’s Intra mode coding for LCEC by Sony |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N 1st Street San Jose, CA 95112 | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This report presents cross-checking test results on Intra mode coding in LCEC proposed by Qualcomm [JCTVC-C263] under the test environment defined by CE5.

# Introduction

In Guangzhou meeting, some improvements in Low Complexity Entropy Coding (LCEC) were reported in various contributions. The Core Experiment 5 (CE5) [1] evaluates all such techniques. In this report, Sony provides cross-checking results for Qualcomm’s Intra mode coding in LCEC [2].

# Cross-checking results

## Coding conditions

The conditions are the same as specified in JCTVC-C500 for low complexity settings.

## Test set

- The same set as specified in JCTVC-C500 is used.

## Evaluation criteria

The criteria are to report the coding efficiency improvement relative to the reference software and to evaluate the coding complexity.

## Results from Qualcomm

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -0.2 | 0.0 |
| Class B | -0.6 | -0.3 | -0.1 |
| Class C | -0.6 | -0.1 | -0.1 |
| Class D | -0.3 | 0.0 | 0.0 |
| Class E | -0.4 | 0.0 | 0.3 |
| All | -0.5 | -0.1 | 0.0 |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | #NUM! |  |  |
|  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | 0.0 | 0.2 |
| Class B | -0.2 | -0.1 | 0.0 |
| Class C | -0.2 | 0.1 | 0.2 |
| Class D | -0.1 | 0.2 | -0.1 |
| Class E |  |  |  |
| All | -0.2 | 0.1 | 0.1 |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.1 | 0.0 | -0.1 |
| Class C | 0.0 | -0.1 | 0.0 |
| Class D | 0.0 | 0.5 | 0.3 |
| Class E | -0.1 | -0.1 | 0.3 |
| All | -0.1 | 0.1 | 0.1 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | #NUM! |  |  |

## Results from Sony

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -0.2 | 0.0 |
| Class B | -0.6 | -0.3 | -0.1 |
| Class C | -0.6 | -0.1 | -0.1 |
| Class D | -0.3 | 0.0 | 0.0 |
| Class E | -0.4 | 0.0 | 0.3 |
| All | -0.5 | -0.1 | 0.0 |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | 0.0 | 0.2 |
| Class B | -0.2 | -0.1 | 0.0 |
| Class C | -0.2 | 0.1 | 0.2 |
| Class D | -0.1 | 0.2 | -0.1 |
| Class E |  |  |  |
| All | -0.2 | 0.1 | 0.1 |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.1 | -0.1 | -0.1 |
| Class C | 0.0 | -0.1 | 0.0 |
| Class D | 0.0 | 0.5 | 0.3 |
| Class E | -0.1 | -0.1 | 0.3 |
| All | -0.1 | 0.1 | 0.1 |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |

# Conclusion

In this report, cross-checking results from Sony for Intra mode coding match those from Qualcomm [2] in BD results. However, the timing information is not available.

# References

- X. Wang, “Low Complexity Entropy Coding Improvements”, JCTVC-C505_r1, Oct. 2010.

- Qualcomm, “Improvements on VLC”, JCTVC-C263, Oct. 2010.