| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D388 |
| --- | --- |

| Title: | CE11: Cross-verification of Samsung's low-complexity adaptive coefficients scanning (JCTVC-C205) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution provided cross checking results of CE11 on adaptive transform coefficients scanning (ACS)  from  JCTVC-C205.  No results were exchanged with the proponent of JCTVCV-C205. JTCVC-C205 provided 1.2%, 0.5% and 0.6% improvements in high efficiency intra-only, random access, and low-delay configurations, respectively. Low complexity configurations were not tested as requested by the proponent.

# Introduction

Received source code TMuC-0.9-CE11-JCTVC-C205.zip from the proponent on Dec 23, 2010. Since the proposed method with LCEC entropy coding was not ready at that time, the proponent requested to perform cross checking on the HE configuration only.

No cross check result was exchanged with the proponent of JCTVC-C205 [2].

# BD Results

The anchor results were provided by the  CE11 coordinator [1].  Since this cross-checking was performed on computers with different configurations, no timing information was provided with this cross-checking. The % significant count was also reported  in accordance with CE11.

|  | Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | -0.7 | -1.3 | -1.1 | 2.38% |
| Class B | -0.9 | -1.8 | -1.8 | -10.49% |
| Class C | -1.3 | -1.6 | -1.5 | -6.10% |
| Class D | -1.2 | -1.5 | -1.5 | -8.26% |
| Class E | -1.8 | -1.8 | -2.2 | -8.53% |
| All | -1.2 | -1.6 | -1.6 | -6.20% |
| Enc Time[%] | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |

|  | Random access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | -0.3 | -0.5 | -0.6 | -0.25% |
| Class B | -0.6 | -1.1 | -1.1 | -11.29% |
| Class C | -0.5 | -0.9 | -0.9 | -11.09% |
| Class D | -0.5 | -0.9 | -0.5 | -10.76% |
| Class E |  |  |  |  |
| All | -0.5 | -0.9 | -0.8 | -8.35% |
| Enc Time[%] | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |

|  | Low delay |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A |  |  |  |  |
| Class B | -0.7 | -0.8 | -0.8 | -11.89% |
| Class C | -0.5 | -0.7 | -0.7 | -13.21% |
| Class D | -0.5 | -1.5 | -0.3 | -11.68% |
| Class E | -0.4 | -0.6 | -2.1 | -19.40% |
| All | -0.6 | -0.9 | -0.9 | -14.04% |
| Enc Time[%] | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |

# References

- JCT-VC, “Low-complexity adaptive coefficients scanning”, JCTVC-C205, JCT-VC Meeting, Guangzhou, CN, 7-15 October, 2010.

- JCT-VC, “CE11: Coefficient scanning and coding”, JCTVC-C511, JCT-VC Meeting, Guangzhou, CN, 7-15 October, 2010.