| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D384 |
| --- | --- |

| Title: | Quantization with Hard-decision Partition and Adaptive Reconstruction Levels for the Low-delay Setting |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Yu Jing Wang Da-ke He En-hui Yang | Tel: Email: | xiayu wangj @rim.com dhe ehyang |
| Source: | Research in Motion Ltd., Canada |  |  |

_____________________________

# Abstract

This contribution proposed a quantization scheme for the low-delay low-complexity setting based on hard decision quantization and adaptive reconstruction levels. Specifically, for each inter frame, the forward quantization process uses the hard decision quantization as implemented in the HM codec, following by a simple statistics collection of reconstruction levels. The reconstruction levels are then selectively transmitted to the decoder and are used for reconstructing the next frame. Compared with the anchor codec with the low-delay low-complexity setting, the proposed scheme reduces the computational complexity while boosting the rate distortion coding performance. In terms of RD performance, the proposed scheme outperforms the anchor codec with the low-delay low-complexity setting by 1.2%, 2.2% and 3.0% by BD-rate for the Y, U, and V, respectively; in terms of complexity, the proposed method saves the RDOQ computation on the encoder side without changing the decoding complexity;

# Introduction

While the performance of a quantization process generally involves in two factors as space partitioning and sample reconstruction, quantization designs in video coding have been largely centralized around one parameter of quantization step size, which tends to bind partitioning and reconstruction together and limits the quantization performance. For example, given a quantization step size q, a conventional hard-decision-based quantization operates by the following:

u = round ( c / q + δ )                 ,                (1)

where c denotes a positive real number to be quantized, δ is the rounding offset, and u is called quantization output. In this case, the de-quantization is carried out as

cˆ = u * q         .

Accordingly, the reconstruction space is given by a series of points of

(0, q, 2q, 3q, …).                                                                (2)

Correspondingly, the partition is given as a series of boundaries of

(  (0.5- δ)q,  (1.5- δ)q,  (2.5- δ)q,   … ).                              (3)

For the best distortion performance, the rounding offset δ needs to be zero; if considering the rate distortion performance, δ may be adaptively selected, as seen in the TMuC reference software [6] and the JM codec of the AVC standard [7]. Still, within the hard-decision quantization scheme, both the partitioning and the reconstruction are largely determined by the quantization step size.

For video data, which is generally non-uniformly distributed, a non-uniform quantization design generally gives a better rate distortion coding performance than a quantizer with a uniform reconstruction space. However, in practice, there are two main problems related to non-uniform quantization: 1) the source distribution is often unknown. Thus in order to design a good non-uniform quantizer, one often needs off-line training to find its partitions and reconstruction levels. Note that even with training, there might still be mismatch between the statistics used to design the quantizer and the actual statistics of the data to be quantized; and 2) the quantization involves a time-consuming search within the non-uniformly distributed partitions. This contribution presents a method with adaptive reconstruction space selection to tackle the problem of the source non-uniformity. Specifically, the reconstruction levels for different quantization outputs are adaptively selected based on a given data sequence.

# Algorithm Description

Denotations are firstly introduced to facilitate the description.

Consider a scalar quantizer with its quantization step size q. Denote it as Q(). For a given set of data to be quantized, the process of quantization is to partition the set into multiple subsets and represent each subset with a single data point, which is usually called a reconstruction level in video coding scenarios. Consider a data set S. Denote the corresponding subsets after applying Q() as S0 , S1, S2, …, Si, … . Specifically, Si may be written as Si = {c(i)n, n=1,2,…, Ni, Q(c(i)n)=i}, where Ni is the size of  Si and c(i)n means the n-th coefficient that is quantized to i and will be reconstructed using the i-th reconstruction level.

For a conventional scalar quantizer with a quantization step size q, the i-th reconstruction level is given as iq. In this work, the reconstruction levels are computed from the data on the encoder side and are selectively signaled to the decoder. Denote the i-th reconstruction level as cu=i. Apparently, the result of minimizing the quantization distortion for Si is

cu=i=1Nin=1Nic(i)n.    (4)

Correspondingly, the i-th reconstruction level may be represented by its variation from the conventional reconstruction point of iq, i.e.,

δi=1Nin=1Nic(i)n−i⋅q.   (5)

Some calculation can show that the distortion deduction of using δi+i⋅qinstead of i⋅q to reconstruct Si is Ni⋅δi2⋅q2. In case of rate distortion optimization with , this distortion deduction may be converted into a rate saving of Ni⋅δi2⋅q2λbits.

From rate distortion optimization point of view, the selective signaling of δidepends on a trade-off between the rate saving of using δi+i⋅qinstead of i⋅qand the extra rate for signaling δi. In practice, a simple scheme based on Ni is used. Specifically, a threshold Ti is defined so that δiis transmitted when   Ni >Ti. On the other hand, for all subsets corresponding to Ni <=Ti , an average is calculated and used for reconstruction, as

δ=Ni≤Tiδi⋅NiNi≤TiNi.      (6)

In general, the above scheme represents a two-pass procedure, which is undesired in practice. To tackle this problem, it is proposed here that reconstruction levels are collected and transmitted based on one frame and are used for coding the next frame.

To sum up, the algorithm is described as follows:

- Initialize the reconstruction levels.

- Collecting reconstruction levels from the current frame. Code the current frame and collect data as (5) and (6).

- Transmitting selected reconstruction levels {δi} and an average level δ.

- Update the reconstruction levels for the next frame based on the transmission. Go the step 2.

3 Experimental Results

Experiments have been conducted for the low-complexity low-delay case, with results shown as follows:

|  | Low delay LoCo RDOQ off |  |  | Low delay LoCo RDOQ off, AQL |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |  |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 1.3 | 4.8 | 5.7 | -0.8 | -2.4 | -1.3 |  |  |
| Class C | 0.7 | 2.4 | 2.9 | -1.5 | -4.8 | -4.4 |  |  |
| Class D | 0.9 | 3.8 | 3.7 | -1.6 | -3.9 | -5.0 |  |  |
| Class E | 0.7 | 5.5 | 2.7 | -0.7 | 3.6 | -1.2 |  |  |
| All | 0.9 | 4.1 | 3.9 | -1.2 | -2.2 | -3.0 |  |  |
| Enc Time[%] | 94% |  |  | XX% |  |  |  |  |
| Dec Time[%] | 101% |  |  | XXX% |  |  |  |  |

Experiments have been conducted for the low complexity low-delay case, after implementing changes agreed in D356, with results shown as follows:

|  | D356 Low delay LoCo RDOQ off |  |  | D356 Low delay LoCo RDOQ off, AQL |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |  |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 1.4 | 4.8 | 5.8 | -0.7 | -2.3 | -1.2 |  |  |
| Class C | 0.6 | 2.4 | 2.6 | -1.6 | -4.6 | -4.7 |  |  |
| Class D | 1.0 | 3.8 | 3.7 | -1.6 | -4.0 | -4.8 |  |  |
| Class E | 0.7 | 5.6 | 2.4 | -0.7 | 3.2 | -1.4 |  |  |
| All | 0.9 | 4.1 | 3.8 | -1.2 | -2.3 | -3.0 |  |  |
| Enc Time[%] | 86% |  |  | XX% |  |  |  |  |
| Dec Time[%] | 102% |  |  | XXX% |  |  |  |  |

The reason encoding/decoding times are not provided for AQL is that tests were not run on the same cluster. This will need to be further verified. The variation of the encoding/decoding time between AQL and random access is expected to be within few percents.

# 4 Conclusions

The relative complexity of RDOQ is increased by 8% when applying D356 optimization

The proposed method achieves encoding complexity reduction while boosting the rate distortion performance by more than 1% compare to the anchor, and by more than 2% compare to RDOQ Off in the low-complexity low-delay setting.

It is proposed that the group consider the adaptive quantization level (AQL) for quantization when RDOQ if off and in particular for low delay settings.

# 5 References

JCTVC-D356 Three digits to speed up the reference encoder, 	F. Bossen (DOCOMO USA Labs)

Patent rights declaration(s)

Research In Motion Ltd may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).