| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D385 |
| --- | --- |

| Title: | Adaptive Loop Filtering Using Multiple Filter Shapes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Faouzi Kossentini Nader Mahdi Hsan Guermazi Mohammed Ali BenAyed | Email: | faouzi@ebriskvideo.com nader@score.com.tn hsan@score.com.tn medali@score.com.tn |
| Source: | eBrisk Video Inc. |  |  |

_____________________________

# Abstract

This contribution presents the advantages of utilizing different filter shapes in the Adaptive Loop Filtering part (that is applied to the luminance samples) of the TMuC 0.9 encoder/decoder. More specifically, we show that the simultaneous use of diamond- and cross-shaped filters yields good complexity-performance trade-offs.

# Introduction

In the TMuC 0.9 implementation [1], 5×5, 7×7, and 9×9 diamond-shaped symmetrical filters are used in the Adaptive Loop Filtering (ALF) of the reconstructed luminance samples [2]. This yields better complexity-performance trade-offs than those of the original KTA ALF [3] [4], where 5x5, 7x7 and 9x9 square-shaped symmetrical filters were employed. This is due to the fact that diamond-shaped symmetrical filters employ approximately 50% of the coefficients, while still attaining essentially the same coding efficiency levels.

However, even diamond-shaped symmetrical filters can be still costly in terms of both side information and computations, since the required number of coefficients to filter a sample is still relatively high. For example, the 9x9 diamond-shaped symmetrical filter (the largest of the three used filters) requires the use of 21 coefficients. Since the TMuC 0.9 encoder is allowed to send to the decoder as many as 16 filters, as many as 336 coefficients may be required to be sent to the decoder. Moreover, as is clear from Figure 1, the ALF of a single sample of the reconstructed picture may require as many as 21 multiplications and 41 additions (40 additions + one addition for the offset) when a 9x9 diamond-shaped symmetrical filter is employed. Finally, the use of three different filter sizes (namely 5x5, 7x7 and 9x9) can be computationally demanding at the encoder.

Figure 1 – Filtering using a 9x9 diamond-shaped symmetrical filter

- Proposed Technique

In ALF, for a given filter shape, there is a fundamental relationship between the filter size (i.e., number of filter coefficients) and the resulting complexity-performance trade-offs. By decreasing the filter size, the encoding/decoding process will be less computationally demanding, however, the coding efficiency can also decrease substantially. On the other hand, increasing the filter size will generally yield an increase in both computational requirements and coding efficiency. The filter size that will yield the best complexity-performance trade-offs will vary depending on video content and other parameters. Therefore, the TMuC 0.9 encoder/decoder allows the use of three different sizes (namely, 5x5, 7x7 and 9x9).

However, by using different shapes, we are able to use a single size for each considered shape, while maintaining good complexity-performance trade-offs. In this document,  we propose a new ALF technique that allows the use of 7x7 diamond-shaped and 13x13 cross-shaped filter sets (see Figure 2), where each filter set can include as many as 16 distinct filters (as in the case of the TMuC 0.9). Since the 7x7 diamond-shaped and the 13x13 cross-shaped symmetrical filters employ only 13 coefficients, the new ALF technique requires a maximum of 13 multiplications and 25 additions (24 additions + one addition for the offset) for the ALF filtering of each luminance sample of the reconstructed picture (Figure 3 & Figure 4). In addition, in order to reduce on-chip memory requirements, two additional candidates, where the numbers of lines (i.e., numbers of coefficients in the vertical direction) are limited to 9 and 7, while also maintaining the same total number of coefficients, have also been considered (Figure 5).

Figure 2 - Diamond-shaped & cross-shaped symmetric filters

Figure 3 Filtering using a 7x7 diamond-shaped symmetrical filter

Figure 4 - Filtering using a 13 cross-shaped symmetrical filter

Figure 5 - Cross-shaped symmetrical filters: a) 9 Lines, b) 7 Lines

The TMuC 0.9 ALF algorithm consists of three steps as shown in Figure 6. Steps 1 and 2 are performed using a set of 5x5 diamond-shaped filters. In Step 3, the 7x7 and 9x9 diamond-shaped filters are tested.

Figure 6 – TMuC 0.9 technique

In the proposed technique, Step 3 is removed and the first two steps (Step 1 & Step 2) are applied twice as shown in Figure 6. Note that the first two steps can be executed simultaneously using a set of 7x7 diamond-shaped filters and a set of cross-shaped filters (13x13, 17x9 or 19x7). In other words, these two steps could be performed in parallel, since there is no dependency between the two paths.

Figure 7 - Proposed technique

In this proposed technique, the new ALF filtering is applied to only the luminance component, and the same TMuC 0.9 ALF filtering is applied to the chrominance components.

# Simulation Results

The simulation experiments were conducted following the testing conditions in JCTVC-B300 [5] and JCTVC-B310_r3 [6]. The coding efficiency (BD-Rate) numbers were obtained for the Low Delay (Table 1), Random Access (Table 2) and Intra Only (Table 3) High-Efficiency profiles. The detailed luminance results for all video sequences are shown in Table 4.

Table 1 and Table 2 show that the proposed technique (with 13x13 cross-shaped filters) provides 0.9% and 0.4 % in average BD-Rate reductions for the Low Delay and Random Access profiles, respectively. Table 3 shows a slight loss of 0.1% for the Intra profile.

Note that the coding efficiency gains decrease for the large video sequences (Classes E, A and B). This is expected because, with the pictures being too large, using two different shapes does not provide enough adaptivity to provide significant coding efficiency gains when compared to the use of a single shape. A solution could be to split each picture into multiple slices and then generating new ALF filters for each slice. The corresponding simulation results will be presented at the next meeting.

It also appears from Table 4 that the proposed technique (which employs diamond-shaped and cross-shaped filters) improves in a big way the coding efficiency for the BQSquare & PartyScene video sequences, both of which containing substantial zooming (IN & OUT). This is likely due to the use of the large cross-shaped filter, which allows the encoder to cover a large space and adapt to such special type of video content, while still using a small number of coefficients.

The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: Intel Core 2 Quad Q8300 @2.50 GHz.

- RAM: 4 Go

- OS:  Windows XP Professional.

Since the original simulation experiments were performed using different PCs, and given our time constraints, we have estimated the encoding times on the above PC for only the first 10 frames of each of the video sequences.

Our simulation results show that the average decoding times for the proposed technique are essentially the same as those for the TMuC 0.9 ALF technique. The only exception is the Intra profile, where we achieved a 12 % reduction in average decoding time, and this is due to the fact that the TMuC 0.9 ALF employs mostly 9x9 diamond-shaped filters when only intra coding is allowed. However, it is important to note that the worst-case decoder complexity is substantially lower for the proposed method. In order to demonstrate the ability of the proposed technique in decreasing worst-case decoder complexity, an evaluation against the highest-complexity TMuC0.9 (i.e., where the ALF employs only 9x9 diamond-shaped symmetrical filters) has been conducted, and in this case, we found that our proposed technique reduces the average decoding time by approximately 10 % (see decoding numbers in red).

The encoding times are slightly larger; however, they can be reduced by exploiting parallelism and reducing the number of encoding passes. Preliminary simulation results show that, by reducing the numbers of passes, the encoding times can be made lower than that of the TMuC 0.9, while still achieving higher coding efficiency than that of the TMuC 0.9.

Table 1 – Simulation Results: Low Delay (High-Efficiency) Configuration (13x13)

|  | Low delay |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.4 | -0.1 | -0.1 |
| Class C | -0.6 | -0.4 | -0.5 |
| Class D | -2.2 | -1.5 | -0.8 |
| Class E | -0.3 | 0.2 | 0.0 |
| All | -0.9 | -0.5 | -0.4 |
| Enc Time[%] | 122% |  |  |
| Dec Time[%] | 100% (90%) |  |  |

Table 2 – Simulation Results: Random Access (High-Efficiency) Configuration (13x13)

|  | Random access |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.1 | 0.1 |
| Class B | 0.0 | -0.1 | -0.2 |
| Class C | -0.4 | -0.1 | 0.1 |
| Class D | -1.2 | -0.1 | -0.1 |
| Class E |  |  |  |
| All | -0.4 | -0.1 | -0.1 |
| Enc Time[%] | 126% |  |  |
| Dec Time[%] | 102% (91%) |  |  |

Table 3 – Simulation Results: Intra Only (High-Efficiency) Configuration (13x13)

|  | Intra Only |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.0 | 0.0 |
| Class B | 0.1 | 0.0 | 0.0 |
| Class C | 0.1 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E | 0.2 | -0.1 | -0.1 |
| All | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | 172 % |  |  |
| Dec Time[%] | 88% (87%) |  |  |

Table 4 – Detailed Results (Luminance Only) (13x13)

|  | Sequences | IntraOnly | RandamAccess | LowDelay |
| --- | --- | --- | --- | --- |
| S01 | Traffic | 0.2 | 0.3 |  |
| S02 | PeopleOnStreet | 0.3 | 0.1 |  |
| S03 | Kimono | 0.0 | -0.1 | -0.2 |
| S04 | ParkScene | 0.1 | 0.1 | -0.7 |
| S05 | Cactus | 0.1 | 0.2 | 0.1 |
| S06 | BasketballDrive | 0.1 | 0.1 | 0.1 |
| S07 | BQTerrace | 0.2 | -0.5 | -1.1 |
| S08 | BasketballDrill | 0.3 | 0.2 | 0.0 |
| S09 | BQMall | 0.1 | -0.1 | -0.5 |
| S10 | PartyScene | 0.0 | -1.6 | -1.9 |
| S11 | RaceHorses | 0.0 | -0.1 | -0.2 |
| S12 | BasketballPass | 0.0 | 0.0 | 0.2 |
| S13 | BQSquare | 0.0 | -4.2 | -8.1 |
| S14 | BlowingBubbles | -0.1 | -0.3 | -0.6 |
| S15 | RaceHorses | -0.1 | -0.2 | -0.3 |
| S16 | Vidyo1 | 0.3 |  | -0.4 |
| S17 | Vidyo3 | 0.2 |  | -0.2 |
| S18 | Vidyo4 | 0.2 |  | -0.4 |
| Average | Class A | 0.2 | 0.2 |  |
|  | Class B | 0.1 | 0.0 | -0.4 |
|  | Class C | 0.1 | -0.4 | -0.6 |
|  | Class D | 0.0 | -1.2 | -2.2 |
|  | Class E | 0.2 |  | -0.3 |
|  | All | 0.1 | -0.4 | -0.9 |

Table 5 and Table 6 show the obtained coding efficiency (BD-Rate) numbers, for the Low Delay HE configuration, when the 17x9 and 19x7 cross-shaped filters (respectively) are employed.

Table 5 shows that the proposed technique using 17x9 cross-shaped filters yields an average BD-rate reduction of 0.4 %. Table 6 shows that, when 17x9 cross-shaped filters are employed, a slight loss of 0.1 % in BD-rate average is obtained.

Table 5 – Simulation Results: Low Delay (High-Efficiency) Configuration (17x9)

|  | Low delay |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class C | -0.4 | -0.3 | -0.4 |
| Class D | -0.8 | -1.1 | -0.3 |
| Class E | -0.1 | 0.0 | -0.1 |
| All | -0.4 | -0.5 | -0.3 |

Table 6 – Simulation Results: Low Delay (High-Efficiency) Configuration (19x7)

|  | Low delay |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class C | -0.2 | -0.2 | -0.1 |
| Class D | -0.2 | -0.7 | 0.0 |
| Class E | 0.7 | -0.2 | -0.3 |
| All | 0.1 | -0.4 | -0.2 |

The detailed luminance results for all Class D, Class C, and Class E video sequences, and for each of the considered 13x13, 17x9 and 19x7 cross-shaped filters, are presented in Table 7.

Table 7 – Detailed Results (Luminance Only)

|  | Sequences | Proposed technique (13x13) | Proposed technique (17x9) | Proposed technique (19x7) |
| --- | --- | --- | --- | --- |
| S08 | BasketballDrill | 0.0 | 0.3 | 0.7 |
| S09 | BQMall | -0.5 | -0.3 | -0.1 |
| S10 | PartyScene | -1.9 | -1.4 | -1.3 |
| S11 | RaceHorses | -0.2 | -0.2 | 0.0 |
| S12 | BasketballPass | 0.2 | 0.2 | 0.3 |
| S13 | BQSquare | -8.1 | -2.7 | -1.3 |
| S14 | BlowingBubbles | -0.6 | -0.5 | -0.3 |
| S15 | RaceHorses | -0.3 | -0.2 | 0.8 |
| S16 | Vidyo1 | -0.4 | -0.3 | 0.0 |
| S17 | Vidyo3 | -0.2 | 0.1 | 1.7 |
| S18 | Vidyo4 | -0.4 | 0.0 | 0.4 |
| Average | Class C | -0.65 | -0.40 | -0.2 |
|  | Class D | -2.20 | -0.80 | -0.2 |
|  | Class E | -0.33 | -0.10 | 0.7 |
|  | All | -1.06 | -0.43 | 0.1 |

# Syntax and semantics

The Slice header information for the proposed technique is described below:

| alf_param() { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| if ( adaptive_loop_filter_flag ) { |  |  |
| filter_shape_flag | 2 | u(1) |
| } |  |  |
| } |  |  |

filter_shape_flag: 1 means that cross-shaped filters are used; 0 means that diamond-shaped filters are used.

# Conclusions

The proposed technique employs both diamond-shaped filters and cross-shaped filters. Our simulation experiments show that the proposed technique (with 13x13 cross-shaped filters) provides 0.4 % and 0.9 % in average BD-Rate reductions for the Random Access and Low Delay HE profiles, respectively. The proposed technique using 17x9 filters provides 0.4 % in average BD-Rate reduction for Low Delay HE configuration, and the proposed technique using 19x7 filters degrades slightly the coding efficiency (0.1 %) for the Low Delay HE configuration.

The average decoder complexity is the same for our method as for the TMuC 0.9; however, the worst-case complexity for the proposed technique is ~10% lower than that of the TMuC 0.9. While the encoding complexity is now significantly larger, it can be reduced (even without using a parallel implementation) by reducing the number of encoding passes. Preliminary results show that we can achieve, using the proposed technique, better tradeoffs between encoding complexity and coding efficiency. Complete simulation results will be presented at the next meeting.

Finally, we recommend that the JCT-VC continue the evaluation of this ALF technique for possible adoption into the HM. Specifically, we recommend verification of this experiment and harmonization with other ALF related tools.

# Reference

[1]	 https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.9/

[2] 	M. Karczewicz, P. Chen, R. Joshi, X. Wang, W. Chien, R. Panchal, “Video coding technology  proposal by Qualcomm Inc.”, JCTVC-A121, April 2010.

[3]	 T. Chujoh, N. Wada, T. Watanabe, G. Yasuda and T. Yamakage, “Specification and experimental results of Quadtree-based Adaptive Loop Filter”, ITU-T Q.6/SG16 Doc., VCEG-AK22, Yokohama, April 2009.

[4]	 http://iphome.hhi.de/suehring/tml/download/KTA/jm11.0kta2.7.zip

[5]	 “Common test conditions and software reference configurations”, JCTVC-B300, Geneva, July 2010.

[6]	 T. Yamakage, T. Chujoh, S. Lei, K. Chono, M. Karczewicz, A. Segall, T. Yamamoto, B. Jeon, J. Lim, J. Xu, “Tool Experiment 10: In-loop filtering”, JCTVC-B310_r3, Geneva, July 2010.

# Patent rights declaration(s)

eBrisk Video Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).