| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D386 WG11 Number: m19164 |
| --- | --- |

| Title: | Constrained Intra source code implementation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Jonatan Samuelsson KI/EAB/TVV/V 164 80 Stockholm Sweden | Tel: Email: | +46761153563 rickard sjoberg@ericsson com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

Constrained Intra prediction is a well-known and well-used concept in the MPEG-4 AVC/H.264 video coding standard. Although Constrained Intra has been part of all the TMuC and HM specifications it has not so far been implemented in the TMuC or HM source code. This paper presents an implementation of Constrained Intra in HEVC.

# Background

Constrained Intra prediction is a well-known and well-used concept in the MPEG-4 AVC/H.264 video coding standard [1]. Constrained Intra is a way to control that a specific block in one frame is not affected by a corruption/error of another frame. This is done by only allowing reconstructed pixels from other Constrained Intra blocks in the same slice for prediction of the current block. By “refreshing” parts of a frame it is possible to recover from errors in applications where IDR-frames are not feasible (or possible) due to latency constrains and limited bandwidth.

# Constrained intra in HEVC

Constrained intra has been part of all the TMuC and HEVC specifications [2] [3] [4] but has until now not been implemented in the software.

During our implementation of Constrained Intra we found that the current way of handling unavailable pixels during Intra prediction needed an update. Consider Figure 1:

Figure 1	 - Neighboring blocks and corresponding prediction pixel positions

The highlighted block I is the current Intra block and blocks A-E are neighboring blocks. The current source code assigns values for all the prediction pixel positions. It starts by setting them all to 128. Then it overwrites them with the actual values from blocks A-E if they are available. Should block B be available but not C, the rightmost prediction pixel value from B is used for all the prediction pixel positions in block C. Similarly, if block D is available but not block E, the prediction pixel in D closest to block E is used for all the prediction pixel positions of block E.

Consider now Figure 2. When constrained intra is off, all pixels are available for Intra prediction. When constrained intra is on, it is not clear what to do with prediction pixel positions of temporally predicted blocks (indicated by 'B' in the figure).

Figure 2	 - Example of neighboring block types with indication of prediction pixel positions

What we propose is to use interpolation between the closest available prediction pixel values when possible and extrapolation otherwise. The interpolation we propose is p = (a+b+1)>>1, where 'p' is a prediction pixel of an unavailable block, 'a' is the closest available prediction pixel to the left (or down) and 'b' is the closest available prediction pixel to the right (or above). In case no prediction pixels in blocks A-E are available we use 128 for all prediction pixels.

We noted that there are two other contributions on the topic of Constrained Intra at this meeting. There is one contribution from NEC [5] and one from Panasonic [6]. The NEC contribution describes a Constrained Intra implementation with a simplified check of surrounding blocks while the Panasonic contribution incidentally uses the same p = (a+b+1)>>1 method that is proposed in this contribution.

Constrained intra was implemented in HM using the same framework as the already existing intra prediction. Availability of blocks and prediction pixels are generalized not only to account for image borders (and slice borders) but also to consider the mode of adjacent blocks for the case when Constrained Intra is on.

We added the config file parameter ConstrainedIntraFlag to the encoder. The value is transmitted from encoder to decoder in the sequence parameter set, as specified in WD1 [4].

The Constrained Intra implementation was verified by modifying the decoder to print all CU coordinates and types (I, P, B or skip) to a text file during decoding. A second decoder modification was to optionally set all pixel values of all prediction pictures to 0 before start decoding each picture. Bitstreams with Constrained Intra turned on was then decoded both normally and with prediction pixels set to 0. A program that compares Intra coded pixel of two sequences was written, using the CU coordinate/types text file. Only low delay sequences was tested since resetting predicted pixel values for random access coding proved difficult. Also, deblocking filter and loop filter was turned off. All low delay anchor bitstreams were checked without any Intra-pixel mismatch.

# Compression efficiency results

The table below shows results for Constrained Intra implemented on top of the Slices ad-hoc source code using the same simplified approach as proposed by NEC [5]. These results are very similar to those reported in [5]. The encoder/decoder times comes from a cluster containing different types of computers and are therefore not reliable.

Table I: BD-rate and encoding and decoding time ratios for common conditions and sequence dependent QPs

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 97% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 110% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2.0 | 2.5 | 2.5 | 1.6 | 1.7 | 2.1 |
| Class B | 1.7 | 3.3 | 3.1 | 1.5 | 2.6 | 2.8 |
| Class C | 2.7 | 4.5 | 4.4 | 2.3 | 3.4 | 3.4 |
| Class D | 2.1 | 3.6 | 3.8 | 1.7 | 2.2 | 2.6 |
| Class E |  |  |  |  |  |  |
| All | 2.1 | 3.6 | 3.5 | 1.8 | 2.6 | 2.8 |
| Enc Time[%] | 96% |  |  | 98% |  |  |
| Dec Time[%] | 109% |  |  | 110% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.6 | 3.9 | 3.8 | 1.4 | 2.9 | 3.4 |
| Class C | 2.8 | 5.4 | 5.4 | 2.3 | 3.8 | 4.1 |
| Class D | 1.6 | 3.4 | 3.6 | 1.2 | 2.6 | 2.7 |
| Class E | 0.7 | 1.6 | 1.2 | 0.7 | 2.2 | 1.8 |
| All | 1.7 | 3.7 | 3.6 | 1.4 | 2.9 | 3.1 |
| Enc Time[%] | 86% |  |  | 95% |  |  |
| Dec Time[%] | 104% |  |  | 108% |  |  |

# References

[1] ITU-T Rec. H.264 | ISO/IEC 14496-10

[2] JCT-VC, "Test Model under Consideration", Doc JCTVC-A205_draft007.doc, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Dresden, Germany, Apr 2010

[3] JCT-VC, "Test Model under Consideration",  Doc JCTVC-B205_draft007.doc, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Jul 2010

[4] Thomas Wiegand, Woo-Jin Han, Benjamin Bross, Jens-Rainer Ohm, Gary J. Sullivan, "WD1: Working Draft 1 of High-Efficiency Video Coding", Doc JCTVC-C403, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, Oct 2010

[5] Keiichi Chono, Hirofumi Aoki, Xuan Jing, Viktor Wahadaniah, ChongSoon Lim, Sue Mon Thet Naing, "Constrained intra prediction for reducing visual artifacts caused by lossy decoder-side memory compression", Doc JCTVC-D086, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, Korea, Jan 2011

[6] Viktor Wahadaniah, ChongSoon Lim, Sue Mon Thet Naing, Xuan Jing, "Constrained Intra Prediction Scheme for Flexible-Sized Prediction Units in HEVC", Doc JCTVC-D094, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, Korea, Jan 2011

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).