| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D387 WG11 Number: m19165 |
| --- | --- |

| Title: | Cross-verification of D127: Leaf Coding Unit Aligned Slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Per Wennersten KI/EAB/TVV/V 164 80 Stockholm Sweden | Tel: Email: | +46761153563 rickard sjoberg@ericsson com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document is a cross-check report of document D127, "Leaf Coding Unit Aligned Slices".

# Introduction

Ericsson has cross-checked proposal D127 from MediaTek. The document reviewed was JCTVC-D127-v1.zip. The source code was made available on January 10. Besides verifying that the results reported in D127 are correct, we have also reviewed the supplied source code.

# Cross-verification

D127 contains the following parts:

- Signaling in the sequence parameter set whether slice addresses are LCU-based or leaf-based

- A two-level signaling of slice address in case of leaf-based slices

- A last_lcu_possible_flag at the start of each LCU in case of leaf-based slices. Only these LCUs have end_of_slice flags sent for its leaf CUs

- A syntax design of the proposal

- Results

## Cross-check results

Only Class D was tested. The proponents provided a set of test cases which were run and cross-verified. The code complied and ran all the test cases without any problems. The only thing was that the definition of UINT_MAX had to be done explicitly in order for the code to compile on Linux. No differences were found in any of the test cases provided.

The compression efficiency of LCU slices was also compared between D127 and D004 (the slice support development ad-hoc group code). The following setting of the D127 software was used:

- SliceMode = 2

- SliceArgument = 1500

- LCUAlignedSliceFlag = 1

- UsePictureRBSP = 1

- SliceBoundaryFilterDisable = 0

- SBFPostLoopEnable = 0

- ALF = 1

The following setting of D004 was used:

- SliceMode = 2

- SliceArgument = 1500

These settings should both produce 1500 byte slices in a similar manner. Figure 1 shows the results.

Figure 1	 - Results comparing 1500 byte slices between D127 and D004

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class D D127 | 4.3 | 5.1 | 5.8 | 3.3 | 2.6 | 2.8 |
| Class D D004 | 3.5 | 3.3 | 3.4 | 4.0 | 3.3 | 3.5 |
|  |  | Random access |  |  | Random access LoCo |  |
| Class D D127 | 2.4 | 5.1 | 5.9 | 2.6 | 1.1 | 1.7 |
| Class D D004 | 2.4 | 3.2 | 3.2 | 2.4 | 1.0 | 1.6 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class D D127 | 2.6 | 2.8 | 3.1 | 1.6 | 0.2 | 0.2 |
| Class D D004 | 1.4 | 1.3 | 2.1 | 1.5 | 0.0 | 0.1 |

We note that D127 is 0.3% less efficient on average.

## Source code comments

D127 has been implemented on top of TMuC version 0.9 source code directly and not on the slice ad-hoc group source code. The code is clearly written, and the implementation of slices is fairly similar to the one performed by the ad-hoc group. Because of this, we do not anticipate any major difficulties for the proponents to port the source code to the ad-hoc group code.

# Conclusion

We believe that leaf-aligned slices would be a very useful feature for HEVC. D127 compiled and ran without any problems. However, we believe a leaf-aligned slice implementation should be based on the ad-hoc group source code. We therefore recommend JCT-VC to add leaf-aligned slices to its requirements and that a core experiment is set up with the focus on leaf-aligned slices.

.