| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D390 |
| --- | --- |

| Title: | CE13: Summary report on intra smoothing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Muhammed Coban Qualcomm Benjamin Bross Fraunhofer HHI Jianle Chen Samsung | Tel: Email: | +1 858 658 3973 mcoban@qualcomm.com +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de jianle.chen@samsung.com |
| Source: | Qualcomm, Fraunhofer HHI, Samsung |  |  |

_____________________________

# Abstract

This document reports summary of core experiment 13 on intra smoothing [1]. Six companies have been registered in CE13 and four proposed tools from two companies have been evaluated according to conditions in CE13 core experiment description. Each proposal had a tool that performed lookup table based intra reference smoothing that depended on intra prediction mode and block size along with a second tool that combined lookup table and signaling. Coding efficiency and complexity results are presented.  Each proposal has a tool that performed lookup table based intra reference smoothing that depends on the intra prediction mode and the block size.

# Introduction

The core experiment evaluates the coding efficiency and complexity of proposed intra smoothing algorithms.

# Participants List

| Company | Participants |
| --- | --- |
| Qualcomm | Muhammed Coban, mcoban@qualcomm.com |
| Fraunhofer HHI | Benjamin Bross, benjamin.bross@hhi.fraunhofer.de |
| Samsung | Jianle Chen, jianle.chen@samsung.com |
| Toshiba | Akiyuki Tanizawa, akiyuki.tanizawa@toshiba.co.jp |
| Sony | Kazushi Sato, Kazushi.Sato@jp.sony.com |
| NEC | Keiichi Chono, chono@ct.jp.nec.com |

# Tools under test

The following tools are tested.

| Proponent | Document | Codebase | Tool description | Cross-checker |
| --- | --- | --- | --- | --- |
| Qualcomm | JCTVC-C234 | TMuC 0.9 | Simplified Intra Smoothing | Fraunhofer HHI, Samsung, Toshiba |
| Fraunhofer HHI | JCTVC-C302 | TMuC 0.9 | Fast Adaptive Intra Smoothing | Qualcomm, Samsung, Sony, NEC |

# Tested Proposals

## Mode Dependent Intra Smoothing

In JCTVC-C234 [2], a lookup table (LUT) based intra smoothing technique was proposed. Depending on the prediction mode (intra direction) and block size, the reference samples for intra prediction are either 1-pass filtered or not.

## Mode Dependent Hybrid Intra Smoothing

In addition to the LUT-table smoothing approach in 4.1, the filtering type (pass) of intra reference samples is signaled for all intra prediction modes except intra DC for blocks sizes greater than 8x8.

## LUT-based Intra Smoothing

In JCTVC-C302 [3], a similar LUT-based intra smoothing technique was proposed. Depending on the prediction mode (intra direction) and block size, the reference samples for intra prediction are either 2-pass filtered or not.

## Fast Adaptive LUT-based Intra Smoothing

In this adaptive approach it is signaled whether the LUT from 4.3 is used or inverted. In this case, inverted means if the table entry is “filter” it is changed to “not filter” and vice versa. For intra DC modes nothing is signaled. For all other modes, the proposed fast estimation is threshold based. Instead of testing only the best mode, several candidates are tested. For this, the RD cost range (rd_range) between best (rd_best) and worst candidate is calculated and only candidates with RD costs lower than 3/8 * rd_range + rd_best are tested.

# Results

Objective rate-distortion measurements using BD-Rate for proposed tools in section 4 have been performed. The values are averaged over all sequences from each class. For the complexity measurement, the anchor encoding encoding and decoding times along with the tool under test’s encoding and decoding times are measured.  The reported times are average over all sequences. Encoding and decoding times are measured on the cluster consisting of different computers.

Each tool was compared against TMuC-0.9 (anchor) [4] with default HM1.0 tool/macro configuration [5]. The coding conditions used for the simulations are specified in [6]. The results are presented in Table 1 and Table 2. Each table represents a test set for intra, random access, and low delay coding configurations using high efficiency (HE) and low complexity (LoCo) settings.

Table 1 Average BD-rates and relative encoding/decoding runtimes for intra, random access, low delay configurations for high efficiency and low complexity profiles for C234 proposals 4.1 and 4.2.

Table 2 Average BD-rates and relative encoding/decoding runtimes for intra, random access, low delay configurations for high efficiency and low complexity profiles for C302 proposals 4.3 and 4.4.

# Conclusion

All proposals in CE13 have been cross verified. For intra only C234 MDIS provides bitrate savings between 0.4% and 0.7% with no increase in encoding/decoding time. C234 MDHIS provides bitrate savings of 0.4% to 0.9% with no significant differences in encoding and decoding time. C302 LUT intra smoothing provides bitrate savings of 0.1% to 0.5% with no increase in encoding/decoding time. C302 fast adaptive LUT based intra smoothing provides bitrate savings of 0.3% to 0.5% with ~9 to 14% increase in encoding complexity.

# References

- Muhammed Coban, Benjamin Bross, Jianle Chen, “Core Experiment 13: Intra Smoothing”, JCTVC-C513, Guangzhou, October, 2010.

- Yunfei Zheng, Muhammed Coban, Marta Karczewicz, “Simplified Intra Smoothing”, JCTVC-C234, Guangzhou, October, 2010.

- Tung Nguyen, Benjamin Bross, Detlev Marpe, Thomas Wiegand, “Improved intra smoothing for UDI and new AIS fast mode”, JCTVC-C302, Guangzhou, October, 2010.

- JCT-VC, “Test Model under Consideration Software 0.9”, https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.9

- T. K. Tan, G. J. Sullivan, J.-R. Ohm, “Summary of HEVC working draft 1 and HEVC test model (HM)”, JCTVC-C405, Guangzhou, October, 2010.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, Guangzhou, October, 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33

# Patent rights declaration(s)

Qualcomm and Fraunhofer HHI may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).