| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D392 |
| --- | --- |

| Title: | Mode dependent intra residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi, Peisong Chen, Marta Karczewicz (Qualcomm) Akiyuki Tanizawa, Jun Yamaguchi (TOSHIBA) Chuohao Yeo, Yih Han Tan (Institute for Infocomm Research) Haitao Yang, Haoping Yu (Huawei) | Tel: Email: | +1-858-658-4511 rajanj@qualcomm.com akiyuki.tanizawa@toshiba.co.jp chyeo@i2r.a-star.edu.sg haopingyu@huawei.com |
| Source: | Qualcomm, TOSHIBA, Institute for Infocomm Research, Huawei |  |  |

_____________________________

# Abstract

For intra predicted blocks, the prediction residual is first transformed and then, the quantized transform coefficients are entropy coded. By selecting transform and scan order based on intra prediction modes, we can realize higher coding efficiency with small complexity increase.

# Mode dependent residual coding

## One Dimensional Directional Unified Transform (1DDUT)

1DDUT utilizes only two 1-D transform matrices, which can specify four combinations of the 2-D transform. A combination is selected for each intra prediction mode. It means that the intra prediction modes are assigned into four categories, which are three directions and no direction (DC prediction). Error: Reference source not found shows the relationship between the transform index and 1D transform matrices, and Table 2 shows the relationship between intra prediction modes and the transform index. The transforms are chosen as follows. T1 is the odd type-III discrete sine transform specified by

Ki,j=22N+1sin(2i−1)jπ2N+1,

where N is the size of the transform. T2 is the discrete cosine transform of type-II. It should be noted that there is no training involved in the choice of the transforms.

Table 1:  Relationship between the TransformIdx and each 1 D transform matrix

| TransformIdx | Vertical 1D Transform | Horizontal 1D Transform |
| --- | --- | --- |
| 0 | T1 | T2 |
| 1 | T2 | T1 |
| 2 | T2 | T2 |
| 3 | T1 | T1 |

Table 2:  Relationship between the IntraPredMode and the TransformIdx

| IntraPredMode [ puPartIdx ] | IntraPredType [ puPartIdx ] | IntraPredAngleID [ puPartIdx ] | TransformIdx [ puPartIdx ] |
| --- | --- | --- | --- |
| 0 | Intra_Vertical | 0 | 0 |
| 1 | Intra_Horizontal | 0 | 1 |
| 2 | Intra_DC | - | 2 |
| 3 | Intra_Vertical | -8 | 3 |
| 4 | Intra_Vertical | -4 | 3 |
| 5 | Intra_Vertical | +4 | 0 |
| 6 | Intra_Vertical | +8 | 0 |
| 7 | Intra_Horizontal | -4 | 3 |
| 8 | Intra_Horizontal | +4 | 1 |
| 9 | Intra_Horizontal | +8 | 1 |
| 10 | Intra_Vertical | -6 | 3 |
| 11 | Intra_Vertical | -2 | 0 |
| 12 | Intra_Vertical | +2 | 0 |
| 13 | Intra_Vertical | +6 | 0 |
| 14 | Intra_Horizontal | -6 | 3 |
| 15 | Intra_Horizontal | -2 | 1 |
| 16 | Intra_Horizontal | +2 | 1 |
| 17 | Intra_Horizontal | +6 | 1 |
| 18 | Intra_Vertical | -7 | 3 |
| 19 | Intra_Vertical | -5 | 3 |
| 20 | Intra_Vertical | -3 | 3 |
| 21 | Intra_Vertical | -1 | 0 |
| 22 | Intra_Vertical | +1 | 0 |
| 23 | Intra_Vertical | +3 | 0 |
| 24 | Intra_Vertical | +5 | 0 |
| 25 | Intra_Vertical | +7 | 0 |
| 26 | Intra_Horizontal | -7 | 3 |
| 27 | Intra_Horizontal | -5 | 3 |
| 28 | Intra_Horizontal | -3 | 3 |
| 29 | Intra_Horizontal | -1 | 1 |
| 30 | Intra_Horizontal | +1 | 1 |
| 31 | Intra_Horizontal | +3 | 1 |
| 32 | Intra_Horizontal | +5 | 1 |
| 33 | Intra_Horizontal | +7 | 1 |

## Coefficient scanning

### In the high efficiency case where CABAC is used for entropy coding, there are three scan orders: zig-zag, horizontal and vertical. The selection of the scan order is dependent on the transform block size and the spatial prediction mode.

### In the low complexity case where LCEC is the entropy coder, transform coefficients are coded according to either a zigzag scan order or a directional scan order. The choice of scan is dependent on the intra prediction mode of the current block. Through a simple transpose mapping, the same directional scan order is shared for both horizontally oriented prediction modes and vertically oriented prediction modes. Within each frame, based on coding statistics, the scan order of the first few (up to 64) coefficients is adaptively adjusted through a swap operation after coding each non-zero coefficient.

We provide results for both adaptive scans and fixed scans.

# Experimental results

The coding efficiency is evaluated in Intra, random access (RA) and low delay (LD) configurations for both the high efficiency and low complexity cases based on the common conditions defined in JCTVC-C500[1]. The coding efficiency is measured by using 4 point BD-Rate.

The experimental results are shown in the Tables 1 and 2. Table 1 includes low-complexity results with adaptation. Table 2 includes low-complexity results without adaptation. From the low-complexity results, it can be seen that adaptation actually has worse performance. This is due to the lack of use of coefficient coding fix proposed in [2]. Table 3 shows low-complexity results with, and without adaptation when coefficient coding fix is applied for Intra LoCo cases. Is should be noted that when the coefficient coding fix is applied to TMuC 0.9, there is a BD-rate improvement of about 1.6%. So the real gains due to mode dependent residual coding should be reduced by 1.6% for luma from the values shown in the table.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.9 | -3.4 | -3.5 | -3.8 | -3.1 | -3.0 |
| Class B | -1.4 | -2.2 | -2.2 | -2.4 | -3.0 | -3.2 |
| Class C | -2.1 | -2.2 | -2.2 | -2.1 | -2.2 | -2.2 |
| Class D | -2.1 | -2.0 | -2.0 | -1.8 | -1.7 | -1.7 |
| Class E | -2.9 | -3.6 | -3.3 | -3.7 | -3.9 | -3.8 |
| All | -2.1 | -2.5 | -2.5 | -2.6 | -2.7 | -2.7 |
| Enc Time[%] | 106% |  |  | 114% |  |  |
| Dec Time[%] | 105% |  |  | 116% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.2 | -1.0 | -0.8 | -1.3 | -0.6 | -0.7 |
| Class B | -0.8 | -1.0 | -1.0 | -1.0 | -1.1 | -1.1 |
| Class C | -1.0 | -0.9 | -1.0 | -0.7 | -0.7 | -0.8 |
| Class D | -0.9 | -0.5 | -0.2 | -0.5 | -0.4 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.9 | -0.8 | -0.7 | -0.8 | -0.7 | -0.8 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -0.4 | -0.7 | -0.4 | -0.3 | -0.5 |
| Class C | -0.4 | -0.7 | -0.6 | -0.2 | -0.4 | -0.3 |
| Class D | -0.2 | -1.0 | -0.4 | -0.2 | -0.1 | -0.1 |
| Class E | 0.0 | -0.6 | -0.3 | -0.1 | -0.7 | -0.7 |
| All | -0.2 | -0.7 | -0.6 | -0.2 | -0.3 | -0.4 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |

Table 1: BD-rate results with VLC adaptation for LoCo

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.9 | -3.4 | -3.5 | -3.7 | -2.6 | -2.5 |
| Class B | -1.4 | -2.2 | -2.2 | -2.3 | -2.7 | -2.8 |
| Class C | -2.1 | -2.2 | -2.2 | -2.3 | -2.2 | -2.3 |
| Class D | -2.1 | -2.0 | -2.0 | -2.3 | -2.0 | -2.0 |
| Class E | -2.9 | -3.6 | -3.3 | -4.0 | -4.0 | -3.9 |
| All | -2.1 | -2.5 | -2.5 | -2.7 | -2.6 | -2.6 |
| Enc Time[%] | 106% |  |  | 105% |  |  |
| Dec Time[%] | 105% |  |  | 107% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.2 | -1.0 | -0.8 | -1.3 | -0.5 | -0.6 |
| Class B | -0.8 | -1.0 | -1.0 | -1.0 | -1.0 | -0.9 |
| Class C | -1.0 | -0.9 | -1.0 | -0.9 | -0.8 | -0.8 |
| Class D | -0.9 | -0.5 | -0.2 | -0.7 | -0.5 | -0.5 |
| Class E |  |  |  |  |  |  |
| All | -0.9 | -0.8 | -0.7 | -0.9 | -0.8 | -0.7 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -0.4 | -0.7 | -0.4 | -0.4 | -0.5 |
| Class C | -0.4 | -0.7 | -0.6 | -0.3 | -0.5 | -0.3 |
| Class D | -0.2 | -1.0 | -0.4 | -0.3 | 0.0 | -0.1 |
| Class E | 0.0 | -0.6 | -0.3 | -0.1 | -0.7 | -0.6 |
| All | -0.2 | -0.7 | -0.6 | -0.3 | -0.4 | -0.4 |
| Enc Time[%] | 101% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table 2: BD-rate results without VLC adaptation for LoCo

|  | BD Rate |  |  |
| --- | --- | --- | --- |
| Fixed scans with VLC coefficient coding fix | -4.6 | -4.1 | -4.1 |
| Adaptive scans with VLC coefficient coding fix | -5.3 | -4.6 | -4.6 |

Table 3: BD-rate results for Intra low-complexity configuration for fixed and adaptive scans with VLC coefficient coding fix.

# Reference

- F. Bossen, “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, CN, Oct., 2010.

- M. Karczewicz, X. Wang, W.-J. Chien, “CE5: Improved coefficient coding with LCEC”, JCTVC-D374, Daegu, South Korea, January 2011.

# Patent rights declaration(s)

Qualcomm, TOSHIBA, Institute for Infocomm Research and Huawei may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).