| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D394 WG11 Number: m19173 |
| --- | --- |

| Title: | CE12: Adaptive Motion Vector Resolution from Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien Peisong Chen Marta Karczewicz 5775 Morehouse Dr. San Diego, CA92130 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com peisongc@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents an adaptive motion vector resolution method within the scope of Core Experiment 12 which was set up in the 3rd JCT-VC meeting. Simulation results show an average 1.3 % coding gain. In terms of run time, the encoding complexity increases 6% on average and the decoding complexity remains the same. In addition, experimental results show the coding gain and complexity tradeoff between the bi-directional motion prediction and the proposed method.

# Introduction

In the 3rd JCT-VC meeting, Core Experiment 12 [1] (CE12) was set up to explore the performance of the different adaptive motion vector resolution methods in the Test Model under Consideration (TMuC). In [2], an AMVRES method is proposed and further investigation was encouraged. This contribution contains the descriptions and the experiment results of the proposed method and its extension to the low complexity configurations.

# Proposed method

## Motion vector resolution flag

In the proposed AMVRES method, the motion accuracy can be adaptively selected to be 1/4th pixel or 1/8th pixel for each motion vector. The choice of motion vector resolution is signaled to the decoder. A syntax element, named motion vector resolution flag (mvres_flag), is designed for this purpose. For uni-directional prediction, a value of 0 (1) implies 1/8th pixel (1/4th pixel) motion vector resolution. For bi-directional prediction, a value of 0 implies both motion vectors are in 1/8th pixel motion vector resolution. A value of 1 implies both motion vectors are in 1/4th pixel motion vector resolution. A value of 2(3) implies only List 0 (List 1) motion vector is in 1/4th pixel motion vector resolution.

For CABAC encoding of mvres_flag, a unary binarization is used. The contexts are defined based on the motion resolution of neighboring PUs, and the CU depth (iCUdepth). Let mvresA_Lx and mvresB_Lx be the motion vector resolution of the motion vectors in List x of the left and upper neighboring PUs, respectively. Note that mvresX_Lx equals to 1 if no motion vector is available. The context index iCtx for the first bit of the mvres_flag is as follow:

- When iMvres = 0, iCtx = iCUdepth/2.

- When iMvres = 1, if iCUdepth < 2, iCtx = 2; otherwise, iCtx = iCUdepth +1.

- When iMvres = 2, iCtx = iCUdepth +4.

where iMvres = (mvresA_L0+ mvresB_L0)/2 for uni-directional prediction and iMvres = (mvresA_L0+ mvresB_L0+ mvresA_L1+ mvresB_L1)/2 for bi-directional prediction.

To code the second bit of mvres_flag, context 9 is used. The third bit is coded as equal probability (no extra context needed).

For LCEC encoding of mvres_flag, a joint coding, which includes inter_pred_idc, ref_idx and mvres_flag is applied. It is similar to LCEC phase 2 implementation.

## Diagonal interpolation filter for high accuracy motion resolution

In [3], a diagonal interpolation filter (DIF) was proposed to support high accuracy motion resolution in the low complexity configurations.  However, DIF in [3] supports only 16 1/8th pel positions that are directly aligned to the full pel positions. The signaling of mvres_flag is then drastically different from the high efficient configurations.

In this contribution, we proposed a DIF that supports all 1/8th pel positions. The subpels that are already supported in [3] remain unchanged. Each of the remaining 1/8th pel positions is obtained by using only a 6 taps filter. The interpolation process is conceptually the same as some particular 1/4th pel positions in [3], e.g. f, i, k, n, and it is equivalent to the process of bi-linear interpolating two or three subpels without intermediate rounding.

# Experiment results

The experiments are performed under common test condition [4]. The following table summarizes the obtained results for the High Efficiency (HE) and Low Complexity (LC) profiles. An average 1.3% coding gain is observed for all configurations, except 1.2 % in LC low delay configuration.  The average encoding complexity increases is 8% on HE and 4% on LC. The decoding complexity increases is around 2%, except for HE low delay configuration where we observed 8% complexity reduction. Note that the encoding complexity is measured in LINUX cluster.

Table 1. Coding performance of AMVRES.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | -0.3 | -0.4 | -0.5 | -0.2 | -0.1 |
| Class B | -1.5 | -0.9 | -0.9 | -1.2 | -0.2 | -0.2 |
| Class C | -1.1 | -0.5 | -0.5 | -1.4 | -0.2 | -0.3 |
| Class D | -1.4 | -0.6 | -0.4 | -1.7 | -0.4 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | -1.3 | -0.6 | -0.6 | -1.3 | -0.2 | -0.2 |
| Enc Time[%] | 109% |  |  | 104% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.1 | -0.9 | -1.3 | -1.1 | -1.0 | -0.9 |
| Class C | -1.1 | -0.7 | -1.0 | -1.4 | -1.0 | -1.2 |
| Class D | -1.8 | -2.1 | -1.2 | -1.3 | -0.8 | -1.1 |
| Class E | -1.1 | -0.4 | 0.0 | -0.9 | -0.6 | -1.1 |
| All | -1.3 | -1.1 | -1.0 | -1.2 | -0.9 | -1.1 |
| Enc Time[%] | 107% |  |  | 103% |  |  |
| Dec Time[%] | 92% |  |  | 102% |  |  |

We also provide the experiment results of the proposed method in IPPP configuration. The anchor is TMuC 0.9 in low delay configuration (GPB=1). Table 2 shows the performance of TMuC 0.9 and the proposed method in low delay configuration but GPB=0. When GPB is disabled, the encoding complexity reduced by 50% for both TMuC 0.9 and the proposed method. Moreover, the decoding complexity also reduced by 15%. With the proposed method, the coding performance loss compared to anchor reduced by 4 % for HE configuration and by 1% for LC configurations.

Table 2. Coding performance of AMVRES under IPPP configuration (GPB=0).

|  | TMuC 0.9, GPB=0 |  |  | Proposed, GPB=0 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low delay | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 6.2 | 6.1 | 4.8 | 2.3 | 0.4 | 1.9 |
| Class C | 5.4 | 4.1 | 4.1 | 1.7 | 0.3 | 0.1 |
| Class D | 7.5 | 4.1 | 5.2 | 0.8 | -2.9 | -1.9 |
| Class E | 5.3 | -4.6 | -5.0 | 1.1 | -7.4 | -7.5 |
| All | 6.2 | 3.1 | 2.9 | 1.6 | -1.9 | -1.3 |
| Enc Time[%] | 58% |  |  | 46% |  |  |
| Dec Time[%] | 92% |  |  | 83% |  |  |

|  | TMuC 0.9, GPB=0 |  |  | Proposed, GPB=0 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low delay LoCo | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.1 | 4.1 | 4.5 | 1.9 | 2.5 | 2.8 |
| Class C | 2.9 | 3.5 | 3.7 | 1.2 | 1.6 | 1.9 |
| Class D | 4.3 | 6.6 | 6.8 | 2.5 | 4.5 | 4.2 |
| Class E | 0.5 | 4.6 | 4.1 | 0.6 | 2.3 | 0.6 |
| All | 2.9 | 4.6 | 4.8 | 1.6 | 2.7 | 2.5 |
| Enc Time[%] | 46% |  |  | 48% |  |  |
| Dec Time[%] | 83% |  |  | 86% |  |  |

# Conclusions

In this contribution, an adaptive motion vector resolution method is proposed to HEVC. Experimental results show that the proposed AMVRES achieves an average BD-rate reduction of 1.3 % for all configurations except low complexity low delay configuration where the proposed method achieves an average BD-rate reduction of 1.2 %. The average encoding complexity increases due to the proposed method is approximately 6% and average decoding complexity increases are marginal. Moreover, the proposed method shows a good tradeoff between coding performance and complexity when compared to GPB. We recommend including the proposed AMVRES in the JCT-VC HM design.

# Reference

- W.-J. Chien, “Core Experiment 12: Adaptive Motion Resolution”, JCTVC-C512, Guangzhou, CN, Oct., 2010

- W.-J. Chien, P. Chen, M. Karczewicz, “TE 12: Evaluation of Adaptive Motion Vector Resolution (AMVRES),” JCTVC-C265, Guangzhou, China, Oct 2010.

- K. Ugur, J. Lainema, “Adaptive MV resolution with directional filters”, JCTVC-C215, Guangzhou, CN, Oct., 2010.

- F. Bossen, “Common conditions and software reference configurations”, JCTVC-C500, Guangzhou, CN, Oct., 2010.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).