| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D395 WG11 number: m19174 |
| --- | --- |

| Title: | Comments on chroma deblocking bug fix 119 and WD1 text |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Discussion |  |  |
| Author(s) or Contact(s): | Andrey Norkin, Rickard Sjöberg Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

A bug fix in chroma deblocking has been found and fixed. The bug fix BUGFIX119 changes the coder behavior. The fix does not correspond to HEVC draft text in document C403 [1]. Other mismatches between the source code and C403 text have also been found. Suggested draft text changes are included.

# Chroma filtering bug fix

The following bug has been found in TMuC0.9 by TI. When CU size is 8, m_aapucBS[iDir][0][uiBsAbsIdx] for some values of uiBsAbsIdx are accessed before being calculated. The fix makes a separate decision on chroma filtering for each part of the chroma block border (which is 2 chroma pixels in length). In TMuC0.9, the decision was made for the edge of 8 chroma pixels in lengths. Therefore, for CUs of size 8, the algorithm was reading from m_aapucBS[iDir][0][uiBsAbsIdx] for the uiBsAbsIdx, for which the values have not been calculated yet. Current C403 text assumes that a common decision for chroma filtering is done for the 8 chroma pixel border. However, the B205 text did not state that the decision for chroma filtering should be made for 8 pixel border. The Working Draft text has probably been changed according to the TMuC source code, which had a bug in it.

Therefore it is suggested to change the standard draft text according to the BUGFIX119.

The BD-rate changes due to the bugfix199 are provided in Table 1 (incomplete results).

Table 1 BD-rate changes after BUGFIX119 (incomplete)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0.0 | 0.0 | -0.1 |
| Class C | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.1 | 0.0 | -0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | #VALUE! | #VALUE! | #VALUE! | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0.0 | -0.1 | -0.2 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | -0.7 | 0.0 | 0.0 | 0.0 | -0.2 |
| Class E | 0.0 | 0.0 | -0.8 | 0.0 | 0.1 | 0.1 |
| All | #VALUE! | #VALUE! | #VALUE! | 0.0 | 0.0 | -0.1 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |

The proposed changes to the standard draft text according to the BUGFIX119 are included.

#### Filtering process for coding unit

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process for luma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( log2CUSize – 3 ).

- For xDk set equal to xB+( k << 3 ), k=0..nD - 1, the following applies:

- Boundary filtering strength bSVer is derived as follows:

bSVer = Max( bS[ 0 ][ xDk ][ yB + i ] ) for i = 0..7 	(8-1)

- Filtering process for block edge in subclause  is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yB ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 0 and the boundary filtering strength bsVer as inputs and the modified luma picture buffer as output.

- For yDk set equal to yB+( k << 3 ), k=0..nD - 1, the following applies:

- Boundary filtering strength bSHor is derived as follows:

bSHor = Max( bS[ 1 ][ xDk + i][ yB ] ) for i = 0..7 	(8-2)

- Filtering process for block edge in subclause  is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xB, yDk ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 0 and the boundary filtering strength bsHor as inputs and the modified luma picture buffer as output.

The filtering process for chroma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( log2CUSize – 3 ).

- For xDk set equal to ( xB / 2 )+( k << 2 ), k=0..nD - 1, the following applies:

- Filtering process for block edge in subclause  is invoked with the chroma location of the coding unit ( xC/2, yC/2), the chroma location of the block ( xDk, yB/2 ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- Filtering process for block edge in subclause  is invoked with the chroma location of the coding unit ( xC/2, yC/2), the chroma location of the block ( xDk, yB/2 ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- For yDk set equal to ( yB / 2 )+( k << 2 ), k=0..nD - 1, the following applies:

- Filtering process for block edge in subclause  is invoked with the chroma location of the coding unit ( xC/2, yC/2), the chroma location of the block ( xB/2, yDk ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSHor as inputs and the modified chroma picture buffer as output.

- Filtering process for block edge in subclause  is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xB/2, yDk ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSHor as inputs and the modified chroma picture buffer as output.

# Other mismatches in deblocking description

Several other mismatches in the deblocking description of Working Draft 1 [1] have been found. It is suggested to change the text according to the test model (or to the previous version of the standard text draft). Suggested changes are included.

#### Filtering process for coding unit

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process for luma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( log2CUSize – 3 ).

- For xDk set equal to xB+( k << 3 ), k=0..nD - 1, the following applies:

- Boundary filtering strength bSVer is derived as follows:

bSVer = Max( bS[ 0 ][ xDk ][ yB + i ] ) for i = 0..7 	(8-3)

- Filtering process for block edge in subclause  is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yB ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 0 and the boundary filtering strength bsVer as inputs and the modified luma picture buffer as output.

- For yDk set equal to yB+( k << 3 ), k=0..nD - 1, the following applies:

- Boundary filtering strength bSHor is derived as follows:

bSHor = Max( bS[ 1 ][ xDk + i][ yB ] ) for i = 0..7 	(8-4)

- Filtering process for block edge in subclause  is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xB, yDk ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 0 and the boundary filtering strength bsHor as inputs and the modified luma picture buffer as output.

The filtering process for chroma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( log2CUSize –3 ).

- For xDk set equal to ( xB / 2 )+( k << 2 ), k=0..nD - 1, the following applies:

- Filtering process for block edge in subclause  is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yB/2 ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- Filtering process for block edge in subclause  is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yB/2 ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- For yDk set equal to ( yB / 2 )+( k << 2 ), k=0..nD - 1, the following applies:

- Filtering process for block edge in subclause  is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xB/2, yDk ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSHor as inputs and the modified chroma picture buffer as output.

- Filtering process for block edge in subclause  is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xB/2, yDk ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSHor as inputs and the modified chroma picture buffer as output.

##### Filtering process for block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	a variable cIdx specifying the chroma component index.

Output of this process is:

–	modified reconstruction of the picture.

Let s’ be a variable specifying a luma or chroma sample array which is derived as follows.

–	If cIdx is equal to 0, s’ represents the luma sample array recPictureL of the current picture.

–	Otherwise, if cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following steps are performed

- The sample values pi,k and qi,k with i = 0..3 and k = 2, 5 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-5)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]		(8-6)

- The variable d is derived as follows:

d = | p2,2 – 2*p1,2 + p0,2 | + | q2,2 – 2*q1,2 + q0,2 | + | p2,5 – 2*p1,5 + p0,5 | + | q2,5 – 2*q1,5 + q0,5 | 	(8-7)

- The variable β is specified as Error: Reference source not found with luma quantization parameter qPL as input.

- If d is smaller than β, for each sample location ( xC + xB, yC + yB + k ), k = 0..7, the following ordered steps apply:

  - The sample values pi and qi with i = 0..3 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-8)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ] 		(8-9)

  - The filtering process for a sample specified in subclause 2.1.1.1.2 is invoked with sample values pi, qi, the chroma component index cIdx, the boundary filtering strength bS, d and β as inputs and the number of filtered samples nD and the filtered sample values pi’ and qi’ as outputs.

  - The filtered sample values pi’ and qi’ with i = 0..nD – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +i, yC + yB + k ] = qi’		(8-10)

s’[ xC + xB – i – 1, yC + yB + k ] = pi’		(8-11)

–	Otherwise (verticalEdgeFlag is equal to 0), for each sample location ( xC + xB +k, yC + yB ), k = 0..7, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 2, 5 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-12)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-13)

- The variable d is derived as follows:

d = | p2,2 – 2*p1,2 + p0,2 | + | q2,2 – 2*q1,2 + q0,2 | + | p2,5 – 2*p1,5 + p0,5 | + | q2,5 – 2*q1,5 + q0,5 | 		(8-14)

- The variable β is specified as Error: Reference source not found with luma quantization parameter qPL as input.

- If d is smaller than β then for each sample location ( xC + xB +k, yC + yB ), k = 0..7, the following ordered steps apply:

  - The sample values pi and qi with i = 0..3 are derived as follows:

qi = s’[ xC + xB +k, yC + yB + i ]		(8-15)

pi = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-16)

  - The filtering process for a sample specified in subclause 2.1.1.1.2 is invoked with sample values pi, qi, the chroma component index cIdx, the boundary filtering strength bS, d and β as inputs and the number of filtered samples nD and the filtered sample values pi’ and qi’ as outputs.

  - The filtered sample values pi’ and qi’ with i = 0..nD – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + i ] = qi’		(8-17)

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’		(8-18)

##### Filtering process for a sample

Inputs of this process are:

–	The sample values, pi and qi with i = 0..3,

–	a variable cIdx specifying the chroma component index,

–	a variable bS specifying the boundary filtering strength.

–	a variable d specifying spatial activity in the areas on both sides of the block boundary in the direction parallel to the direction of filtering.

–	a threshold β.

Output of this process is:

–	The number of filtered samples nD,

–	The filtered sample values, pi’ and qi’ with i = 0..nD – 1,

When the variable bS is not equal to 0, the following applies:

–	If cIdx is equal to 0, the filtering for luma sample is specified as the following ordered steps:

- If bS is greater than 2, the variable tC is specified as Error: Reference source not found with luma quantization parameter ( qPL + 4 ) as input otherwise the variable tC is specified as Error: Reference source not found with luma quantization parameter qPL as input.

- When d is less than β, the following applies:

- If d is less than ( β >> 2 ), | p3 – p0 | + | q0 – q3 | is less than ( β >> 3 ) and | p0 – q0 | is less than ( 5*tC + 1 ) >> 1, the following strong filtering applies while nD is set equal to 3:

p0’ = Clip1Y( ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )	(8-19)

p1’ = Clip1Y( ( p2 + p1 + p0 + q0 + 2 ) >> 2 )	(8-20)

p2’ = Clip1Y( ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 )	(8-21)

q0’ = Clip1Y( ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )	(8-22)

q1’ = Clip1Y( ( p0 + q0 + q1 + q2 + 2 ) >> 2 )	(8-23)

q2’ = Clip1Y( ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )	(8-24)

- Otherwise, the following weak filtering applies while nD is set equal to 2:

 = Clip3( -tC, tC, ( 13*( q0 – p0 ) + 4*( q1 – p1 ) - 5*( q2 – p0 ) + 16 ) >> 5 )	(8-25)

p0’ = Clip1Y( p0 +  )		(8-26)

q0’ = Clip1Y( q0 -  )		(8-27)

p1’ = Clip1Y( p1 + /2 )		(8-28)

q1’ = Clip1Y( q1 – /2 )		(8-29)

–	Otherwise (cIdx is not equal to 0), the filtering for chroma sample is specified as follows while nD is set equal to 1:

- When bS is greater than 2, the filtered sample values pi’ and qi’ are derived as follows:

 = Clip3( -tC, tC, ( ( ( ( q0 – p0 ) << 2 ) + p1 – q1 + 4 ) >> 3 ) )	(8-30)

p0’ = Clip1C( p0 +  )		(8-31)

q0’ = Clip1C( q0 -  )

# References

- T. Wiegand, W.-J. Han, J.-R. Ohm, G. J. Sullivan (editors), “High Efficiency Video Coding (HEVC) text specification Working Draft 1”, JCTVC-C403, JCT-VC Meeting, Guangzhou, Oct. 2010.