| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D397 |
| --- | --- |

| Title: | A New Adaptive Interpolation Filtering Technique |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Faouzi Kossentini Nader Mahdi Mohammed Ali Ben Ayed | Email: | faouzi@ebriskvideo.com nader@score.com.tn medali@score.com.tn |
| Source: | eBrisk Video Inc. |  |  |

_____________________________

# Abstract

In this contribution, an Adaptive Interpolation Filtering (AIF) technique using adaptive filters is proposed for interpolation filtering of the luminance samples of video sequences. The proposed technique allows the encoder to switch, for each of the three groups of positions, between the TMuC 0.9 fixed filter (12-tap and 6-tap for High Efficiency and Low Complexity profiles, respectively) and a newly-generated AIF filter. Compared to the TMuC 0.9, the proposed technique yields average BD-rate reductions of 0.6%-0.9% for the two Low Delay profiles and the Random Access / Low-Complexity profile.

# Introduction

This contribution describes an Adaptive Interpolation Filtering (AIF) technique using adaptive filters for interpolation filtering during the motion compensation part of video encoding/decoding. For each picture, the encoder is configured, for each group of sub-sample positions, to employ either the TMuC 0.9 fixed filter or a newly-generated filter. If the newly-generated filter is selected for the considered group of sub-sample positions, its coefficients are also sent to the decoder.

# AIF Using Adaptive Filters

In this section, we present some details on the proposed AIF technique, including the selection of an AIF filter for each of the sub-sample positions, the generation of the AIF filter coefficients and their transmission to the decoder.

## AIF Filter Selection

The proposed AIF technique, although based partly on the Switched Interpolation Filtering with Offsets (SIFO) technique [1], is quite different. Instead of selecting, for each sub-sample position, among several predefined (fixed) interpolation filters, the proposed technique involves a selection, for each of the three groups of ¼, ½ and ¾ sub-sample positions (shown in Figure 1 and Figure 2 below), between the corresponding predefined TMuC 0.9 filter and a newly-generated filter. The three predefined filters used for the High-Efficiency profiles are shown in Table 1, those for the Low- Complexity profiles are shown in Table 2. The three newly-generated filters used for the High-Efficiency profiles are shown in Table 3, those for the Low-Complexity profiles are shown in Table 4. The selection process is based on the Lagrangian method. More specifically, for each one of three groups of ¼, ½ and ¾ sub-sample positions, a Lagrangian cost is computed by adding the mean square error between the source sample values and the filtered and reconstructed sample values (with each of the two filters), plus a bias that is a function, through a Lagrangian multiplier, of the number of bits required to convey the filter or filter reference to the decoder. The filter with the lowest Lagrangian cost is selected.

Figure 1: ¼, ½ and ¾ groups of sub-sample positions for the High-Efficiency profiles

Figure 2: ¼, ½ and ¾ groups of sub-sample positions for the Low-Complexity profiles

Table 1: Predefined (TMuC 0.9) 12-tap separable filters for the High-Efficiency profiles

| ¼ | {-1, 5, -12, 20,-40, 229, 76, -32, 16, -8, 4,-1 } |
| --- | --- |
| ½ | {-1, 8, -16, 24,-48, 161, 161, -48, 24, -16, 8,-1 } |
| ¾ | {-1, 4, -8, 16, -32, 76, 229, -40, 20, -12, 5,-1 } |

Table 2: Predefined (TMuC 0.9) 6-tap directional filters for the Low-Complexity profiles

| ¼ | {3, -15, 111, 37, -10, 2 } |
| --- | --- |
| ½ | {3, -17, 78, 78, -17, 3 } |
| ¾ | {2, -10, 37, 111, -15, 3 } |

Table 3: Newly-generated 12-tap separable filters for the High-Efficiency profiles

| ¼ | {a0, a1, a2, a3, a4, a5, a6, a7, a8, a9, a10, a11 } |
| --- | --- |
| ½ | {b0, b1, b2, b3, b4, b5, b5, b4, b3, b2, b1, b0 } |
| ¾ | {a11, a10, a9, a8, a7, a6, a5, a4, a3, a2, a1, a0} |

Table 4: Newly-generated 6-tap directional filters for the Low-Complexity profiles

| ¼ | {a0, a1, a2, a3, a4, a5 } |
| --- | --- |
| ½ | {b0, b1, b2, b2, b1, b0 } |
| ¾ | {a5, a4, a3, a2, a1, a0 } |

## Generation of the Filter Coefficients

There are many ways of generating, for each video picture or video slice, the coefficients of the three AIF adaptive filters. The encoder is free to employ any method, as long as the newly-generated filters have the same structure shown in Table 2.

In this contribution, we employ the filter-coefficients estimation method used in [2]. This method consists of the following steps:

Step 1: The displacement vectors are first estimated using the TMuC 0.9 12-tap filters.

Step2: The values of the filter coefficients are estimated (through linear regression) by minimizing the energy of the prediction errors after performing motion-compensated prediction using the displacement vectors from Step 1.

## Transmission of the Filter Coefficients

The coefficients of each newly-generated AIF adaptive filter are encoded using 8-bit (LC profiles) or 9-bit (HE profiles) quantization of the differences between the values of the AIF filter coefficients and the corresponding values of the TMuC 0.9 filter coefficients.

## Syntax Description

As described above, our objective is to allow the use of a newly-generated filter for the interpolation in the motion estimation/compensation process. Therefore, the syntax in the slice header is also changed as follows to support the new AIF. The use_generated_filter_flag is inserted in the slice header to indicate whether the newly-generated filter is used. In the case where the use_generated_filter_flag is equal to 1, three additional flags should be read (one flag for each group; use_generated_filter_group0_flag, use_generated_filter_group1_flag, and use_generated_filter_group2_flag). These flags are sent to specify whether the generated filter is used for each of the considered groups. When one of them is equal to 1, it will be followed by the filter coefficients and the positions where the generated filter should be applied. Note that for the case where the use_generated_filter_group0_flag is equal to 1 and the use_generated_filter_group2_flag is also equal to 1, there are no additional coefficients to read for group 2. In fact, in this case, the decoder jumps directly to read the positions where the generated filter will be applied since the required filter coefficient values are identical to those read for group 0.

Table 5: Syntax definition for the proposed AIF technique

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| use_generated_filter_flag | 2 | u(1) |
| if (use_generated_filter_flag) { |  |  |
| use_generated_filter_group0 | 2 | u(1) |
| if (use_generated_filter_group0) { |  |  |
| for (i =0; i < filter_size; i ++ ) { |  |  |
| diff_filter_coef0_2[i]; | 2 | se(v) |
| } |  |  |
| for (sub_pos =0; sub_pos < num_pos_in_group0; sub_pos ++ ) { |  |  |
| filter_index_group0[sub_pos]; | 2 | u(1) |
| } |  |  |
| } |  |  |
| use_generated_filter_group1 | 2 | u(1) |
| if (use_generated_filter_group1) { |  |  |
| for (i =0; i < filter_size; i ++ ) { |  |  |
| diff_filter_coef1[i]; | 2 | se(v) |
| } |  |  |
| for (sub_pos =0; sub_pos < num_pos_in_group1; sub_pos ++ ) { |  |  |
| filter_index_group1[sub_pos]; | 2 | u(1) |
| } |  |  |
| } |  |  |
| use_generated_filter_group2 | 2 | u(1) |
| if (use_generated_filter_group2) { |  |  |
| if (! use_generated_filter_group0) { |  |  |
| for (i =0; i < filter_size; i ++ ) { |  |  |
| diff_filter_coef0_2[i]; | 2 | se(v) |
| } |  |  |
| } |  |  |
| for (sub_pos =0; sub_pos < num_pos_in_group2; sub_pos ++ ) { |  |  |
| filter_index_group2[sub_pos]; | 2 | u(1) |
| } |  |  |
| } |  |  |
| } |  |  |
| --- |  |  |
| } |  |  |

# Simulation Results

The simulation experiments were conducted following the testing conditions in JCTVC-B300 [3] and JCTVC-B310_r3 [4]. The coding efficiency (BD-Rate) numbers were obtained for the Low Delay profiles as well as the Random Access / Low-Complexity profile (Table 6). The proposed AIF technique was compared to the TMuC 0.9 technique. The detailed BD-Rate results of for the Luminance component are shown in Table 7.

The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: Intel Core 2 Quad Q8300 @2.50 GHz.

- RAM: 4 Go

- OS:  Windows XP Professional.

It is clear, from the tables, that the proposed technique provides coding efficiency gains over the TMuC 0.9 fixed filtering technique. Note that the coding efficiency gains decrease for the large video sequences (Classes E, A, and B). This is expected because the newly-generated AIF filters (with the pictures being too large) do not provide enough adaptivity to provide significant coding efficiency gains when compared to the predefined filters. A solution could be to split each picture into multiple slices and then generating three AIF filters for each slice. The corresponding simulation results will be presented at the next meeting.

The required additional complexity at the encoder and/or decoder can be quite small. Except for the decoding of two distinct AIF filters (the ¼ and ¾ AIF filters employ the same coefficients, though in opposite order) for each video picture/slice (with this being the worst-case scenario), there is no additional decoder complexity. Moreover, except for the encoding of the two distinct AIF filters for each video picture/slice, the added complexity at the encoder depends on the application at hand. In fact, the new technique can be used to send two special AIF filters for each slice, picture, group of pictures or even for a set of video sequences that have similar characteristics (e.g., in the case of a special application space).

Table 6: Simulation Results (Summary)

|  | Random Access LoCo |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |  |  |
| Class A | - |  | - |  | - |  |  |  |  |
| Class B | - |  | - |  | - |  |  |  |  |
| Class C | -0.6 |  | -0.2 |  | -0.4 |  |  |  |  |
| Class D | -0,. |  | -0.4 |  | -0.2 |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| All | -0.55 |  | -0.3 |  | -0.3 |  |  |  |  |
| Enc Time[%] | 150% |  |  |  |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  |  |  |  |
|  | Low Delay |  |  |  |  | Low Delay LoCo |  |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.6 | -0.5 |  | -0.6 |  | - |  | - | - |
| Class C | -1.1 | -0.6 |  | -0.7 |  | -0.7 |  | -0.6 | -0.4 |
| Class D | -1.3 | -0.9 |  | -0.5 |  | -0.6 |  | -1.1 | -1.2 |
| Class E | -0.5 | -0.2 |  | -0.4 |  | - |  | - | - |
| All | -0.9 | -0.6 |  | -0.6 |  | -0.65 |  | -0.85 | -0.8 |
| Enc Time[%] | 135% |  |  |  |  | 140% |  |  |  |
| Dec Time[%] | 100% |  |  |  |  | 101% |  |  |  |

Table 7: Detailed Y BD-Rate Results

| Sequences | Random access (LoCo) | LowDelay | LowDelay (LoCo) |
| --- | --- | --- | --- |
| Traffic | - |  |  |
| PeopleOnStreet | - |  |  |
| Kimono | - | -0.4 | - |
| ParkScene | - | -0.3 | - |
| Cactus | - | -0.1 | - |
| BasketballDrive | - | -0.6 | - |
| BQTerrace | - | -1.7 | - |
| BasketballDrill | -0.2 | -0.5 | -0.7 |
| BQMall | -0.6 | -0.7 | -1.1 |
| PartyScene | -1.3 | -2.6 | -1.0 |
| RaceHorses | -0.2 | -0.4 | -0.2 |
| BasketballPass | -0.1 | -0.3 | -0.1 |
| BQSquare | -1.4 | -3.6 | -1.5 |
| BlowingBubbles | -0.5 | -1.3 | -0.7 |
| RaceHorses | -0.1 | -0.2 | -0.1 |
| Vidyo1 |  | -0.4 | - |
| Vidyo3 |  | -0.5 | - |
| Vidyo4 |  | -0.7 | - |

# Conclusion and Recommendation

The proposed technique provides coding efficiency gains of 0.6%-0.9% over the TMuC 0.9 fixed filtering technique, while also requiring essentially the same decoder complexity. The additional encoder complexity can vary greatly, depending on the video application. In some cases, the new AIF adaptive filters can be decided off-line, and the encoder will only need to encode and transmit at most two filters. In other cases, the encoder will generate the coefficients as part of a pre-analysis phase.

We recommend that the JCT-VC continue the evaluation of this new AIF technique for possible adoption into the HM. More specifically, we recommend the verification of our simulation experiments and the harmonization with other AIF related tools.

# References

- M. Karczewicz, Y. Ye, and Peisong Chen, “Switched interpolation filter with offset,” ITU-T/SG 16, VCEG-AI35, Jul. 2008.

- S. Wittmann, T. Wedi, “Separable Adaptive Interpolation Filter,” ITU-T SG16/Q.6 Doc. T05-SG16-C-0219, Geneva, Switzerland, June 2007.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-B300, Geneva, July 2010.

- T. Yamakage, T. Chujoh, S. Lei, K. Chono, M. Karczewicz, A. Segall, T. Yamamoto, B. Jeon, J. Lim, J. Xu, “Tool Experiment 10: In-loop filtering”, JCTVC-B310_r3, Geneva, July 2010.

# Patent rights declaration(s)

eBrisk Video Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).