| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D398 |
| --- | --- |

| Title: | CE6: Cross verification of Samsung chroma intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Muhammed Coban 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1 858 658 3973 mcoban@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This document reports cross verification results for CE6 [1] : Samsung’s chroma intra prediction by reconstructed luma samples JCT-VC C206 [2] (T4). T4 addresses chroma intra mode coding improvement and encoding complexity reduction by using only DM and DC mode. Qualcomm’s cross verification results match those produced by Samsung.

# Introduction

This document reports cross verification results for CE6: Samsung’s chroma intra prediction by reconstructed luma samples (T4). T4 addresses chroma intra mode coding improvement and encoding complexity reduction by using only DM and DC mode. In DM, chroma mode is derived from luma angular mode.

# Results

Cross verification results are summarized in Table 1. Rate-distortion measurements using BD-Rate for proposed tools have been performed [3] . Encoding and decoding times are measured cluster run times.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | -0.3 | 0.0 | -0.7 | 0.4 | 1.4 |
| Class B | -0.6 | 0.3 | 0.4 | -0.7 | 0.6 | 1.4 |
| Class C | -0.6 | 0.5 | 0.6 | -0.5 | 0.7 | 0.9 |
| Class D | -0.4 | 0.8 | 0.7 | -0.3 | 1.0 | 0.9 |
| Class E | -1.0 | -0.2 | -0.2 | -0.6 | 0.8 | 1.9 |
| All | -0.6 | 0.3 | 0.4 | -0.5 | 0.7 | 1.3 |
| Enc Time[%] | 82% |  |  | 95% |  |  |
| Dec Time[%] | 83% |  |  | 102% |  |  |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | 0.2 | 0.9 | -0.4 | 1.0 | 2.1 |
| Class B | -0.3 | 0.5 | 0.9 | -0.3 | 1.1 | 1.5 |
| Class C | -0.2 | 0.4 | 0.7 | -0.2 | 1.2 | 1.5 |
| Class D | -0.2 | 0.9 | 1.2 | -0.1 | 1.6 | 1.3 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | 0.5 | 0.9 | -0.2 | 1.2 | 1.5 |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 93% |  |  | 111% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.3 |
| Class C | -0.1 | -0.3 | -0.1 | 0.0 | -0.1 | 0.0 |
| Class D | 0.0 | -0.6 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class E | -0.2 | 0.3 | -0.2 | -0.1 | 0.6 | 1.7 |
| All | -0.1 | -0.2 | 0.0 | 0.0 | 0.1 | 0.4 |
| Enc Time[%] | 95% |  |  | 106% |  |  |
| Dec Time[%] | 99% |  |  | 116% |  |  |

Table 1: Experimental results

The computing platform is summarized in Table 2.

|  | Decoding | Encoding |
| --- | --- | --- |
| OS | Linux (64bit) | Linux (64bit) |
| Compiler | gcc 4.1.2 | gcc 4.1.2 |

Table 2: Computing platform

# Conclusion

Cross verification results of CE6: Samsung JCT-VC C206 chroma intra prediction by reconstructed luma samples was reported. The results match those produced by Samsung, and the software was inspected.

# Reference

- “Core Experiment 6: Intra Prediction Improvement”, JCTVC-C506, 3rd JCT-VC Meeting, Guangzhou, CN, October 2010.

- “Chroma intra prediction by reconstructed luma samples”, JCTVC-C206, 3rd JCT-VC Meeting, Guangzhou, CN, October 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).