| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D399 |
| --- | --- |

| Title: | CE7: Mode dependent intra residual coding analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peisong Chen, Rajan Joshi, Yufei Zheng, Muhammed Coban, Marta Karczewicz | Tel: Email: | 1-858-845-1347 peisongc@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In this contribution, we compare several different methods of mode dependent intra residual coding. They are mode dependent fix scan,  adaptive switch scan and adaptive switch KLT+scan. We will show the trade off between coding efficiency and complexity.

# Mode dependent residual coding

We use one dimensional directional unified transform (1DDUT) proposed in [1]. It utilizes only two 1-D transform matrices, which allows four combinations when we do 2-D separable transform and each direction can pick one of the two 1-D transform matrices.  A combination is selected based on the intra prediction mode or the scan order depending on different methods.  All three methods introduced in this contribution use only three scan orders: zig-zag, horizontal and vertical.

For each method, the coding efficiency is evaluated in Intra, RandomAccess (RA) and LowDelay (LD) configurations for the high efficiency case based on the common conditions defined in JCTVC-C500[2]. The coding efficiency is measured by using 4 point BD-Rate.

## Mode dependent fix scan

In this method, there are three scan orders: zig-zag, horizontal and vertical. Which scan order to use depends on the transform block size and the spatial prediction direction. There is no need to signal the selection of the scan order in the bitstream.

Table 1: Mode dependent fix scan

## Adaptive switch scan

In this scheme, there are still three scan orders which is the same as the previous method. But the difference is that the selection of the scan order is explicitly signaled in the bitstream. Therefore, the encoder has to test each scan order to determine which is the best in the sense of RD cost.

Table 2: Adaptive switch scan

## Adaptive switch KLT+scan

In this method, the selection of transform is associated to the scan order. If the zig-zag scan is used, the transform to be used depends on the intra prediction mode, which is the same as Mode dependent fix scan described in section 1.1. However, if one of the other two scan orders is used, then the selection of transform only depends on whether it is the horizontal scan or the vertical scan. Which of the three scan orders to be used is decided based on RD cost.

Table 3: adaptive switch KLT+scan

# Reference

- A. Tanizawa, et al, “TE7: One-dimensional directional unified transform”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C080, 3rd Meeting: Guangzhou, CN, Oct., 2010.

- F. Bossen, “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, CN, Oct., 2010.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).