| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D400 |
| --- | --- |

| Title: | CE11: Cross verification of Motorola’s zigzag scan for CABAC/PIPE |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Muhammed Coban 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1 858 658 3973 mcoban@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This document reports cross verification results for CE11 [1] : Motorola’s zigzag scan for CABAC/PIPE JCT-VC C114 [2] . JCTVC-C114 proposes use of zigzag scanning pattern for significance map coding. Qualcomm’s cross verification results match those produced by Motorola.

# Introduction

This document reports cross verification results for CE11: Motorola’s zigzag scan for CABAC/PIPE. This proposal addresses use of zigzag scanning pattern for significance map coding.

# Results

Cross verification results are summarized in Table 1. Rate-distortion measurements using BD-Rate for proposed tools have been performed [3] . Encoding and decoding times are measured cluster run times. The number of bins used for coding of significant_coeff_flag syntax element is reported.

|  | Intra |  |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.5 | 0.4 | 2.30% | 0.0 | 0.0 | 0.0 |
| Class B | 0.2 | 0.4 | 0.5 | 2.46% | 0.0 | 0.0 | 0.0 |
| Class C | 0.4 | 0.6 | 0.6 | 2.64% | 0.0 | 0.0 | 0.0 |
| Class D | 0.4 | 0.6 | 0.6 | 2.40% | 0.0 | 0.0 | 0.0 |
| Class E | 0.4 | 0.4 | 0.2 | 3.80% | 0.0 | 0.0 | 0.0 |
| All | 0.3 | 0.5 | 0.5 | 2.72% | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 83% |  |  |  | 105% |  |  |
| Dec Time[%] | 85% |  |  |  | 103% |  |  |

|  | Random access |  |  |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.6 | 0.6 | 2.09% | 0.0 | 0.0 | 0.0 |
| Class B | 0.2 | 0.2 | 0.5 | 2.09% | 0.0 | 0.0 | 0.0 |
| Class C | 0.3 | 0.3 | 0.3 | 2.34% | 0.0 | 0.0 | 0.0 |
| Class D | 0.2 | 0.4 | 0.6 | 2.15% | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |  |
| All | 0.2 | 0.3 | 0.5 | 2.17% | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 93% |  |  |  | 97% |  |  |
| Dec Time[%] | 88% |  |  |  | 106% |  |  |

|  | Low delay |  |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |  |
| Class B | 0.2 | 0.3 | 0.3 | 2.18% | 0.0 | 0.0 | 0.0 |
| Class C | 0.2 | 0.2 | 0.2 | 2.67% | 0.0 | 0.0 | 0.0 |
| Class D | 0.2 | -0.4 | 0.6 | 2.06% | 0.0 | 0.0 | 0.0 |
| Class E | 0.1 | 0.6 | 0.6 | 4.37% | 0.0 | 0.0 | 0.0 |
| All | 0.2 | 0.2 | 0.4 | 2.82% | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 92% |  |  |  | 95% |  |  |
| Dec Time[%] | 88% |  |  |  | 99% |  |  |

Table 1: Experimental results

The computing platform is summarized in Table 2.

|  | Decoding | Encoding |
| --- | --- | --- |
| OS | Linux (64bit) | Linux (64bit) |
| Compiler | gcc 4.1.2 | gcc 4.1.2 |

Table 2: Computing platform

# Conclusion

Cross verification results of CE11: Motorola JCT-VC C114 zigzag scan for CABAC/PIPE was reported. The results match those produced by Motorola, and the software was inspected.

# Reference

- “Core Experiment 11: Coefficient scanning and coding”, JCTVC-C511, 3rd JCT-VC Meeting, Guangzhou, CN, October 2010.

- “Zigzag scan for CABAC/PIPE”, JCTVC-C114, 3rd JCT-VC Meeting, Guangzhou, CN, October 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).