| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D401 |
| --- | --- |

| Title: | Comments on Generalized P and B Pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen, Coban Muhammed, Wei-Jung Chien, and Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-845-6589 cheny@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Generalized P and B (GPB) pictures are supported in the working draft of HEVC. Comments on the GPB pictures are given in this proposal, on the related modifications for slice level syntax elements and decoding processes for such a picture.

# Introduction

According to the current working draft, a GPB slice has two reference picture lists and they are identical. Each prediction unit of a GPB slice may either be uni-directional predicted from reference picture list 0, or bi-directional predicted.

The current design considers a GPB slice as a B slice.

# Problem

The identified problems of the current GPB design are as follows.

- No explicit way to indicate whether a B slice is a GPB slice or a normal B slice. It is necessary for the decoder to indentify if a slice is a GPB slice since the parsing of the syntax element (inter_pred_idc) in prediction unit relies on it. So, for each slice, a comparison of the reference picture list 0 and reference picture list 1 must be done to check if a slice is a GPB slice.

- Given the information of reference picture list 0, the signaling of the information of reference picture list 1 for a GPB slice is redundant, since the reference picture list 1 and the reference picture list 0 are identical anyway. However, such a redundant signaling is not prohibited in the current GPB design. The redundantly signaled syntax elements include num_ref_idx_l1_active_minus1 and the reference picture list modification (RPLM) table. They both take a certain amount of bits which are useless.

- For a GPB slice, the decoding processes for reference picture list construction of the list 1 are useless.

# Proposal

We believe that the current GPB slice design has defects and suggest resolve above problems properly.

## Indicating a GPB slice

One straightforward way of solving the problem 1) and 2) is to consider the GPB slice still as a B slice and introduce a flag in the slice header. However, considering the simplicity of the specification, this solution is not recommended. It requires e.g., modifications of the conditions for the signaling of num_ref_idx_l1_active_minus1, and the reference picture list modification, illustrated as follows:

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_lctb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| frame_num | 2 | u(v) |
| if( IdrPicFlag ) |  |  |
| idr_pic_id | 2 | ue(v) |
| pic_order_cnt_lsb | 2 | u(v) |
| if ( slice_type = = B ) |  |  |
| slice_gpb_flag | 1 | u(1) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B && !slice_gpb_flag) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| ... |  |  |

| ref_pic_list_modification( ) { | C | Descriptor |
| --- | --- | --- |
| if( slice_type % 5 != 2 ) { |  |  |
| ref_pic_list_modification_flag_l0 | 2 | u(1) |
| if( ref_pic_list_modification_flag_l0 ) |  |  |
| do { |  |  |
| modification_of_pic_nums_idc | 2 | u(1) |
| if(modification_of_pic_nums_idc = = 0 ¦ ¦ modification_of_pic_nums_idc = = 1 ) |  |  |
| abs_diff_pic_num_minus1 | 2 | ue(v) |
| else ( modification_of_pic_nums_idc = = 2 ) |  |  |
| long_term_pic_num | 2 | ue(v) |
| } while( modification_of_pic_nums_idc != 3 ) |  |  |
| } |  |  |
| if( slice_type % 5 = = 1 && !slice_gbp_flag ) { |  |  |
| ref_pic_list_modification_flag_l1 | 2 | u(1) |
| if( ref_pic_list_modification_flag_l1 ) |  |  |
| do { |  |  |
| modification_of_pic_nums_idc | 2 | u(1) |
| if(modification_of_pic_nums_idc = = 0 ¦ ¦ modification_of_pic_nums_idc = = 1 ) |  |  |
| abs_diff_pic_num_minus1 | 2 | ue(v) |
| else if( modification_of_pic_nums_idc = = 2 ) |  |  |
| long_term_pic_num | 2 | ue(v) |
| } while( modification_of_pic_nums_idc != 3 ) |  |  |
| } |  |  |
| } |  |  |

Similar modifications might be need in other places in the specification, e.g., prediction weight table.

To avoid such modifications of the conditions in various places, it is proposed that a new slice type, namely GPB slice type can be introduced, that only the following modifications in the slice header is needed. Note that such a GPB slice type (denoted as GPB_type) should satisfy the following: GPB_type% 5 != 2 and GPB_type% 5 != 1.

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_lctb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| frame_num | 2 | u(v) |
| if( IdrPicFlag ) |  |  |
| idr_pic_id | 2 | ue(v) |
| pic_order_cnt_lsb | 2 | u(v) |
| if( slice_type = = P ¦¦ slice_type = = GPB ¦ ¦ slice_type = = B ) { |  |  |
| … |  |  |

We may also consider the GPB as a P slice. In such a case, similarly a flag needs to be added in the slice header. However, if traditional uni-directional prediction P slices are foreseen to coexist with GPB slices, confusion might be introduced, in terms of the meaning of “P”, which by default is considered as uni-directional.

## Avoiding decoding processes for list 1 construction

It is also proposed that the decoding processes for reference picture list construction in the current HEVC WD should be modified accordingly such that when a slice is indentified as a GPB slice, no reference picture construction processes for the reference picture list 1 will be invoked.

# Conclusion

In this contribution, we identified problems in the high level syntax design of GPB slices and proposed the following solutions: 1) To use a new slice type for GPB slices to avoid the signaling of the redundant syntax elements and complicated modifications of the HEVC specification; 2) To avoid reference picture list construction for list 1 when the slice is a GPB slice.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).