| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D403 WG11 Number: m19182 |
| --- | --- |

| Title: | CE5: Cross-check of Qualcomm/Cisco proposal on improved intra prediction mode coding in LCEC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yih Han Tan Chuohao Yeo Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 yhtan@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution provides a summary of verifications performed by the Institute for Infocomm Research on Qualcomm/Cisco’s proposal on improved intra prediction mode coding in LCEC. The simulation results match the results provided by Qualcomm/Cisco.

- Introduction

In Qualcomm/Cisco’s proposal [1], coding of intra prediction modes is modified for 4x4 (17 modes), 8x8, 16x16 and 32x32 blocks (34 modes) in LCEC. In the reference, the first binary symbol decoded determines whether the most probable mode is chosen. If the intra-prediction mode is not the most probable mode, a fixed length code is used to code the prediction mode, with the length of the code determined by the total number of prediction modes for the decoded PU.

In the proposed approach, one of two sets of variable length codes can be used to code a PU’s prediction mode. The selection depends whether intra prediction modes of the PU’s left and above neighboring blocks are same.

The adaptive mapping table that maps the decoded code numbers to the intra-prediction directions is updated after the decoding of each intra-prediction directions. One table is used by the 4x4 PUs while another table is shared by the 8x8, 16x16 and 32x32 PUs.

# Simulation Results

The test conditions as specified in the common test conditions are followed [2] and tests were conducted on all the low-complexity coding configurations as stipulated by CE5 [3]. The implementation is on TMuC 0.9.

## RD results

Table 1 shows the BD-Rate [4] results for the proposed scheme vs the TMuC 0.9 anchors.

Table 1. BD-Rate results for proposed intra prediction mode coding [1]

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -0.4 | -0.3 |
| Class B | -0.5 | -0.2 | -0.2 |
| Class C | -0.5 | -0.2 | -0.1 |
| Class D | -0.4 | -0.1 | -0.1 |
| Class E | -0.5 | -0.3 | -0.3 |
| All | -0.5 | -0.2 | -0.2 |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.1 | 0.1 |
| Class B | -0.2 | -0.1 | -0.1 |
| Class C | -0.2 | -0.1 | 0.0 |
| Class D | -0.1 | -0.2 | 0.0 |
| Class E |  |  |  |
| All | -0.2 | -0.1 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.0 | -0.1 | -0.2 |
| Class C | 0.0 | -0.1 | 0.0 |
| Class D | -0.1 | 0.1 | -0.1 |
| Class E | -0.1 | -0.2 | 0.3 |
| All | 0.0 | -0.1 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

More detailed coding results can be found in the Excel attachment.

## Complexity

There is no significant change in encoding or decoding time.

# Conclusion

We have cross-checked the intra prediction mode coding with LCEC proposal from Qualcomm and Cisco, studied the algorithm and verified their coding simulation results.

# References

[1] M. Karczewicz,  X. Wang, W.-J. Chen , A. Fuldseth, “CE5: Improved intra prediction mode coding with LCEC,” JCTVC-D366, Daegu, Korea, Jan 2011.

[2] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-C500, Guangzhou, China, Oct 2010.

[3] X. Wang, “Low complexity entropy coding improvements,” JCTVC-C505, Guangzhou, China, Oct 2010.

[4] Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.

For avoidance of doubt, positive BD-Rate implies that the proposed method uses more rate, while a negative BD-Rate implies that the proposed method improves coding performance.