| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January, 2011 | Document: JCTVC-D406 WG11 Number: m19186 |
| --- | --- |

| Title: | Cross-verification of SONY's proposal on low complexity parametric adaptive loop filter (JCTVC-D270) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document reports cross-check results of SONY’s proposal on low complexity parametric adaptive loop filter (JCTVC-D270). It is verified that the coding efficiency data for low complexity configurations that SONY provided is accurate. It is recommended to carry out CEs on the ALF proposals including the PALF to seek a new ALF design which addresses the ALF complexity concerns on both the encoder and decoder sides.

# Introduction

This document reports cross-check results of SONY’s proposal on low complexity parametric adaptive loop filter (JCTVC-D270) [2]. Only the LOCO configurations are tested as requested by proponents.

# Algorithm Description

Instead of estimating and transmitting wiener filter coefficients for each frame, the parametric adaptive loop filter (PALF) [2] uses a pre-defined set of Wiener filters, and transmits only the filter index at LCU level to achieve the goal of adaptive wiener filtering. Compared to the current HM1 ALF, the PALF has potential of improving the coding efficiency by transmitting filter indices instead of filter coefficients, and reducing encoder side complexity by using single pass to determine the best filter for each LCU. The following table summarizes the pre-defined Wiener filters used in the proposed PALF algorithm:

| Wiener filters | Number of filters | Filter size | Filter coefficient precision | Remark |
| --- | --- | --- | --- | --- |
| Luma (high efficiency) | 32 | 9 x 9 | 18 -bit | Non-separable, symmetric, effective 41 tap filter |
| Luma (low-complexity) | 32 | 9 x 9 | 18 -bit | Non-separable, symmetric, effective 41 tap filter |
| chroma | 32 | 5 x 5 | 18-bit | Non-separable, asymmetric, effective 25 tap filter |

For the low-complexity configurations, 32 luma (low-complexity) and 32 chroma pre-trained wiener filters are used.

# Test Conditions

The TMuC-0.9 software is used for evaluation; the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and low-complexity reference configurations specified in [1] are followed, except that the ALF flag is set to 1 in the low-complexity configurations for the verification purpose.

# Testing Results

## Comparison results against data provided by SONY

Perfect match has achieved when compared to the LOCO data provided by SONY. Please refer to TMuC0 9-TI.vs.Sony-palf010511v05.xlsm for detail.

Due to lack of anchor data for TMuC-0.9 LOCO configurations with ALF on, the coding efficiency gain of the proposed PALF algorithm against the HM1 ALF for the low-complexity configurations cannot be quantified in this document.

## Comparison results between TMuC-0.9 LOCO w/ ALF on and SONY PALF

The PALF is compared with the TMuC-0.9 anchor with the ALF turned on in the low complexity configurations. As shown in table below, compared to the ALF the PALF provides about 20% speedup, but leads to 3.1% and 1.4% loss in the RA and LD LOCO configurations, respectively.

For the detailed results please refer to TMuC0.9-LC_ALF_anchorvs.SONY_PALF.xls.

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 3.3 | 0.9 | 1.2 |
| Class B | 1.5 | 1.0 | 0.3 |
| Class C | 0.8 | 1.2 | 1.4 |
| Class D | 0.6 | 1.6 | 2.0 |
| Class E | 2.6 | 1.5 | 1.5 |
| All | 1.5 | 1.2 | 1.2 |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 92% |  |  |
|  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2.0 | -0.5 | -0.1 |
| Class B | 2.1 | -0.8 | -2.6 |
| Class C | 2.5 | -2.4 | -1.9 |
| Class D | 5.4 | -0.8 | -1.2 |
| Class E |  |  |  |
| All | 3.1 | -1.2 | -1.7 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 79% |  |  |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 1.1 | -3.2 | -4.5 |
| Class C | 2.0 | -4.1 | -5.1 |
| Class D | 2.7 | -1.5 | -1.4 |
| Class E | -0.9 | -4.3 | -6.6 |
| All | 1.4 | -3.2 | -4.3 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 81% |  |  |

# Conclusions

It has verified that the coding efficiency data of PALF for low-complexity configurations that SONY provided is accurate.

While the proposed parametric adaptive loop filter algorithm has the potential of improving the coding efficiency and reducing the encoder side complexity, its decoder complexity deserves further investigation. The large number of pre-defined wiener filters (i.e. 32 9x9, 32 5x5), longer filter tap length (i.e. 41 tap and 25 tap), and higher precision of filter coefficients (i.e. 18 bits) could make the PALF more expensive than the HM1 ALF on the decoder side. It is recommended to carry out CEs on the ALF proposals including the PALF to seek a new ALF design which addresses the both encoder and decoder ALF complexity concerns raised in the various contributions.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, 3rd. JCT-VC Meeting, Guangzhou, CN, October 2010.

- E. Maani, W. Liu and L. Dong, “Low complexity parametric adaptive loop filter”, JCTVC-D270, 4th JCT-VC Meeting, Daegu, KR, January 2011