| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D407 |
| --- | --- |

| Title: | Verification of mode dependent intra residual coding (JCTVC-D392) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Seungwook Park Jaehyun Lim Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | 82-2-6272-9179 seungwook.park@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution reports a verification result of mode dependent intra residual coding by Qualcomm, TOSHIBA, Institute for Infocomm Research and Huawei [1]. Qualcomm provided the source code, draft proposal and RD results. We inspected their source code comparing with their proposal draft and compiled it properly. The test has been done without any mismatch between encoder and decoder reconstruction data. We observed the identical RD result as the results in the excel sheet provided by Qualcomm.

# Verification

We have inspected the source code comparing with the draft proposal provided by Qualcomm and verified that software coincided exactly with their proposal. In detail, source code regarding the proposed method consists of mainly four parts. First part is a mapping table between 34 modes to 4 combinations of proposed 2-D transform. This part is defined by TSB_1DDUT_MAPPING & SIMPLIFY_KLT macros. Second part is related to a scheme of a kernel combination between DST and DCT, which is defined by I2R_DCTSDT. Third part is a coefficient scanning schemes, which is defined QC_COEFF_SCAN macro. And the remaining part is related to a performing the actual transforming process and it is defined by SMALL_KLT marco.

We have compiled the source code and tested it on Windows XP64 and Intel i7 950 (3 GHz) with 12G RAM.

# Results

Table 1~ 3 is the detail experimental results.

Table 1. RD result and complexity increase of Intra Only

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.9 | -3.4 | -3.5 | -3.8 | -3.1 | -3.0 |
| Class B | -1.4 | -2.2 | -2.2 | -2.4 | -3.0 | -3.2 |
| Class C | -2.1 | -2.2 | -2.2 | -2.1 | -2.2 | -2.2 |
| Class D | -2.1 | -2.0 | -2.0 | -1.8 | -1.7 | -1.7 |
| Class E | -2.9 | -3.6 | -3.3 | -3.7 | -3.9 | -3.8 |
| All | -2.1 | -2.5 | -2.5 | -2.6 | -2.7 | -2.7 |
| Enc Time[%] | 110%(+4%) |  |  | 113%(-1%) |  |  |
| Dec Time[%] | 105%(0%) |  |  | 113%(-3%) |  |  |

Table 2. RD result and complexity increase of Random Access

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.2 | -1.0 | -0.8 | -1.3 | -0.6 | -0.7 |
| Class B | -0.8 | -1.0 | -1.0 | -1.0 | -1.1 | -1.1 |
| Class C | -1.0 | -0.9 | -1.0 | -0.7 | -0.7 | -0.8 |
| Class D | -0.9 | -0.5 | -0.2 | -0.5 | -0.4 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.9 | -0.8 | -0.7 | -0.8 | -0.7 | -0.8 |
| Enc Time[%] | 103%(+2%) |  |  | 101%(0%) |  |  |
| Dec Time[%] | 101%(+1%) |  |  | 102%(-2%) |  |  |

Table 3. RD result and complexity increase of Low Delay

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -0.4 | -0.7 | -0.4 | -0.3 | -0.5 |
| Class C | -0.4 | -0.7 | -0.6 | -0.2 | -0.4 | -0.3 |
| Class D | -0.2 | -1.0 | -0.4 | -0.2 | -0.1 | -0.1 |
| Class E | 0.0 | -0.7 | -0.4 | -0.1 | -0.7 | -0.7 |
| All | -0.2 | -0.7 | -0.6 | -0.2 | -0.3 | -0.4 |
| Enc Time[%] | 101%(0%) |  |  | 101%(-1%) |  |  |
| Dec Time[%] | 100%(-1%) |  |  | 101%(-2%) |  |  |

The value in parenthesis is the complexity difference between the result of proponent and cross-checker.
Comparing with the excel sheet provided by Qualcomm, BD rate is exactly same and the difference of complexity increase is marginal within 4%.

# Conclusion

We reported the cross-check results of mode dependent intra residual. According to the experimental results, the software provided by Qualcomm produces the same RD performance with same complexity increase as stated in the proponent’s result. So we concluded JCTVC-D392 is well verified.

# Reference

- JCTVC-D392, “Mode dependent intra residual coding,” 4th JCT-VC Meeting, Daegu, Jan. 2011.