| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D408 |
| --- | --- |

| Title: | Crosscheck of Samsung and FastVDO's proposal JCTVC-D365 on simplified quantization/dequantization matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xin Zhao, Li Zhang, Siwei Ma, Wen Gao Institute of Digital Media, Peking University Beijing, China | Tel: Email: | +86-010-62753424 xzhao@jdl.ac.cn zhanglili@jdl.ac.cn swma@jdl.ac.cn wgao@jdl.ac.cn |
| Source: | Peking University |  |  |

_____________________________

# Abstract

In this document, the crosscheck results of Samsung and FastVDO's proposal JCTVC-D365 on simplified quantization/dequantization matrices are reported.

# Introduction

Samsung and FastVDO provided their implementation on the proposed simplified quantization and dequantization scaling matrices. In their implementation, the original scaling matrices for 16×16 and 32×32 transforms in TMuC 0.9 are replaced by simple matrices with equal elements. Simulation results are obtained under test conditions defined in [1] including Intra High-Efficiency, Intra Low-Complexity, Random Access High-Efficiency, and Random Access Low-Complexity, Low Delay High-Efficiency and Low Delay Low Complexity.

# Cross-verification results

The BD-Rate [2] results provided by Samsung matched with our results for all test conditions and sequences. Detailed results are shown in Table 1, 2 and 3.

Table 1 RD results for Intra configurations

| Sequence Class | Intra High Efficiency |  |  | Intra Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD_YRate | BD_URate | BD_VRate | BD_YRate | BD_URate | BD_VRate |
| Class A avg | 0.11 | 0.12 | 0.09 | 0.12 | 0.09 | 0.10 |
| Class B avg | 0.16 | 0.02 | 0.08 | 0.12 | 0.01 | 0.01 |
| Class C avg | 0.03 | 0.07 | 0.03 | 0.01 | 0.02 | 0.05 |
| Class D avg | 0.02 | 0.06 | 0.02 | 0.00 | 0.01 | 0.01 |
| Class E avg | 0.10 | -0.06 | -0.21 | 0.06 | 0.06 | 0.05 |
| All average | 0.09 | 0.04 | 0.01 | 0.06 | 0.03 | 0.03 |

Table 2 RD results for Random Access configurations

| Sequence Class | Random Access High Efficiency |  |  | Random Access Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD_YRate | BD_URate | BD_VRate | BD_YRate | BD_URate | BD_VRate |
| Class A avg | 0.00 | 0.03 | 0.39 | 0.03 | 0.13 | 0.13 |
| Class B avg | 0.04 | 0.00 | -0.14 | 0.03 | -0.03 | -0.03 |
| Class C avg | 0.00 | 0.01 | 0.00 | 0.01 | -0.08 | -0.08 |
| Class D avg | -0.01 | 0.21 | 0.31 | 0.02 | 0.08 | -0.03 |
| Class E avg |  |  |  |  |  |  |
| All average | 0.01 | 0.06 | 0.09 | 0.02 | 0.01 | -0.02 |

Table 3 RD results for Low Delay configurations

| Sequence name | Low Delay High Efficiency |  |  | Low Delay Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD_YRate | BD_URate | BD_VRate | BD_YRate | BD_URate | BD_VRate |
| Class A avg |  |  |  |  |  |  |
| Class B avg | -0.01 | 0.02 | -0.15 | -0.06 | -0.06 | -0.14 |
| Class C avg | -0.05 | -0.04 | 0.03 | 0.01 | -0.05 | 0.05 |
| Class D avg | 0.02 | -0.28 | 0.15 | -0.02 | 0.28 | 0.01 |
| Class E avg | -0.09 | 0.09 | -0.61 | -0.17 | -0.42 | -0.08 |
| All average | -0.03 | -0.06 | -0.12 | -0.05 | -0.04 | -0.04 |

The encoding and decoding running times are not shown because the test results are obtained on different test environments.

# References

[1]	Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, CN, 7-15 October, 2010.

[2] Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.