| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D411 |
| --- | --- |

| Title: | CE9: Test results on subtest 3.1.s and 3.1.t |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Tammy Lee | Email: | ilkoo.kim@samsung.com Tammy.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution contains results of subtest 3.1.s and 3.1.t for CE9 on motion vector coding. Attached spreadsheet contains detailed data of the results.

# Introduction

In this document, subtest 3.1.s and 3.1.t of CE9 defined as in CE9 description [1] are performed.

Table 1. Description of test 3.1.s and 3.1.t for CE 2

|  | Skip | Direct | Merge | Software / Configuration | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- | --- | --- |
| 3.1.s | AMVP based | 0 | Modified PU based 2 | HHI’s version of HM1.0 modified by DOCOMO (available Nov. 9th) | NTT DOCOMO | Samsung |
| 3.1.t | AMVP based | 0 | Modified PU based 3 | HHI’s version of HM1.0 modified by DOCOMO (available Nov. 9th) | NTT DOCOMO | Samsung |

Table 2 and table 3 describe the macro setting of software to test 3.1.s and 3.1.t configurations.

Table 2. Description of macro to configure AMVP-based skip and disable direct mode

| skip | direct | (0,0) * | left | above | median | corner | collocated | macro in HM1.0 modified by HHI and DOCOMO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AMVP based | 0 | 1 | 1 | 1 | 1 | 1 | 1 | SAMSUNG_MRG_SKIP_DIRECT 1 HHI_MRG_ONLY_ZERO_CORNER_COL_FOR_SKIP 0 DOCOMO_ DISABLE _DIRECT 1 |

Table 3. Description of macro to configure modified PU-based merging 2 and 3

| merge | left | above | collocated | corner** | macro in HM1.0 modified by HHI |
| --- | --- | --- | --- | --- | --- |
| Mod. PU/(CU) based 2 | 1 | 1 | 1 | 1 | #define HHI_MRG 1 #define HHI_MRG_PU 1(0) #define HHI_MRG_CU 0(1) #define HHI_MRG_COLLOCATED 1 #define HHI_MRG_CORNER 1 |
| Mod. PU/(CU) based 3 | 1 | 1 | 1 | 1 | #define HHI_MRG 1 #define HHI_MRG_PU 1(0) #define HHI_MRG_CU 0(1) #define HHI_MRG_CBF_2Nx2N 0 #define HHI_MRG_COLLOCATED 1 #define HHI_MRG_CORNER 1 |

# Test Results

Table 4 shows the summarized results for subtest 3.1.s. When compared with the anchor in JCTVC-C500, this configuration achieves average -0.9% BD-rate reduction. The average encoding and decoding times are 86% and 102%, respectively.

Table 4. Summarized results of 3.1.s for 4 configurations

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -0.3 | -0.2 | -1.5 | -1.1 | -0.8 |
| Class B | -0.6 | 0.0 | -0.1 | -1.1 | -0.6 | -0.4 |
| Class C | -0.7 | -0.4 | -0.4 | -1.3 | -0.7 | -0.8 |
| Class D | -0.9 | -0.4 | -0.4 | -1.4 | -1.1 | -0.9 |
| Class E |  |  |  |  |  |  |
| All | -0.8 | -0.3 | -0.3 | -1.3 | -0.8 | -0.7 |
| Enc Time[%] | 80% |  |  | 91% |  |  |
| Dec Time[%] | 101% |  |  | 106% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | 0.2 | 0.2 | -0.1 | -0.6 | -0.9 |
| Class C | -0.8 | -0.4 | -0.5 | -0.7 | -1.1 | -1.1 |
| Class D | -0.9 | -0.9 | -0.5 | -0.8 | -1.3 | -1.1 |
| Class E | -1.6 | -1.4 | -1.5 | -1.2 | -1.5 | -1.7 |
| All | -0.8 | -0.5 | -0.5 | -0.6 | -1.1 | -1.1 |
| Enc Time[%] | 83% |  |  | 91% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |

Table 5 shows the summarized results for subtest 3.1.t. When compared with the anchor in JCTVC-C500, this configuration achieves average -1.2% BD-rate reduction. The average encoding and decoding times are 87% and 104%, respectively.

Table 5. Summarized results of 3.1.t for 4 configurations

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -0.3 | 0.1 | -1.9 | -1.4 | -1.2 |
| Class B | -0.7 | -0.1 | 0.0 | -1.3 | -0.8 | -0.6 |
| Class C | -0.8 | -0.4 | -0.5 | -1.7 | -1.0 | -1.1 |
| Class D | -1.0 | -0.7 | -0.5 | -1.9 | -1.4 | -1.2 |
| Class E |  |  |  |  |  |  |
| All | -0.8 | -0.4 | -0.3 | -1.7 | -1.1 | -1.0 |
| Enc Time[%] | 80% |  |  | 93% |  |  |
| Dec Time[%] | 102% |  |  | 109% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | 0.1 | 0.1 | -0.9 | -1.2 | -1.3 |
| Class C | -0.7 | -0.5 | -0.4 | -1.5 | -1.7 | -1.6 |
| Class D | -0.8 | -1.3 | -0.8 | -1.6 | -2.0 | -1.6 |
| Class E | -1.5 | -1.7 | -1.5 | -2.2 | -2.2 | -2.3 |
| All | -0.8 | -0.7 | -0.6 | -1.5 | -1.7 | -1.6 |
| Enc Time[%] | 85% |  |  | 93% |  |  |
| Dec Time[%] | 101% |  |  | 106% |  |  |

# Conclusions

The test 3.1.s shows -0.9% BD-rate saving with 86%/102% encoding/decoding time, respectively. The test 3.1.t shows -1.2% BD-rate saving and 87% and 104% encoding/decoding time, respectively.

# References

- J. Jung and B. Bros, Core Experiment 9: Motion Vector Coding, in 3rd JCT-VC meeting , no. JCTVC-C509, Oct. 2010.

- J. Jung and B. Bros, CE9: summary report for CE9 on motion vector coding, in 4th JCT-VC meeting , no. JCTVC-D149, Jan. 2011.