| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D412 WG11 Number: m19208 |
| --- | --- |

| Title: | CE1: Cross-check report on Bi-directional optical flow of Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Eunyong Son, Jiwook Jung, Sehoon Yea Convergence Lab., LG Electronics Inc. 221, Yangjae-dong, Seocho-gu, Seoul, Korea | Tel: Email: | eunyong.son@lge.com jiwook.jung@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

The purpose of this document is to report the result of cross-check on Bi-directional optical flow by Samsung Electronics[1]. The verification task has been done successfully and the cross-checked RD-rate results match those provided by Samsung Electronics.

- Verification

Simulation results have been generated by performing Bi-directional optical flow for improving motion compensation. We have used TMuC version 0.9 software, encoding has been done by using Linux cluster(comprised of five Linux machines) and decoding process has been done in two different environments. Decoding environment 1 is a linux machine with Intel Xeon processor, and decoding environment 2 is a windows XP laptop computer. In each decoding process, we simulated output generation option both enabled and disabled. Output generation is enabled with “-o” option when we type “TAppDecoder” command. More details on simulation conditions are shown in Table 1.

|  | Encoding environment (x5) | Decoding environment 1 | Decoding environment 2 |
| --- | --- | --- | --- |
| CPU | Intel Core i7 960 3.20GHz | Intel Xeon X5550 2.67GHz | Intel Core i5 M520 2.4GHz |
| Memory | 12GB | 16GB | 4GB |
| OS | Cent OS release 5.4 | Cent OS release 5.5 | Windows XP |
| Compiler | gcc 4.1.2 64bit compiler | gcc 4.1.2 64bit compiler | Visual C++ 2008 32bit compiler |

Table 1. Simulation Environment

- Results

We evaluated BD-rate as well as encoding/decoding time for each sequence and the summary of the results are shown in Table 2 ~ Table 4. And all simulation results are shown in the attached excel file. As specified in [1], the “Random access high efficiency” and the “Random access low complexity” configurations are used for this test.

|  | Random access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Samsung result |  |  | Cross check result |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.7 | -0.8 | -0.7 | -1.7 | -0.8 | -0.7 |
| Class B | -1.6 | -0.7 | -0.8 | -1.6 | -0.7 | -0.8 |
| Class C | -1.6 | -1.0 | -1.0 | -1.6 | -1.0 | -1.0 |
| Class D | -2.8 | -1.5 | -1.4 | -2.8 | -1.5 | -1.4 |
| Class E |  |  |  |  |  |  |
| All | -1.9 | -1.0 | -1.0 | -1.9 | -1.0 | -1.0 |

Table 2. Random access configuration BD-rate result

|  | Random access low complexity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Samsung result |  |  | Cross check result |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.1 | -1.0 | -1.0 | -2.1 | -1.0 | -1.0 |
| Class B | -2.7 | -0.8 | -0.7 | -2.7 | -0.8 | -0.7 |
| Class C | -4.5 | -1.8 | -1.7 | -4.5 | -1.8 | -1.7 |
| Class D | -9.1 | -4.0 | -3.8 | -9.1 | -4.0 | -3.8 |
| Class E |  |  |  |  |  |  |
| All | -4.8 | -1.9 | -1.8 | -4.8 | -1.9 | -1.8 |

Table 3. Random access low complexity configuration BD-rate result

|  | Random access |  |  | Random access low complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Samsung result | Cross check result |  | Samsung result | Cross check result |  |
| Encoding time (%) | 112% | 139% |  | 111% | 121% |  |
| Decoding time (%) | 199% | Env1 (with –o) | 190% | 248% | Env1 (with –o) | 278% |
|  |  | Env1 (without –o) | 195% |  | Env1 (without –o) | 308% |
|  |  | Env2 (with –o) | 192% |  | Env2 (with –o) | 265% |
|  |  | Env2 (without –o) | 199% |  | Env2 (without –o) | 274% |

Table 4. Encoding / Decoding Time

- Conclusion

According to the Table 2 & Table 3, BD-rate performance of Y, U, V results in both configurations was identical to that of Samsung. Notably it took 2~3 times longer than TMuC 0.9 to decode if proposed scheme was employed. Especially in Random Access low complexity mode, the decoding speed of bi-directional optical flow was much slower. Finally, we could observe that the decoding speed was highly dependent on the specific simulation environment.

- References

| [1] | Elena Alshina, Alexander Alshin, Woo-Jin Han “CE1: Samsung’s test for bi-directional optical flow” Doc JCTVC-D329, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, Jan 2011 |
| --- | --- |