| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D413r2 WG11 Number: m19224 |
| --- | --- |

| Title: | CE9: Cross-Check Result of 3.1s and 3.1t |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yoshitaka Morigami, Kazushi Sato GCO W12F 1-11-1 Osaki Shinagawa Tokyo 141-0032 Japan | Tel: Email: | +81-3-5435-3308 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document provides cross-check result of the experiment 3.1s and 3.1t in CE9.

- Description

Document [3] is Samsung’s proposal on coding efficiency of AMVP Skip and HHI_MRG_CBF_2Nx2N of HHI's PU merging syntax.

This document reports the cross-check result of [3] by Sony.

# Simulation

## Simulation Condition

Basic setting is same as the one described in [2] with RandomAccess, RandomAccess_Loco, LowDelay, and LowDelay_Loco conditions.

As anchor HM1.0 is used, and Samsung provided software for experiment 3.1s and 3.1t to Sony.

## Simulation Result

Summary of the result of the experiment 3.1s is shown in Table 2.2.1 and 2.2.2. Complete result is shown in “JCTVC-D413_s.xls”. Summary of the result of the experiment 3.1s is shown in Table 2.2.3 and 2.2.4. Complete result is shown in “JCTVC-D413_t.xls”.

As can be seen up to 2% gain can be obtained with the proposed method.

Table 2.2.1

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -0.3 | -0.2 | -1.5 | -1.1 | -0.8 |
| Class B | -0.6 | 0.0 | -0.1 | -1.1 | -0.6 | -0.4 |
| Class C | -0.7 | -0.4 | -0.4 | -1.3 | -0.7 | -0.8 |
| Class D | -0.9 | -0.4 | -0.4 | -1.4 | -1.1 | -0.9 |
| Class E |  |  |  |  |  |  |
| All | -0.8 | -0.3 | -0.3 | -1.3 | -0.8 | -0.7 |
| Enc Time[%] | 80% |  |  | 89% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |

Table 2.2.2

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | 0.2 | 0.2 | -0.1 | -0.6 | -0.9 |
| Class C | -0.8 | -0.4 | -0.5 | -0.7 | -1.1 | -1.1 |
| Class D | -0.9 | -0.9 | -0.5 | -0.8 | -1.3 | -1.1 |
| Class E | -1.6 | -1.4 | -1.5 | -1.2 | -1.5 | -1.7 |
| All | -0.8 | -0.5 | -0.5 | -0.6 | -1.1 | -1.1 |
| Enc Time[%] | 85% |  |  | 92% |  |  |
| Dec Time[%] |  |  |  | 104% |  |  |

Table 2.2.3

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -0.3 | 0.1 | -1.9 | -1.4 | -1.2 |
| Class B | -0.7 | -0.1 | 0.0 | -1.3 | -0.8 | -0.6 |
| Class C | -0.8 | -0.4 | -0.5 | -1.7 | -1.0 | -1.1 |
| Class D | -1.0 | -0.7 | -0.5 | -1.9 | -1.4 | -1.2 |
| Class E |  |  |  |  |  |  |
| All | -0.8 | -0.4 | -0.3 | -1.7 | -1.1 | -1.0 |
| Enc Time[%] | 80% |  |  | 89% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |

Table 2.2.4

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | 0.1 | 0.1 | -0.9 | -1.2 | -1.3 |
| Class C | -0.7 | -0.5 | -0.4 | -1.5 | -1.7 | -1.6 |
| Class D | -0.8 | -1.3 | -0.8 | -1.6 | -2.0 | -1.6 |
| Class E | -1.5 | -1.7 | -1.5 | -2.2 | -2.2 | -2.3 |
| All | -0.8 | -0.7 | -0.6 | -1.5 | -1.7 | -1.6 |
| Enc Time[%] | 85% |  |  | 93% |  |  |
| Dec Time[%] | 101% |  |  | 105% |  |  |

# Conclusion

Experimental result of 3.1s and 3.1t in CE13 has been verified. Up to 2% gain can be obtained with the proposed method. Higher gain is obtained with low-complexity conditions.

# References

- “WD1: Working Draft 1 of High-Efficiency Video Coding”, JCTVC-C403, October 2010

- “Common test conditions and software reference configurations”, JCTVC-C500, October 2010

- “CE9: Test results on subtest 3.1.s and 3.1.t”, JCTVC-C411, January 2011