| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D414 WG11 Number: m19237 |
| --- | --- |

| Title: | CE3 : Cross-check report of Toshiba’s Non-uniform tap length filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jiwook Jung, Eunyong Son, Sehoon Yea Convergence Lab., LG Electronics Inc. 221, Yangjae-dong, Seocho-gu, Seoul, Korea | Tel: Email: | jiwook.jung@lge.com eunyong.son@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

This contribution provides the result of a cross-check on Non-uniform Tap Length Filter (NTLF) by Toshiba. [1] The verification task was performed successfully and the cross-checked RD-rate results match those provided by Toshiba.

- Simulation Setup

Simulation results have been generated by performing Luma interpolation using Non-uniform Tap Length Filter (NTLF) for improving motion estimation. All Simulations have been done by using a Linux cluster (five nodes, OS : Linux CentOs 5.4 / 64bits). In each decoding process, we simulated with the output generation option disabled (i.e., without “–o” option). More details on simulation conditions are shown in Table 1.

|  | Environment |
| --- | --- |
| Computers | 5 nodes Linux cluster |
| CPU | Intel Core i7 960 3.20GHz |
| Memory | 12GB |
| OS | Cent OS 5.4 / 64bits |
| Compiler | gcc 4.1.2 |

Table 1. Simulation Environment

- Results

We evaluated BD-rate as well as encoding/decoding time for each sequence and the summary of the results are shown in Table 2 . And all simulation results are shown in the attached excel file. We tested “Randomaccess HE”, “Randomaccess LoCo”, “Lowdelay HE” and “Lowdelay LoCo” modes using test conditions in JCTVC-C503r5 [2].

Table 2	Average results of luminance BD-rate (%)

|  |  | Low complexity |  | High efficiency |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 8/4-tap |  | 12/8-tap (1) |  | 12/8-tap (2) |
| Random access |  | Toshiba | LG | Toshiba | LG |  |
|  | Class A | 0.1 | 0.1 | -0.1 | -0.1 | N/A |
|  | Class B | -1.1 | -1.1 | -0.2 | -0.2 | N/A |
|  | Class C | -4.0 | -4.0 | 0.1 | 0.1 | N/A |
|  | Class D | -8.8 | -8.8 | 0.2 | 0.2 | N/A |
|  | Class E | N/A | N/A | N/A | N/A | N/A |
|  | All | -3.7 | -3.7 | 0.0 | 0.0 | N/A |
|  | Enc Time[%] | 108% | 105% | 100% | 99% | N/A |
|  | Dec Time[%] | 112% | 102% | 94% | 99% | N/A |
| Low delay |  | Toshiba | LG | Toshiba | LG |  |
|  | Class A | N/A | N/A | N/A | N/A | N/A |
|  | Class B | -2.8 | -2.8 | -0.6 | -0.6 | N/A |
|  | Class C | -4.5 | -4.5 | -0.3 | -0.3 | N/A |
|  | Class D | -8.6 | -8.6 | -0.2 | -0.2 | N/A |
|  | Class E | -2.8 | -2.8 | -0.8 | -0.8 | N/A |
|  | All | -4.6 | -4.6 | -0.5 | -0.5 | N/A |
|  | Enc Time[%] | 107% | 106% | 100% | 100% | N/A |
|  | Dec Time[%] | 109% | 109% | 94% | 96% | N/A |

As shown in Table 2, the BD-rate performance of the Y, U, V coding results in all the configurations matches that of Toshiba’s results. However, the encoding/decoding time results are slightly different from Toshiba’s results.

- Conclusion

This contribution provides the result of a cross-check on Non-uniform Tap Length Filter (NTLF) by Toshiba [1]. The cross-check was performed successfully and the cross-checked BD-rate results match those provided by Toshiba. The encoding/decoding time results also showed reasonably matches considering the use of different simulation platforms. .

- References

| [1] | T. Chujoh, K. Kanou and T. Yamakage,“TE3 subtest 4: High Accuracy Interpolation Filter (HAIF),” JCTVC-C078, 3rd. Meeting, Guangzhou, CN, 7-15 Oct, 2010. |
| --- | --- |
| [2] | T. Chujoh, “CE3: Interpolation for MC (Luma),” Joint Collaborative Team on Video Coding, JCTVC-C503r5, Guangzhou, Oct. 2010. |