| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: WG11 Number: |
| --- | --- |

| Title: | CE4: Cross-check report of Nokia’s Chroma interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jiwook Jung, Eunyong Son, Sehoon Yea Convergence Lab., LG Electronics Inc. 221, Yangjae-dong, Seocho-gu, Seoul, Korea | Tel: Email: | jiwook.jung@lge.com eunyong.son@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

This contribution provides a cross-check result of Nokia’s Chroma interpolation filter [1]. The verification task was done successfully and the cross-checked BD-rate results match those provided by Nokia.

- Simulation Setup

Simulation results have been generated by employing Nokia’s chroma interpolation filter. We have used TMuC version 0.9 software as anchor and source codes provided by Nokia. All Simulations have been done on a Linux cluster (five nodes, OS : Linux CentOs 5.4 / 64bits). More details on simulation conditions are shown in Table 1.

|  | Environment |
| --- | --- |
| Computers | 5 nodes Linux cluster |
| CPU | Intel Core i7 960 3.20GHz |
| Memory | 12GB |
| OS | Cent OS 5.4 / 64bits |
| Compiler | gcc 4.1.2 |

Table 1. Simulation Environment

- Results

We evaluated the BD-rate as well as encoding/decoding time for each sequence and the summary of the results are shown in Table 2 and Table 3. Full simulation results are provided in the attached excel file. We tested “Randomaccess HE”, “Randomaccess LoCo”, “Lowdelay HE” and “Lowdelay LoCo” modes using test conditions in JCTVC-C504 [2].

|  | Random access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  | YUV |  |
|  | Toshiba | LG | Toshiba | LG | Toshiba | LG | Toshiba | LG |
| Class A | 0.0 | 0.0 | -0.6 | -0.6 | -0.7 | -0.7 | -0.2 | -0.2 |
| Class B | 0.0 | 0.0 | -1.2 | -1.2 | -1.8 | -1.8 | -0.5 | -0.5 |
| Class C | 0.0 | 0.0 | -1.2 | -1.2 | -1.1 | -1.1 | -0.4 | -0.4 |
| Class D | 0.0 | 0.0 | -1.0 | -1.0 | -1.2 | -1.2 | -0.4 | -0.4 |
| Class E |  |  |  |  |  |  |  |  |
| All | 0.0 | 0.0 | -1.1 | -1.1 | -1.3 | -1.3 | -0.4 | -0.7 |

|  | Random access loco |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  | YUV |  |
|  | Toshiba | LG | Toshiba | LG | Toshiba | LG | Toshiba | LG |
| Class A | 0.0 | 0.0 | -0.5 | -0.5 | -0.6 | -0.6 | -0.2 | -0.2 |
| Class B | 0.0 | 0.0 | -1.6 | -1.6 | -1.8 | -1.8 | -0.5 | -0.5 |
| Class C | 0.0 | 0.0 | -2.7 | -2.7 | -2.5 | -2.5 | -0.9 | -0.9 |
| Class D | 0.0 | 0.0 | -2.9 | -2.9 | -3.6 | -3.6 | -0.9 | -0.9 |
| Class E |  |  |  |  |  |  |  |  |
| All | 0.0 | 0.0 | -2.1 | -2.1 | -2.3 | -2.3 | -0.7 | -0.7 |

Table 2. Random access configuration BD-rate result

|  | Low delay |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  | YUV |  |
|  | Toshiba | LG | Toshiba | LG | Toshiba | LG | Toshiba | LG |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1 | -0.1 | -0.8 | -0.8 | -1.3 | -1.3 | -0.4 | -0.4 |
| Class C | -0.1 | -0.1 | -1.0 | -1.0 | -1.1 | -1.1 | -0.5 | -0.5 |
| Class D | -0.1 | -0.1 | -1.3 | -1.3 | -0.6 | -0.6 | -0.4 | -0.4 |
| Class E | -0.1 | -0.1 | -1.1 | -1.1 | -1.2 | -1.2 | -0.4 | -0.4 |
| All | -0.1 | -0.1 | -1.0 | -1.0 | -1.1 | -1.1 | -0.4 | -0.4 |

|  | Low delay Loco |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  | YUV |  |
|  | Toshiba | LG | Toshiba | LG | Toshiba | LG | Toshiba | LG |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.2 | -0.2 | -2.0 | -2.0 | -3.0 | -3.0 | -0.8 | -0.8 |
| Class C | -0.3 | -0.3 | -2.2 | -2.2 | -2.0 | -2.0 | -1.0 | -1.0 |
| Class D | -0.4 | -0.4 | -2.3 | -2.3 | -2.8 | -2.8 | -1.0 | -1.0 |
| Class E | -0.1 | -0.1 | -0.3 | -0.3 | -0.8 | -0.8 | -0.2 | -0.2 |
| All | -0.3 | -0.3 | -1.8 | -1.8 | -2.3 | -2.3 | -0.8 | -0.8 |

Table 3. Low delay configuration BD-rate result

As shown in the tables, the BD-rate performance of the Y, U, V coding results in all the configurations matches that of Nokia’s results.

- Conclusion

According to the Table 2 & Table 3, the BD-rate performance of Y, U, V results is identical to Nokia’s results.

- References

| [1] | “Video coding technology proposal by Tandberg”, Nokia, Ercisson, Joint Collaborative Team on Video Coding, JCTVC-A119 |
| --- | --- |
| [2] | E. Alshina, “CE4: Interpolation for MC (Chroma),” Joint Collaborative Team on Video Coding, JCTVC-C504, Guangzhou, Oct. 2010. |