| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D416 |
| --- | --- |

| Title: | LCEC RDOQ speedup |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jianle Chen | Email: | v.seregin@samsung.com jianle.chen@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes complexity reduction for LCEC RDOQ with minor software modification in a similar manner that already existed in RDOQ for CABAC. Proposed method provides 0.0%, 0.1% and 0.0% performance loss and with the 81%, 93% and 93% running encoder time in low complexity intra-only, random access, and low-delay configurations, respectively.

# Introduction

In current test model (HM), Rate Distortion Optimized Quantization (RDOQ) technique is adopted in both high efficiency (HE) and low complexity (LC) configurations.

This method improves quantization by minimising rate-distortion function and adjusts the rounding offset [1, 2].

In HE case RDOQ implementation is done considering up to three quantized candidates for every transform coefficient level: zero, Lfloor and Lceil.

To speed up the algorithm several simplifications are made including that if coefficient (blue point) is closer (measured as absolute difference between the coefficient and its reconstructed value) to Lfloor than to Lceil only value Lfloor is considered.

In LCEC RDOQ implementation however, for every level, number of candidates is up to five: zero, level-2, level-1, level, level+1 without speeding up by considering level values itself.

As can be seen, LCEC RDOQ can be optimised in similar to HE case way, and unnecessary comparisons for transform coefficients can be removed. This optimisation was performed by removing level-2 and level-1 candidate, and if coefficient is closer to Lfloor than to Lceil only value Lfloor is considered. So, number of candidates reduces to three at most.

# Experimental results of optimised LCEC RDOQ

LCEC RDOQ was optimised as discussed above and simulation results with comparison to unmodified TMuC 0.9 anchor is the following.

Table 1. Average BD-rate saving (against TMuC anchor)

|  | Optimised RDOQ |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Enc | Dec |
| LoCo Intra | 0.0 | 0.0 | 0.0 | 81% | 100% |
| LoCo Random access | 0.1 | 0.1 | 0.1 | 93% | 103% |
| LoCo Low delay | 0.0 | 0.1 | 0.0 | 92% | 96% |

# Conclusion

RDOQ design in LC case was harmonised with HE RDOQ, bringing complexity reduction up to 19% of encoder running time with negligible performance loss. We suggest including this optimised RDOQ into HM.

# References

- En-hui Yang and Xiang Yu. “Rate Distortion Optimization of H.264 with Main  profile Compatibility”, ISIT, pp. 282-286, July 2006.

- Marta Karczewicz, Yan Ye and Insuk Chong. “Rate Distortion Optimized Quantization”, VCEG Meeting Document VCEG-AH21, January, 2008.

# Patent rights declaration(s)

Samsung Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).