| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D417 WG11 Number: m19364 |
| --- | --- |

| Title: | Cross-check report of Sony’s AMVP modification (JCTVC-D303) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto Yukinobu Yasugi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | Tel: +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp yasugi.yukinobu@sharp.co.jp, |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides cross-check result of JCTVC-D303 [1], in which AMVP modification to avoid using median prediction is proposed. The experimental results completely match with those provided by the proponent in terms of bitrate and PSNR.

# Simulation condition

The experiment was carried out based on the configuration files in TMuC 0.9 except that MRG is set to 0. Random Access and Low Delay conditions were tested in both High Efficiency and Low Complexity configurations.

Reference:	TMuC 0.9, MRG=0

Compared:	Software provided by Sony, MRG=0

The computing platform used for the experiment is summarized in Table 1. In the experiments, encoding was done using the grid system composed of variety of PCs, but it was configured so that the same-performance PC is used to encode both anchor and the proposed method for each test point. Decoding was done using the same machine for both anchor and the proposed method.

Table 1: Computing platforms

|  | Decoding | Encoding |
| --- | --- | --- |
| OS | Linux (64bit) | Linux (64bit) |
| CPU | Intel Core2 Quad Q9550 2.83GHz Intel Xeon X5667 3.07GHz Intel Core2 Quad 2.66GHz | Intel Core2 Quad Q9550 2.83GHz Intel Xeon X5667 3.07GHz Intel Core2 Quad 2.66GHz |
| Compiler | gcc 4.4.3 | gcc 4.4.3 |

# Simulation result

The experimental results are summarized in Table 2. Detailed results are included in the attachment (JCTVC-D417xls).

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.3 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| Class C | 0.0 | -0.1 | -0.1 | -0.1 | -0.1 | 0.0 |
| Class D | -0.1 | 0.1 | -0.1 | -0.1 | -0.1 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.0 | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.3 | 0.2 | 0.1 | 0.2 | 0.3 |
| Class C | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.2 | 0.0 | 0.2 | 0.1 |
| Class E | -0.2 | -1.1 | 0.0 | -0.2 | -0.2 | -0.3 |
| All | 0.0 | -0.1 | 0.1 | 0.0 | 0.1 | 0.1 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 2: Experimental results

# Verification

The experimental results in above section were compared with the results in JCTVC-D303, and they completely match in terms of bitrate and PSNR.

# Reference

- Kazushi Sato, “Consideration on AMVP”, JCTVC-D303, 4th JCT-VC Meeting, Daegu, January 2011