| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January, 2011 | Document: JCTVC-D421 WG11 Number: m19400 |
| --- | --- |

| Title: | Extension of uni-prediction simplification in B slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshinori Suzuki TK Tan 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan Wei Jung Chien Ying Chen Peisong Chen Muhammed Coban 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | +81 46 840 3517 suzukiyos@rd.nttdocomo.co.jp +65 9799 3861 tktan.p@rd.nttdocomo.co.jp +1-858-845-1795 wchien@qualcomm.com cheny@qualcomm.com peisongc@qualcomm.com mcoban@qualcomm.com |
| Source: | NTT DOCOMO, Inc., Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution the simplification for the entropy coding of the prediction mode in the generalized B slices is further extended to cover more cases than just in the situation where list 0 and list 1 are identical.  The same benefit can be achieved for other cases where list 0 and list 1 share common entries but are not identical.

The crosscheck results can be found in JCTVC-D442.

- Introduction

In the generalized B slices, an inter partition has three possible prediction mode, indicated by the syntax element inter_pred_idc.  These are the bi-prediction mode with reference pictures from list 0 and list 1 (BiPred), uni-prediction mode with one reference picture from list 0 (Pred_L0) and uni-prediction mode with one reference picture from list 1 (Pred_L1).

The lists used in the uni-pred prediction mode is coupled with (or the same as) the lists used in the BiPred mode to facilitate the motion vector prediction process.  However, there is no specific reason why there should be two types of uni-prediction modes especially in the entropy coding of the prediction modes as illustrated by the JCTVC-C278 / JCTVC-C285 proposal [1][2] that was adopted into the HM, where the Pred_L1 prediction mode is disallowed under the special condition when list 0 and list 1 are identical.

In the same spirit, it is proposed in this contribution that the entropy coding of the prediction mode can be further simplified for other cases where the two lists are not identical but have shared entries.

This simplification can be achieved in the syntax parsing process where a combined list, LC is created for the purpose of entropy coding.  This list contains the mapping of the entries in list 0 and list 1.  After the combined uni-prediction mode is decoded, it is mapped to the appropriate Pred_L0 and Pred_L1 prediction mode for the subsequent decoding process.

With this it is unnecessary to conditionally switch the number of prediction modes based on the contents of list 0 and list 1.  Hence inter_pred_idc can be reduced to indicate only two possibilities in all cases: namely bi-prediction modes with reference pictures from list 0 and list 1 (BiPred) as before and uni-prediction mode with one reference picture from combined list (Pred_LC).

- Definition of uni-prediction in B slices

The syntax element for inter_pred_idc is defined as follows.

inter_pred_idc[ i ] specifies whether uni-prediction or bi-prediction is used for the current partition with partition index i. When inter_pred_idc[i] is not present, it shall be inferred to be equal to Pred_L0 when slice_type is equal to P slice and BiPred when slice_type is equal to B slice.

## Initialization of the combined list from list 0 and list 1

The combined list is created by initializing it with the entries from list 0 and list 1.  This is done by interleaving the entries of list 0 and list 1 in ascending order of the indices, beginning with the smallest index of list 0.  Indices which point to the same reference pictures that are already included in the combined list are skipped. Figure 1 shows the flow chart for this process.  Figure 2 shows an example of the combined list for the case where the relative output order of the reference pictures and the current picture is shown in Figure 3.

Figure 1: Flow chart for creating the combined list (LC) from list 0 (L0) and list 1 (L1).

Figure 2: Example of the mapping between the combined list and list 0 and list 1.

Figure 3: Illustration of reference and current pictures in output picture order.

## Signaling of the combined list

Combined list is created from entries in list 0 and list 1. The entries which form the combined list may be signaled in the slice_header(). The following syntax describes the signaling process, ref_pic_list_combination(), that describes the combined list modification. In the slice header ref_pic_list_combination( ) operation follows the ref_pic_list_modification( ) operation.

Reference picture list combination syntax

| ref_pic_list_combination( ) { | C | Descriptor |
| --- | --- | --- |
| if( slice_type % 5 = = 1 ) { // b slice |  |  |
| ref_pic_list_combination_flag | 2 | u(1) |
| if( ref_pic_list_combination_flag ) { |  |  |
| num_com_ref_list_active_minus1 | 2 | ue(v) |
| ref_pic_list_modification_flag_lc | 2 | u(1) |
| if( ref_pic_list_modification_flag_lc ) |  |  |
| for ( i =0; i <= num_com_ref_list_active_minus1; i++ ) { |  |  |
| pic_from_list_0_flag | 2 | u(1) |
| ref_idx_list_curr | 2 | ue(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Reference picture list combination semantics

ref_pic_list_combination_flag equal to 1 indicates that the reference picture list 0 and the reference picture list 1 are combined to be an additional combined list used for the prediction units being uni-directional predicted. This flag equal to 0 indicates that the reference picture list 0 and reference picture list 1 are identical thus reference picture list 0 is used as the combined list. The combined reference picture list is set to be empty at the start of the loop defined in this table.

num_com_ref_list_active_minus1+1 specifies the number of reference pictures selected from reference picture list 0 or reference picture list 1 in the combined list.

ref_pic_list_modification_flag_lc equal to 1 specifies that the syntax element pic_from_list_0_flag and ref_idx_list_curr are present for specifying the mapping for the entries of the combined list to the entries of reference picture list 0 and reference picture list 1.

ref_pic_list_modification_flag_lc equal to 0 specifies that these syntax elements are not present. The combined reference picture list is initialized according to section 2.1 of this contribution (JCTVC-D421).

pic_from_list_0_flag indicates the current reference picture added into the combined list is from reference picture list 0 or reference picture list 1. When this flag is equal to 1, the picture is from the reference picture list 0, and the CurrRefPicList is reference picture list 0; when this flag is equal to 0, the picture is from the reference picture list1, and the CurrRefPicList is reference picture list 1;

ref_idx_list_curr indicates the reference index of the picture in the CurrRefPicList to be appended at the end of the combined list.

  - Mapping of Pred_LC to Pred_L0 and Pred_L1

In the encoder, the prediction modes Pred_L0, Pred_L1 and reference index are mapped to the combined prediction mode Pred_LC and combined reference index for entropy coding.  Similarly, in the decoder the decoded combined prediction mode Pred_LC and combined reference index are mapped to one of the two prediction modes Pred_L0 or Pred_L1, and reference index before applying the remaining decoding process.

In the encoder, the mapping is done by finding the reference index of the combined list that contains the same reference frame as the list 0 or list 1 used for the uni-prediction motion compensation.

In the decoder, the mapping is done by finding the reference index of the list 0 or list 1 which contains the same reference frame as the reference frame pointed to by the combined reference index of the combined list.  In the case where the reference frame can be found in list 0 and list 1, the list with the smaller index shall be selected.  In the case where both indices are the same, then list 0 shall be selected.

# Simulation Results

Simulations were conducted for the above proposal using the common conditions defined in [3].  Table 1and Table 2 below show the results for random access and low delay conditions.  As expected, there is no significant difference between the proposed method and the reference, because the proposed method is a superset of the current default HM method.

Table 1: Simulation result for default random access condition

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 |
| Class D | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2: Simulation result for default low delay condition

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | -0.6 | 0.5 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

## First alternate configuration

Two other encoding configurations are tested to illustrate the cases where the current default HM method does not apply.  The first case is when the random access configuration is extended to use 2 reference pictures each in list 0 and list 1 instead of 1 as in the current default.  This is accomplished by setting the following configuration parameters.

NumOfReferenceB_L0 : 2     # Number of reference frames for L0 for B-slices

NumOfReferenceB_L1 : 2     # Number of reference frames for L1 for B-slices

### Results of extending the reference list (first case)

Table 4 shows the results of the random access condition with two reference pictures in list 0 and list 1 compared to the default configuration with one reference pictures in list 0 and list 1.  BD-Rate savings of 2.2% and 2.6% were obtained for the cases of high efficiency and low complexity, respectively.

Table 3: Simulation result for random access condition with 2 reference pictures in list 0 and list 1

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.8 | -2.3 | -2.3 | -1.6 | -1.8 | -1.6 |
| Class B | -2.7 | -2.4 | -2.3 | -2.3 | -0.9 | -0.7 |
| Class C | -2.4 | -1.7 | -2.0 | -2.9 | -1.8 | -1.7 |
| Class D | -1.4 | -1.0 | -0.9 | -3.3 | -1.4 | -0.9 |
| Class E |  |  |  |  |  |  |
| All | -2.2 | -1.8 | -1.8 | -2.6 | -1.4 | -1.1 |
| Enc Time[%] | 150% |  |  | 179% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |

### Results of extending the reference list (first case) with the proposal

Table 4 shows the results of the random access condition with two reference pictures in list 0 and list 1 with the current proposal compared to the default configuration.  BD-Rate savings of 2.5% and 3.1% were obtained implying that a further BD-Rate savings of 0.3% and 0.5% were obtained when the current proposal is added, for the cases of high efficiency and low complexity, respectively.  A reduction in encoding times of approximately (150-137)/1.5 = 9% and (179 – 158)/1.79 = 12% are also observed for the above two cases, respectively.  Note that the low delay case was not tested since list 0 and list 1 would be identical.

Table 4: Simulation result for random access condition with 2 reference pictures in list 0 and list 1 and the proposal

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.0 | -2.7 | -2.7 | -2.1 | -2.1 | -2.1 |
| Class B | -3.1 | -3.0 | -3.0 | -2.9 | -1.6 | -1.2 |
| Class C | -2.5 | -2.0 | -2.2 | -3.2 | -2.2 | -2.0 |
| Class D | -1.8 | -1.4 | -1.2 | -3.8 | -1.9 | -1.4 |
| Class E |  |  |  |  |  |  |
| All | -2.5 | -2.3 | -2.3 | -3.1 | -1.9 | -1.6 |
| Enc Time[%] | 137% |  |  | 158% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |

## Second alternate configuration

The second case is when the random access and low delay configurations were extended to use 4 reference pictures in list 0 and 2 reference pictures in list 1.  This is accomplished by setting the following configuration parameters.

NumOfReferenceB_L0 : 4     # Number of reference frames for L0 for B-slices

NumOfReferenceB_L1 : 2     # Number of reference frames for L1 for B-slices

### Results of extending the reference list (second case)

Table 8 shows the results of the random access condition with 4 reference pictures in list 0 and 2 reference pictures in list 1.  BD-Rate savings of 2.7% and 3.1% were obtained for the cases of high efficiency and low complexity, respectively.

Table 5: Simulation result for random access condition with 4 reference pictures in list 0 and 2 reference pictures in list 1.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.1 | -2.8 | -2.6 | -1.4 | -2.0 | -1.8 |
| Class B | -3.6 | -3.0 | -2.9 | -3.0 | -1.3 | -1.0 |
| Class C | -3.0 | -2.3 | -2.5 | -3.4 | -1.9 | -1.9 |
| Class D | -1.7 | -1.2 | -1.0 | -3.7 | -1.5 | -1.3 |
| Class E |  |  |  |  |  |  |
| All | -2.7 | -2.3 | -2.2 | -3.1 | -1.6 | -1.4 |
| Enc Time[%] | 204% |  |  | 251% |  |  |
| Dec Time[%] | 101% |  |  | 97% |  |  |

Table 6 shows the results of the low delay condition with 4 reference pictures in list 0 and 2 reference pictures in list 1.  BD-Rate savings of 2.8% and 3.8% were obtained for the cases of high efficiency and low complexity, respectively.

Table 6: Simulation result for low delay condition with 4 reference pictures in list 0 and 2 reference pictures in list 1.

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -2.5 | -3.0 | -3.3 | -2.9 | -3.5 | -4.1 |
| Class C | -2.4 | -2.2 | -2.2 | -3.6 | -4.8 | -4.7 |
| Class D | -2.6 | -3.6 | -2.7 | -5.2 | -6.5 | -6.6 |
| Class E | -4.2 | -5.9 | -7.2 | -3.8 | -2.7 | -4.1 |
| All | -2.8 | -3.5 | -3.6 | -3.8 | -4.4 | -4.9 |
| Enc Time[%] | 163% |  |  | 183% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

### Results of extending the reference list (second case) with the proposal

Table 7 shows the results of the random access condition with 4 reference pictures in list 0 and 2 reference picture in list 1.  BD-Rate savings of 3.3% and 3.9% were obtained implying that a further BD-Rate savings of approximately 0.6% and 0.8% were obtained when the current proposal is added, for the cases of high efficiency and low complexity, respectively.  A reduction in encoding time of approximately (204-191)/2.04 = 6% and (251 – 235)/2.51 = 6% is also observed for the above two cases, respectively.

Table 7: Simulation result for random access condition with 4 reference pictures in list 0 and 2 reference pictures in list 1.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.6 | -3.5 | -3.4 | -2.3 | -2.7 | -2.5 |
| Class B | -4.2 | -3.6 | -3.6 | -3.7 | -2.1 | -1.8 |
| Class C | -3.3 | -2.8 | -2.9 | -4.0 | -2.6 | -2.4 |
| Class D | -2.3 | -1.6 | -1.5 | -4.6 | -2.1 | -1.9 |
| Class E |  |  |  |  |  |  |
| All | -3.3 | -2.8 | -2.8 | -3.9 | -2.3 | -2.1 |
| Enc Time[%] | 191% |  |  | 235% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |

Table 8 shows the results of the low delay condition with 4 reference pictures in list 0 and 2 reference pictures in list 1.  BD-Rate savings of 3.4% and 4.8% were obtained implying that a further BD-Rate savings of approximately 0.6% and 1.0% were obtained when the current proposal is added, for the cases of high efficiency and low complexity, respectively.  A reduction in encoding time of approximately (163-120)/1.63 = 26% and (183 – 129)/1.83 = 29.5% is also observed for the above two cases, respectively.

Table 8: Simulation result for low delay condition with 4 reference pictures in list 0 and 2 reference pictures in list 1.

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -2.8 | -3.1 | -3.4 | -3.6 | -3.9 | -4.3 |
| Class C | -3.0 | -2.8 | -2.8 | -4.7 | -5.5 | -5.6 |
| Class D | -3.5 | -4.1 | -3.4 | -6.4 | -7.1 | -7.5 |
| Class E | -4.8 | -7.4 | -8.4 | -5.0 | -3.4 | -4.8 |
| All | -3.4 | -4.1 | -4.2 | -4.8 | -5.0 | -5.5 |
| Enc Time[%] | 120% |  |  | 129% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 9 shows a summary of the approximate BD-rate gains and encoding times obtained for the above test cases when the corresponding reference for each of the coding conditions is used instead of the HM0.9 default reference.

Table 9: Overall summary of the improvements obtained when the current proposal is added where the corresponding reference is used instead of the default HM0.9 in each of the above altenate cases.

|  | 2 ref in L0, 2 ref in L1 |  | 4 ref in L0, 2 ref in L1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access | Random access LoCo | Random access | Random access LoCo | Low delay | Low delay LoCo |
| Average Y BD-rate | -0.3% | -0.5% | -0.6% | -0.8% | -0.6% | -1.0% |
| Enc Time[%] | 91% | 88% | 94% | 94% | 74% | 70% |

# Conclusions

The results show that the current proposal gives the same performance as the method currently in the HM when list 0 and list 1 are identical.  It further gives BD-rate improvements of between 0.3% to 1.0% with reduction in encoding time to 94% to 70% for cases where the entries of list 1 is the same as list 0 but in different order, where list 1 is a subset of list 0 or where the lists have overlapping entries.

This contribution shows that we can extend the simplifications to the coding of the uni-prediction mode in B slices in cases where list 0 and list 1 share common entries and obtain the same improvements as the simplifications adopted for the case where list 0 and list 1 are identical.

This proposal is a generalization of the current HM method and it is proposed that the simplification be extended with this proposal.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Bin Li et. al., “Redundancy reduction in B-frame coding at temporal level zero”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C278, 3rd Meeting: Guangzhou, China, 7-15 October, 2010.

- Wei-Jung Chien et. al., “Modified Uni-directional Inter Prediction in Generalized P and B Picture”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C278, 3rd Meeting: Guangzhou, China, 7-15 October, 2010.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, China, 7-15 October, 2010.