| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D423 |
| --- | --- |

| Title: | CE3: Verification results of Samsung’s 8 Tap DCTIF with 6 bits coefficients under Encoder Restriction by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Liwei Guo and In Suk Chong and Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: 1-858-658-5673 Email: liweig@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution shows verification results of JCTVC-D344 for CE3.  Coding efficiency results are confirmed. No mismatch between encoder and decoder is observed.  Relative encoding and decoding time have slightly difference, and the difference is under noise level.

# Introduction

This contribution is a report of verification results of JCTVC-D344, which is a contribution to CE3. Our cross check covers only part of the proposal which is DCTIF 8tap filter with 6 bit coefficients under Encoder Restriction. In addition, source code is reviewed.

# Experimental Results

Our encoding environment is using 64bit linux cluster and decoding was done on sinlge 64 bit windows PC. Only low-complexity cases (both random access and low delay) are required for cross-checking. There was no mismatch between encoder and decoder. There is no mismatch of overall BD-rate between the verification results and the proponent’s results.

Table 1. Results of Cross-checker

|  |  |  | Random access LoCo |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
|  | Class A | 3.1 | 1.8 | 1.5 |  |  |  |
|  | Class B | 1 | 1.3 | 1.3 | -0.2 | 0.3 | 0.6 |
|  | Class C | -2.8 | 0.1 | -0.1 | -3.2 | -2.6 | -2.8 |
|  | Class D | -8.1 | -2.9 | -2.9 | -7.4 | -6.5 | -6.7 |
|  | Class E |  |  |  | 0.9 | 1 | 1.4 |
|  | All | -2.2 | -0.1 | -0.2 | -2.5 | -2 | -1.9 |
|  | Enc Time[%] | 104% |  |  | 102% |  |  |
|  | Dec Time[%] | 108% |  |  | 106% |  |  |

Table 2 Samsung’s Results

|  |  |  | Random access LoCo |  | Low delay LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
|  | Class A | 3.1 | 1.8 | 1.6 |  |  |  |  |
|  | Class B | 1 | 1.3 | 1.3 | -0.1 | 0.3 | 0.6 |  |
|  | Class C | -2.8 | 0.2 | -0.2 | -3.2 | -2.7 | -2.8 |  |
|  | Class D | -8.1 | -2.9 | -2.9 | -7.5 | -6.5 | -6.8 |  |
|  | Class E |  |  |  | 0.9 | 1.1 | 1.5 |  |
|  | All | -2.2 | -0.1 | -0.2 | -2.5 | -2 | -1.9 |  |
|  | Enc Time[%] | 101% |  |  | 96% |  |  |  |
|  | Dec Time[%] | 114% |  |  | 107% |  |  |  |

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).