| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D430 |
| --- | --- |

| Title: | Evaluation of Entropy Slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Muhammed Coban Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1 858 658 3973 mcoban@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Entropy slice concept was proposed to HEVC [1]. Entropy slices provide the flexibility of decoupling the decoding and the reconstruction loop enabling possibly higher level parallel processing of video data. In this contribution we evaluate the coding performance of entropy slices using the software provided in [1].

# Introduction

Entropy slices provide the flexibility of decoupling the decoding and the reconstruction loop enabling possibly higher level parallel processing of video data. Entropy slices contain a sequence of largest coding units (LCUs). Entropy slices are similar to standard slices in the sense that they break the context model dependency between slices enabling independent parsing of the entropy slices. However unlike standard slices, entropy slices allow usage of neighboring slice information for reconstruction and deblocking processes.

# Results

We use the software provided in [1] that implements entropy slice concept into TMuC 0.7 for the evaluation of coding performance of entropy slices. We defined the slices such that there is a slice per LCU row.  In order to evaluate the performance of entropy slices versus the standard slices, we modified the software to add capability of configuring the entropy slices to behave like standard slices (completely breaking the dependency between slices). In this case both entropy slices and entropy slices that are configured as standard slices use the same entropy slice header.

We have also disabled the MDDT, ROT, EDGE_BASED_PREDICTION, PLANAR_INTRA macros in the software. We then coded all sequences using the configuration files described in JCTVC-B300 [2].  We ran following 3 configurations:

- Reference: no entropy slices and standard slices

- Entropy slices: one entropy slice per LCU row

- Simulated standard slices: one standard slice per LCU row

We have evaluated the high efficiency mode only using CABAC entropy coder. We have compared coding performance of (2) and (3) with respect to the reference (1). The results are shown in Table 1.

| Case 1: Entropy Slices |  |  |  |  |  |  | Case 2: Simulated Standard Slices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  |  |  |  |  | Intra |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  |  |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | 0.1 |  | 0.0 | 0.1 |  |  | Class A | 1.7 |  | 1.6 | 1.7 |  |
| Class B | 0.1 |  | 0.1 | 0.1 |  |  | Class B | 2.1 |  | 2.0 | 2.0 |  |
| Class C | 0.1 |  | 0.0 | 0.1 |  |  | Class C | 2.3 |  | 2.0 | 2.1 |  |
| Class D | 0.1 |  | 0.1 | 0.0 |  |  | Class D | 1.4 |  | 1.3 | 1.3 |  |
| Class E | 0.6 |  | 0.8 | -0.2 |  |  | Class E | 4.9 |  | 5.5 | 4.3 |  |
| All | 0.2 |  | 0.2 | 0.0 |  |  | All | 2.4 |  | 2.4 | 2.2 |  |
| Enc Time[%] | 100% |  |  |  |  |  | Enc Time[%] | 86% |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  | Dec Time[%] | 99% |  |  |  |  |
|  |  |  | Random access |  |  |  |  |  |  | Random access |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  |  |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | 3.0 |  | 4.7 | 4.1 |  |  | Class A | 3.6 |  | 5.3 | 4.8 |  |
| Class B | 5.2 |  | 6.9 | 5.5 |  |  | Class B | 6.4 |  | 8.4 | 7.2 |  |
| Class C | 4.7 |  | 5.9 | 5.9 |  |  | Class C | 5.7 |  | 7.0 | 7.0 |  |
| Class D | 4.1 |  | 4.8 | 4.8 |  |  | Class D | 4.7 |  | 5.2 | 5.4 |  |
| Class E |  |  |  |  |  |  | Class E |  |  |  |  |  |
| All | 4.5 |  | 5.8 | 5.2 |  |  | All | 5.4 |  | 6.8 | 6.3 |  |
| Enc Time[%] | 99% |  |  |  |  |  | Enc Time[%] | 97% |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  | Dec Time[%] | 101% |  |  |  |  |
|  | Low delay |  |  |  |  |  |  | Low delay |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  |  |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A |  |  |  |  |  |  | Class A |  |  |  |  |  |
| Class B | 4.6 |  | 7.3 | 6.3 |  |  | Class B | 5.0 |  | 8.0 | 7.0 |  |
| Class C | 3.7 |  | 4.9 | 4.8 |  |  | Class C | 4.1 |  | 5.6 | 5.6 |  |
| Class D | 3.8 |  | 3.9 | 3.8 |  |  | Class D | 3.9 |  | 4.1 | 4.0 |  |
| Class E | 15.1 |  | 14.2 | 12.9 |  |  | Class E | 15.4 |  | 14.6 | 13.5 |  |
| All | 6.1 |  | 7.2 | 6.5 |  |  | All | 6.4 |  | 7.7 | 7.1 |  |
| Enc Time[%] | 96% |  |  |  |  |  | Enc Time[%] | 96% |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  | Dec Time[%] | 99% |  |  |  |  |

Table 1: Average BD-rates [3] and relative encoding/decoding runtimes for high efficiency profile for entropy slices (case 1) and simulated standard slices (case 2).

# Conclusion

We have evaluated the coding performance of entropy slices and standard slices using the software provided in [1].

# References

- Kiran Misra, Jie Zhao and Andrew Segall, “Entropy Slices for Parallel Entropy Coding”, JCTVC-C256, Guangzhou, October, 2010.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-B300, Geneva, July, 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).