| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January, 2011 | Document: JCTVC-D439 WG11 Number: m19438 |
| --- | --- |

| Title: | Cross verification of JCTVC-D273 from TI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document reports cross-check results of the Panasonic proposal of modified derivation process of temporal motion vector predictor. It is verified that the coding efficiency data that Panasonic provided is accurate. It is also confirmed that the source code changes match the algorithm described in JCTVC-D273 [2].

# Introduction

This document reports cross-check results of the Panasonic proposal of Modified derivation process of temporal motion vector predictor [2].

# Algorithm Description

It has found that the following code has been added to TComDataCU.cpp, which basically adds an additional check of the availability of the co-located temporal motion vector predictor (MVP) in other direction if the temporal MVP is found to be unavailable based on the method defined in HM1. From the software implementation point of view, this change only requires one additional availability check; for the hardware implementation, it may need one additional memory access (to load the co-located temporal vectors in both directions simultaneously), depending on the architecture.

This code change matches the algorithm description in JCTVC-D273[2].

#ifdef PANASONIC_AMVPTEMPORALEXT else if ( pcCUColocated && !pcCUColocated->isIntra(uiAbsPartAddr) && pcCUColocated->getCUMvField(eColRefPicList2)->getRefIdx(uiAbsPartAddr) >= 0 ) { Int iColPOC = pcCUColocated->getSlice()->getPOC(); Int iColRefPOC = pcCUColocated->getSlice()->getRefPOC(eColRefPicList2, pcCUColocated->getCUMvField(eColRefPicList2)->getRefIdx(uiAbsPartAddr)); TComMv cColMv = pcCUColocated->getCUMvField(eColRefPicList2)->getMv(uiAbsPartAddr); Int iCurrPOC = m_pcSlice->getPOC(); Int iCurrRefPOC = m_pcSlice->getRefPic(eRefPicList, iRefIdx)->getPOC(); TComMv cMv; Int iScale = xGetDistScaleFactor(iCurrPOC, iCurrRefPOC, iColPOC, iColRefPOC); if (iScale == 1024) { cMv = cColMv; } else { cMv = cColMv.scaleMv( iScale ); } clipMv(cMv); pInfo->m_acMvCand[pInfo->iN++] = cMv ; } #endif

# Test Conditions

The TMuC-0.9 software is used for the evaluation, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Testing Results

## Comparison results against data provided by Panasonic

Perfect match has been achieved when compared to the data provided by Panasonic. For detail please refer to JCTVC-D439_TIvs.Panasonic.xls.

## Results compared to TMuC0.9 anchor

It is confirmed that there is an average about 0.3%-0.4% gain by introducing Panasonic’s proposal into the motion vector prediction. (see table below and refer to JCTVC-D439_anchorvs.TURes.xls for detail)

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | -0.6 | -0.6 | -0.7 | -0.5 | -0.4 |
| Class B | -0.3 | -0.2 | -0.4 | -0.2 | -0.1 | -0.1 |
| Class C | -0.4 | -0.4 | -0.4 | -0.3 | -0.3 | -0.4 |
| Class D | -0.3 | -0.3 | -0.2 | -0.3 | -0.3 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.4 | -0.3 | -0.4 | -0.3 | -0.3 | -0.3 |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 0% |  |  | 104% |  |  |
| Dec Time[%] | 0% |  |  | 106% |  |  |

# Conclusions

In this document the performance of Panasonic proposal on modified derivation process of temporal motion vector predictor is verified. It is recommended to consider this in the HEVC design.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, 3rd. JCT-VC Meeting, Guangzhou, CN, October 2010.

- JCT-VC.” Modified derivation process of temporal motion vector predictor”, JCTVC-D273, 4th JCT-VC Meeting, Daegu, KR, January 2011