| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January, 2011 | Document: JCTVC-D440 WG11 Number: m 19441 |
| --- | --- |

| Title: | Evaluation results on TPE |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document reports the TPE (transform precision extension) performance with respect to the TPE extension bits. It is recommended to use 2-bit TPE bits in the low-complexity configurations because it provides almost identical quality gains as compared to the 4-bit TPE bits which are used in the current TMuC-0.9 low-complexity configurations.

# Introduction

In the first test model of HEVC (HM1) the TPE (transform precision extension) was adopted into the low complexity configurations.

The document reports the TPE coding efficiency with respect to the number of TPE extension bits, it has been found that 2-bit TPE bits provides almost same quality gains as the 4-bit TPE bits in the low complexity configurations.

# Test Conditions

The TMuC-0.9 software is used for the evaluation, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

To change the TPE bits in the low complexity configurations when the number of TPE extension bits varies, in the simulation the following constants in CommonDef.h have been modified as follows:

| TPE bits = 4 | TPE bits = 3 | TPE bits = 2 | TPE bits = 1 | TPE bits = 0 |
| --- | --- | --- | --- | --- |
| g_iShift8x8 = 4; g_iShift16x16 = 4; g_iShift32x32 = 4; g_iShift64x64 = 4; | g_iShift8x8 = 3; g_iShift16x16 = 3; g_iShift32x32 = 3; g_iShift64x64 = 3; | g_iShift8x8 = 2; g_iShift16x16 = 2; g_iShift32x32 = 2; g_iShift64x64 = 2; | g_iShift8x8 = 1; g_iShift16x16 = 1; g_iShift32x32 = 1; g_iShift64x64 = 1; | g_iShift8x8 = 0; g_iShift16x16 = 0; g_iShift32x32 = 0; g_iShift64x64 = 0; |

# Testing Results

## TPE coding efficiency w.r.t TPE extension bits

The TPE coding efficiency is tested with TPE extension bits 0, 1, 2, 3 for the low complexity configurations. Table 1 shows the BD-rate increase compared to the anchor (which has 4 bits TPE extension) when the TPE extension bits are reduced. As shown in Table 1, using 2-bit TPE extension already captures almost all the potential gain of the TPE.

|  | Intra LOCO (TPE bits) |  |  |  | Random access LOCO (TPE bits) |  |  |  | Low-delay LOCO (TPE bits) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| Class A | 1.1 | 0.2 | 0.0 | 0.0 | 1.0 | 0.1 | 0.0 | 0.0 |  |  |  |  |
| Class B | 1.6 | 0.3 | 0.0 | 0.0 | 1.6 | 0.1 | 0.0 | 0.0 | 0.8 | 0.1 | 0.0 | 0.0 |
| Class C | 0.4 | 0.2 | 0.0 | 0.0 | 0.6 | 0.0 | 0.0 | 0.0 | 0.3 | 0.1 | 0.0 | 0.0 |
| Class D | 0.2 | 0.1 | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 1.8 | 0.0 | 0.0 | 0.0 |
| Class E | 2.0 | 0.3 | 0.1 | 0.0 |  |  |  |  | 1.1 | 0.0 | 0.0 | 0.1 |
| All | 1.0 | 0.2 | 0.0 | 0.0 | 0.9 | 0.1 | 0.0 | 0.0 | 1.6 | 0.1 | 0.0 | 0.0 |

Table 1 BD-rate increase of Y component (TPE extension bits = 0, 1, 2, 3 vs. anchor TPE extension bits = 4)

For detailed results please refer to

TMuC0.9-anchor_vs.TPE0.xls

TMuC0.9-anchor_vs.TPE1.xls

TMuC0.9-anchor_vs.TPE2.xls

TMuC0.9-anchor_vs.TPE3.xls

# Conclusions

Evaluation results reveal that using 2-bit TPE extension already captures almost all of the potential TPE gain. It is recommended to reduce the TPE extension bits from 4 to 2 in the HEVC low-complexity configurations.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, 3rd. JCT-VC Meeting, Guangzhou, CN, October 2010.