| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D445 WG11 Document: m19461 |
| --- | --- |

| Title: | CE10: Core transform design |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by JCT-VC |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala, FastVDO LLC Madhukar Budagavi, Texas Instruments Arild Fuldseth Cisco Rajan Joshi Qualcomm Il-Koo Kim Samsung Electronics | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1-443-455-0394 pankajtva@yahoo.com madhukar@ti.com arildfuld@cisco.com rajanj@qualcomm.com ilkoo.kim@samsung.com |
| Source: | FastVDO LLC, Texas Instruments, Cisco, Qualcomm, Samsung Electronics |  |  |

_____________________________

# Objective

This CE shall investigate the design of core spatial transforms, and study the tradeoff between coding efficiency and complexity, return evidence and a recommendation for design selection. The anchor for comparison shall be the transforms in the HM. Proposals that use partial frequency (e.g., D257) are included as complexity reduction techniques at the cost of efficiency, as well as dynamic range control methods (e.g., D071). Proposals that use such methods incorporate them into the complexity analyses below.

# Participant list

| Name | Affiliation | Email |
| --- | --- | --- |
| Pankaj Topiwala | FastVDO LLC | pankajtva@yahoo.com |
| Madhukar Budagavi | Texas Instruments | madhukar@ti.com |
| Arild Fuldseth | Cisco | arildfuld@cisco.com |
| Rajan Joshi Ilkoo Kim | Qualcomm Samsung Electronics | rajanj@qualcomm.com ilkoo.kim@samsung.com |

# Description of tools

Main: D036, D224, D256, D339, D365.

Supplementary: D071, D257.

# Core Experiment Tasks

B.1. Coding efficiency

B.1.1. Measure the RD performance of candidate transform designs using common conditions D600 and HM 2.0 software. If a transform design uses clipping etc. on intermediate variables, this clipping should be included in coding efficiency tests.

B.1.2. Also test at low and high QP values.   Provide BD-Rate results for the following QP sets: 1, 5, 9, 13 and 36, 42, 47, 51 to see if there is unusual behavior at these uncommon test points.  Run this test on 2 sec of video to speed up this test. Number of frames to encode is provided in the table below.

|  |  | Number of frames to encode |
| --- | --- | --- |
| S01 | Traffic | 60 |
| S02 | PeopleOnStreet | 60 |
| S03 | Kimono | 48 |
| S04 | ParkScene | 48 |
| S05 | Cactus | 100 |
| S06 | BasketballDrive | 100 |
| S07 | BQTerrace | 120 |
| S08 | BasketballDrill | 100 |
| S09 | BQMall | 120 |
| S10 | PartyScene | 100 |
| S11 | RaceHorses (C) | 60 |
| S12 | BasketballPass | 100 |
| S13 | BQSquare | 120 |
| S14 | BlowingBubbles | 100 |
| S15 | RaceHorses (D) | 60 |
| S16 | Vidyo1 | 120 |
| S17 | Vidyo3 | 120 |
| S18 | Vidyo4 | 120 |

Remark: To do by proponents, anyone can crosscheck.

B.2. Complexity

Analyze the complexity of candidate transform designs using the following parameters:

- Arithmetic operations

- Software execution speed

- Dynamic range

- Parallelizability on SIMD

- Throughput

- HW

Proponents are encouraged to provide additional complexity analyses voluntarily.

Discussion

Complexity is multidimensional, and it is important to capture the essence of this space, but at the same time, keep the analysis space limited in size. A number of complexity metrics are listed below, and it was the sense of the group to do more, rather than less, testing.  These tests are constructed to explore suitability of designs in both software and hardware.  Some of these tests are "paper" tests, while others require actual coding and optimization. At this time, some tests are considered required, while others are optional. This is based on the fact that some tests require optimization, in software or hardware, which it may be unreasonable to require proponents to produce.  Nevertheless, it is the intent of the group to conduct even the optional tests, by recruiting volunteers. It is recognized that we are dealing with an exceptionally short meeting cycle (6 weeks), which makes completing all tasks in one meeting cycle challenging.  It is hoped to collect all relevant evidence in at most two meeting cycles.

B.2.1. Arithmetic Operations

Report the number of arithmetic operations needed for implementing the 2D inverse transform, in terms of multiplications, additions, shifts, compares. If the design allows flexibility in the way it is implemented, report the range of arithmetic operations available (for example, methods with and without multiplications).  Include in the count TPE and final rounding if used.  [Paper study]

Remark: To do by proponents, anyone can crosscheck.

B.2.2. Software Execution Speed

Develop a software model of candidate (transform + quantization modules) in single-thread C++ compatible with current HM software.  Develop a common test environment for testing all candidates.  In software tests below, run the tests N times, for N sufficiently large to smooth variations (e.g., N = 100K). Report total execution times.

B.2.2.1. IBDI Off

Develop a test input frame of a random noise (9-bit, [-255, 255]), and run N trials of the modules:

[forward transform, quantization, dequantization, inverse transform].

B.2.2.2. IBDI On

Develop a test input frame of a random noise (11-bit, [-1020, 1020]), and run N trials of the modules:

[forward transform, quantization, dequantization, inverse transform].

Remark: Can do by proponents, others can crosscheck. Date: 2/20.  But prefer to have someone build the software environment to facilitate testing all designs by anyone.  TI volunteers to develop a common test environment.  Date: 2/20. (Can we do this by 2/10?.)

B.2.3. Dynamic Range Analysis

(Optional)

B.2.3.1. Report if the candidate transform design is exactly invertible in fixed-point arithmetic, and if so, at what bitdepth.    [Paper study]

B.2.3.2. In the software module developed for B.2.2., test the dynamic range of the data at four points: prior to dequantization, prior to inverse transform, after first-stage inverse transform, and after second-stage inverse transform.  Example input may be data consisting of +/- 255, either random, or Hadamard-like. (TBD)

Remark:  The methods of D071 (clipping, etc.) are available to any candidate design.  If used, this exercise is then unnecessary.  However, that clipping functionality should be part of the software module tested for coding efficiency.

Remark: To do by proponents, anyone can crosscheck. Alternatively, better to work within a common environment, so that this can be checked by anyone.

B.2.4. Parallelizability on SIMD

Develop an optimized implementation for a target SIMD architecture (e.g., Intel Core i7 platform) for candidate designs [forward transform, quantization, dequantization, inverse transform]. Combine source codes into one environment (machine, OS, compiler, SW package) for comparative testing.

Remark: We have a volunteer to conduct this test: David Flynn (BBC).  Recommend that this be done in a common environment so anyone can test, crosscheck.

B.2.5. Throughput.

Report the number of cascaded operators (multipliers/adders) in the longest execution path.  [Paper study]

Remark: Ideally done by our hardware specialists, rather than proponents.

[Optional]

B.2.6. HW

B.2.6.1. Develop optimized hardware models (RTL) of candidate transform designs, and determine the silicon area required, where optimization is for a common hardware condition.  Details to be determined.

B.2.6.2. Determine the hardware speed.  From area and speed, power consumption can be estimated. This subtask is intended to be a paper study, given B.2.6.1.

Remark: Need specific tasks, common hardware conditions, distribution of task responsibilities..  When will these be available? Let's get this by 2/7 if possible.  Not expecting this task to be done in 1 meeting cycle.

Volunteers:

TI / Samsung volunteer to implement three proposals (Cisco/TI, Samsung/FastVDO, Qualcomm) in hardware, and cross-verify, for this test - which is greatly appreciated by the group.

# Timeline

Timeline1: Common test framework for stand-alone testing in B.2.2 and B.2.3 expected to be available.

Timeline2: HM 2.0 software expected to be available (2 weeks after 1/28/11).

Timeline3: Final CExx description uploaded

Timeline4: Cross-verification begins by this date

Timeline5: Reporting of all results to CExx coordinators complete

Timeline6: Input documents uploaded