| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D446 |
| --- | --- |

| Title: | CE1: Cross-verification report for the part of JCTVC-D167 proposed by Intel |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Motoharu Ueda JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-9945 ueda.motoharu@jk-holdings.com |
| Source: | JVC KENWOOD Holdings, Inc. |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results for 9candidates SDME without MV refinement setting of JCTVC-D167 proposed by Intel.
The results show that 9 candidate based SDME technique provides 1.7% gain at random access high efficiency case with 19% increase in encoding time and 6% increase in decoding time. These performance comparison results are matched with the results of JCTVC-D167 and the complexity comparison results are close to the results of JCTVC-D167.

# Introduction

The verified method is self derivation of motion estimation (SDME) technique proposed by Intel and evaluated [1] in CE1 activity.

The proposed technique of SDME decrease the complexity of encoder and decoder by using rounded candidates to integer pixel positions to avoid fractional pixel interpolation, and controlled the setting of the number of candidates and performing MV refinement or not.

For the setting of 6 candidates with/without MV refinement, cross-verification report was reported [2].

In this report, cross-verification results for 9 candidates SDME without MV refinement setting described as the additional results (table.9) in JCTVC-D167 are reported.

# Experiments

## Test conditions

In CE1 activity [3], the common test conditions [4] are used for experiments of performance and complexity toward HM. In this report, the cross-verification was performed to the 9candidates SDME without MV refinement setting (table.9 in JCTVC-D167) of Intel’s proposal, and it was reported results at random access case high efficiency.

## Experimental environment

The implemented software based on TMuC0.9 was distributed by Intel and the cross-checker compiled the software themselves and executed.

The experimental environment is shown in Table 1.

Table 1 Experimental environment

| CPU | Intel Core i7 870 2.93GHz |
| --- | --- |
| Memory | 8GB (DDR3) |
| OS | Windows XP SP2 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | Win32(encoder/decoder) |

## Simulation Results

### Performance comparison

Table 2 shows the performance comparison results at random access high efficiency.

Table 2 Performance comparison results at random access high efficiency

|  | Random access |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.7 | -2.8 | -2.6 |
| Class B | -1.6 | -1.7 | -1.6 |
| Class C | -1.5 | -1.8 | -1.9 |
| Class D | -1.5 | -1.8 | -1.7 |
| Class E |  |  |  |
| All | -1.7 | -1.9 | -1.8 |

The results show that 9candidates SDME without MV refinement technique provides 1.7% BD-rate [5] gain at random access high efficiency case against the HM (TMuC0.9 anchor).

These results are matched with the results of the proposal JCTVC-D167.

### Complexity comparison

For the complexity comparison of decoder, two results of decoding time are used for comparison. One is decoding time with writing out decoded yuv (with –o option), and the other is decoding time without writing out decoded yuv (without –o option).

Table 3 shows the complexity comparison results at random access high efficiency.

Table 3 Complexity comparison results at random access high efficiency

|  | Random access High |
| --- | --- |
| Encoding time | 119% |
| Decoding time with "-o" option | 106% |
| Decoding time without "-o" option | 107% |

The results of runtime are close to the results of 9 candidates SDME without MV refinement in JCTVC-D167.

## Source code verification

We have verified the source code distributed by Intel about the macro INTEL_DMVD.

In the case that INTEL_DMVD is defined as 1, SDME technique is performed for Bi-predictive interpolation case. The algorithm of SDME is controlled by the parameters in the configuration files.

# Conclusions

This contribution reports cross-verification results of 9candidates SDME without refinement setting of JCTVC-D167 proposed by Intel.

The results show that 9 candidate based SDME technique provides 1.7% gain at random access high efficiency case with 19% increase in encoding time and 6% increase in decoding time. These performance comparison results are matched with the results of JCTVC-D167 and the complexity comparison results are close to the results of JCTVC-D167.

The rounded-candidates based SDME technique can improve coding efficiency under random access condition with small increase of runtime

.

# References

[1] “CE1: Report of self derivation of motion estimation in TMuC 0.9”,

JCTVC-D167, Daegu, January 2011.

[2] “Report of CE1 :Decoder-Side Motion Vector Derivation”,

JCTVC-D098, Daegu, January 2011

[3] “Core Experiment 1:”,

JCTVC-C501, Guangzhou, October 2010.

[4] “Common test conditions and software reference configurations”,

JCTVC-C500, Guangzhou, October 2010.

[5] G. Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. and Victor Company of Japan, Limited may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).