| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D448 |
| --- | --- |

| Title: | CE1 Subtest1: A joint proposal of candidate-based decoder-side motion vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Yi-Jen Chiu1 Lidong Xu1 Wenhao Zhang1 Hong Jiang1 Mingyuan Yang2 Sixin Lin2 Haoping Yu2 | Emails: | Yi-jen.chiu@intel.com lidong.xu@intel.com wenhao.zhang@intel.com hong.h.jiang@intel.com myyang@huawei.com linsx@huawei.com haopingyu@huawei.com |
| Source: | Intel Corp.1, Huawei Technologies Co., Ltd.2 |  |  |

_____________________________

# Abstract

This contribution describes a joint recommendation of the techniques of JCTVC-D167 and JCTVC-D295: candidate-based Decoder-side Motion Vector Derivation (DMVD) technique to improve the coding efficiency on TMuC 0.9 reference software. The recommendation of the Candidate-based-DMVD (C-DMVD) has demonstrated that an overall 1.97% BD bitrate improvement for the test of random access (high efficiency) category under the common test conditions with 23% increase in encoding time and 5% increase in decoding time. A description about the normative syntax change about C-DMVD in the form of HEVC specification is included. Mitsubishi is currently cross-checking the result and the estimate of the increase in encoding time is around 20% and the increase of decoding time is around 7% .

- Introduction

Various decoder-side motion vector derivation (DMVD) techniques have been proposed to improve the coding efficiency of hybrid based video encoder [1-9]. Candidate-based decoder-side motion vector derivation (C-DMVD) techniques are the popular techniques to be study in CE1 subtests 1 and subtest 2. In this proposal, a recommendation of common tools from JCTVC-D167 and JCTVC-D295 is presented to improve the coding efficiency based on the common techniques described in [10-11]. Template matching and motion estimation are not part of this recommendation.

# Proposed Techniques

## C-DMVD overview

The candidate-based C-DMVD is proposed to improve the coding efficiency of B picture for the next generation of video coding standard. Figure 1 shows the block diagram of a typical hybrid-based video encoder with SDME module being added. The C-DMVD module performs motion search only on reference pictures, and the produced motion vector could be applied to current target prediction unit (PU) for motion compensation when the best RDO performance is achieved with C-DMVD mode during the phase of coding mode selection. On the video decoder side, the decoder performs the identical C-DMVD operation to produce the motion compensated predictions for the target PU when the said target PU is encoded in SDME mode.

Figure 1 - Block diagram of hybrid video encoding with SDME being supported

## C-DMVD for B picture

- A PU-Based coding

- 9 candidate motion vector pairs: spatial, temporal and mirror

- Asymmetrical rounding process to provide for integer rounding

- SAD calculation based on two reference blocks pointed by a motion vector pair.

- CMVP coding mode, an indication flag per PU

  - Skip/Direct modes

  - 2NxN, Nx2N

## Tool description

### 9 candidate motion vector pairs

The C-DMVD module first checks 9 candidate motion vector pairs. The candidates were rounded to integer pixel positions to avoid fractional pixel interpolation. The 9 candidates are selected from the MVs of spatial neighboring PUs and temporal collocated PU, as illustrated in Figure 2 and depicted in Table 1.

All candidate MVs are scaled (by POC distances) to the nearest forward and backward reference pictures. Each candidate consists of one forward MV and one backward MV. Since the proposed DMVD is performed between the two reference pictures, the decoding process does not have dependency on neighbor PU reconstructions.

Figure 2 – Candidate MVs from spatial neighboring PUs

Table 1 - Description of the 9 candidates

| Base MV | 0 | zero MV |
| --- | --- | --- |
| Predicted MVs | 1 | MVs of left neighbor PU A: a. If it is bi-predicted, use the two MVs b. If it is FW predicted, get the BW MV by mirroring the FW MV c. If it is BW predicted, get the FW MV by mirroring the BW MV d. If it is intra coded, this candidate will be unavailable |
|  | 2 | MVs of top neighbor PU B (detail condition as described in 1) |
|  | 3 | MVs of top-right neighbor PU C (detail condition as described in 1) |
| Spatial neighbor MVs Scaled by POC distances | 4 | MVs of below-left neighbor PU E (detail condition as described in 1) |
|  | 5 | MVs of top-left neighbor PU D (detail condition as described in 1) |
|  | 6 | Median filtered MVs of the above candidates 1, 2 and the first available candidates of 3, 4 and 5. |
| Temporal Neighbor MVs Scaled by POC distances | 7 | MVs of collocated neighbor PU in List0 reference (detail condition as described in 1) |
|  | 8 | MVs of collocated neighbor PU in List1 reference (detail condition as described in 1) |

### Asymmetrical rounding process used in search of MV in DMVD

To reduce the complexity of getting the best motion vector from the candidate set, an unsymmetrical rounding process is used to reduce the number of candidate motion vectors by moving the motion vectors on the fractional pixel locations to the nearby integer pixel locations. Then only these motion vectors at the integer pixel locations are evaluated in the process searching for the best motion vector for the current coding block.

Below is the rounding process, and its characteristics are illustrated in the table below:

Int mv_temp = mv & 0x3;

if ((mv_temp==1) && (mv < 0))
      mv= mv - 1;
   else if((mv_temp == 3) && (mv > 0))
      mv = mv +1;

mv = ((mv >>2)<<2);

| Fractional pixel location | -1 1/2 | -1 1/4 | -1 | -3/4 | -2/4 | -1/4 | 0 | 1/4 | 2/4 | 3/4 | 1 | 1 1/4 | 1 2/4 | 1 3/4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rounding Process | -2 | -2 | -1 | -1 | -1 | -1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 |

### C-DMVD based coding modes

The C-DMVD based motion vector derivation method can be used as a new coding mode in addition to the existing coding modes of the regular ME based coding structure. In bitstream syntax, a C-DMVD flag is added for each PU to signal the occurrence of C-DMVD mode. The C-DMVD mode is applied to the bi-prediction coding modes of all PU sizes (Skip/Direct modes, 2NxN, Nx2N). For 2Nx2N PUs, SDME is applied only to Skip/Direct modes to tradeoff complexity

# Required modifications to HEVC text specification

To support the proposed DMVD techniques in HEVC, the following modification to HEVC text specifications [12] is provided.

## Syntax

The modifications to syntax tables are marked in yellow.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| bit_depth_luma_increment | ue(v) |
| bit_depth_chroma_increment | ue(v) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| interpolation_filter_flag | u(1) |
| BiDMVDFlag | u(1) |
| if( BiDMVDFlag ) { |  |
| BiDMVDRefineFlag | u(1) |
| BiDMVDSampledSADFlag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( BiDMVDFlag && slice_type == B && !slice_is_GPB) | u(1) ¦ ae(v) |
| skip_dmvd_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( !skip_dmvd_flag[ x0 ][ y0 ]){ |  |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } else if( PredMode = = MODE_INTRA ) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( !prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { /* MODE_MERGE, MODE_DIRECT, MODE_INTER */ |  |
| if( merge_flag[ x0 ][ y0 ] && NumMergeCandidates > 1 ) { |  |
| merge_left_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } else if( PredMode = = MODE_DIRECT ) { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( BiDMVDFlag && slice_type == B && !slice_is_GPB && inter_pred_idc[ x0 ][ y0 ]== Pred_Bi) |  |
| direct_dmvd_flag[ x0 ][ y0 ] | ue(1) ¦ ae(v) |
| if( !direct_dmvd_flag[ x0 ][ y0 ]) { |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 && NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 && NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( BiDMVDFlag && slice_type == B && !slice_is_GPB && inter_pred_idc[ x0 ][ y0 ]== Pred_Bi && PartMode != PART_2Nx2N && PartMode != PART_NxN) |  |
| pu_dmvd_flag[ x0 ][ y0 ] | ue(1) ¦ ae(v) |
| if( !pu_dmvd_flag[ x0 ][ y0 ]) { |  |
| if( inter_pred_idc[ x0 ][ y0 ] ! = Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0 | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1 | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Semantics

BiDMVDFlag specifies whether the DMVD method is used in bitstream.

- If 1, DMVD derivation method is enabled. Otherwise, it is disabled.

BiDMVDRefineFlag

- If 1, SDME refinement is enabled to be performed after rounded-candidate checking.

- Otherwise, SDME refinement is disabled.

BiDMVDSampledSADFlag

- If 1, SAD is calculated using down-sampled PU in SDME process.

- Otherwise, SAD is calculated using the full PU in SDME process.

skip_dmvd_flag[x0][y0] specifies the MV derivation method for Skip mode. Default value is 0.

- If 1, DMVD derivation method is used.

- Otherwise HM original derivation method is used.

pu_dmvd_flag[x0][y0] specifies the MV derivation method for the PU if it is predicted in bi-direction mode. It is not applied to 2Nx2N and NxN PUs. Default value is 0.

- If 1, DMVD derivation method is used.

- Otherwise HM original derivation method is used.

direct_dmvd_flag[x0][y0] specifies the MV derivation method for bi-direction predicted direct mode. Default value is 0.

- If 1, DMVD derivation method is used.

- Otherwise HM original derivation method is used.

## Decoding process

The following paragraph is provided to be added into the decoding process section of [10].

### Derivation process for MV components and ref. indices of DMVD prediction units

DMVD prediction is only applicable to the bi-directional prediction modes of non-GPB B slices.

Input to this process is a prediction partition index puPartIdx.

Outputs of this process are

- luma motion vectors mvL0 and mvL1 and when ChromaArrayType is not equal to 0, the chroma motion vectors mvCL0 and mvCL1,

- reference indices refIdxL0 and refIdxL1.

The variables mvLX and refIdxLX (with X being 0 or 1) are derived as follows.

- The variables refIdxLX are set to the indices of the nearest reference frame in the forward and backward reference buffers, i.e., refIdxL0 = refIdxL1 = 0.

- The variables mvLX are obtained through the SDME process described in Section 2.

# Performance

The encoder is complied in Linux OS and is tested in a compute farm consisting of hundred of CPU cores. The computing cores in the system may not be identical, and information of the log of encoding time ratio may produce slightly different result in each occurrence of the simulation. The decoder was tested on a single workstation with the following configurations: Windows Server 2003 Enterprise x64 Edition, Intel Xeon CPU E5335 @ 2.00GHz, 8GB RAM, and Visual Studio 2008.

## C-DMVD for B-picture

The proposed C-DMVD techniques were implemented onto TMuC0.9 Reference Software and evaluated under the test conditions defined by the CE1 group [12]. The experiment is investigated on the test case of random access because C-DMVD is for improving the coding efficiency of B picture.

Table 2 shows the C-DMVD performance compared with the CE1 anchors. In high efficiency case, the average BD bitrate reduction for Luma component is about 1.97% with about 23% increase in encoding time and about 5% increase in decoding time.

Table 2 - C-DMVD performance (high efficiency)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.85 | -3.04 | -2.96 | -2.09 | -2.08 | -2.04 |
| Class B | -1.74 | -1.63 | -1.64 | -1.48 | -1.39 | -1.45 |
| Class C | -1.80 | -1.98 | -2.06 | -1.34 | -1.44 | -1.48 |
| Class D | -1.98 | -1.96 | -2.13 | -1.56 | -1.54 | -1.47 |
| Class E |  |  |  |  |  |  |
| All | -1.97 | -2.00 | -2.06 | -1.54 | -1.54 | -1.54 |
| Enc Time[%] | 123% |  |  | 126% |  |  |
| Dec Time[%] | 105% |  |  | 114% |  |  |

Table 3 shows the data of Table 2 test from cross-checker (Mitsubishi). The numbers in blue are the estimate value: there are still 3 test cases yet to complete at the preparation of this document.

Table 3 - C-DMVD performance (high efficiency) (From Cross-checker)

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.85 | -3.04 | -2.96 | -2.09 | -2.08 | -2.04 |
| Class B | -1.74 | -1.63 | -1.64 | -1.48 | -1.39 | -1.45 |
| Class C | -1.80 | -1.98 | -2.07 | -1.34 | -1.44 | -1.48 |
| Class D | -1.98 | -1.96 | -2.13 | -1.56 | -1.54 | -1.47 |
| Class E |  |  |  |  |  |  |
| All | -1.97 | -2.00 | -2.06 | -1.54 | -1.54 | -1.54 |
| Enc Time[%] | 120% |  |  | 117% |  |  |
| Dec Time[%] | 107% |  |  | 119% |  |  |

# Conclusions

With the performance result demonstrated sizeable coding efficiency improvement with feasible complexity increase, the proponents suggest JCT-VC committee to adopt the proposed techniques into HM to be further studied by broader community.

# References

- S. Kamp, M. Evertz, M. Wien, “Decoder side motion vector derivation,” ITU-T Q.6/SG16 33rd Meeting Document VCEG-AG16, October 2007.

- Y.-W. Huang, C.-Y. Chen, C.-W. Hsu, J.-L. Lin, Y.-P. Tsai, J. An, S. Lei, “TE1: Decoder-side motion vector derivation with switchable template matching”, Meeting Document JCTVC-A076, April 2010.

- W.-J. Chien, M. Karczewicz, P. Chen, “TE1: Decoder-side motion vector derivation report from Qualcomm”, Meeting Document JCTVC-A097, April 2010.

- M. Ueda, “TE1.a: Implementation report of Refinement Motion Compensation using DMVD on TMuC”, Meeting Document JCTVC-C138, Oct. 2010.

- M. Yang, S. Lin, D. Wang, J. zhou, and H. Yu, “TE1：Huawei report on DMVD improvements in TMuC”, Meeting Document JCTVC-C097, Oct. 2010.

- Y. Itani, S. Sekiguchi, and T. Murakami, “TE1 report: implicit direct vector derivation”, Meeting Document JCTVC-C124, Oct. 2010.

- Y. Chiu, L. Xu, W. Zhang, H. Jiang, “Description of video coding technology proposal: self derivation of motion estimation and adaptive (Wiener) loop filtering”, Meeting Document JCTVC-A106, April 2010.

- Y. Chiu, L. Xu, W. Zhang, H. Jiang, “TE1: Fast techniques to improve self derivation of motion estimation”, Meeting Document JCTVC-B047, July 2010.

- Y. Chiu, L. Xu, W. Zhang, H. Jiang, “TE1: Report of self derivation of motion estimation improvement in TMuC”, Meeting Document JCTVC-C127, October 2010.

- Y. Chiu, L. Xu, W. Zhang, H. Jiang, “CE1: Report of self derivation of motion estimation in TmuC0.9”, Meeting Document JCTVD-C167, January, 2011.

- M. Yang, S. Lin, H. Yu, “TE1：Huawei report on TMDMVD and STDM in HM”, Meeting Document JCTVC-D295, January, 2011.

- “WD1: Working Draft 1 of High-Efficiency Video Coding”, Meeting Document JCTVC-C403, Oct. 2010.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

Intel Corporation, Huawei Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).