| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D453 |
| --- | --- |

| Title: | Cross-check report of mode dependent transform coefficients scanning from JCTVC-D393 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jianle Chen Vivienne Sze Krit Panusopone Ali Tabatabai Muhammed Coban | Email: | jianle.chen@samsung.com sze@ti.com krit@motorola.com Ali.Tabatabai@am.sony.com mcoban@qualcomm.com |
| Source: | TI, Sony, Samsung Electronics, Qualcomm |  |  |

_____________________________

# Abstract

This contribution summarizes the cross-check activities on transform coefficients scanning method in CE11. Mode Dependent Coefficient Scanning scheme of JCTVC-D393 was investigated.

Code was verified to be a match with proposal. High efficiency configuration results are partially cross-checked by Texas Instruments (Intra only configuration are fully cross-checked). Low complexity configuration results were cross-checked fully by Samsung and partially by Sony.

# Introduction

Mode Dependent Coefficient Scanning (MDCS) scheme of JCTVC-D393 has been modified to accommodate the changes related to replacement of the HHI transform coding scan with zigzag scan, and the simplification of the significance map coding context JCTVC-D260. The proposed MDCS algorithm is applied to intra TU coding. Three scan patterns are used: zigzag (0), horizontal (1) and vertical (2) as shown in Figure 1.

Figure 1: MDCS coefficient scan patterns, zigzag (0), horizontal (1), vertical (2).

The scanning order of a TU is determined by the intra prediction mode and the TU size using a fixed look-up table (LUT). Luma blocks use Table 1 and the chroma blocks use Table 2.

| Mode TU | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32x32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 16x16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8x8 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |
| 4x4 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |

Table 1: LUT for intra luma coefficient scan index selection.

| Mode TU | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| 32x32 |  |  |  |  |  |
| 16x16 | 0 | 0 | 0 | 0 | Same as corresponding luma block |
| 8x8 | 0 | 0 | 0 | 0 | Same as corresponding luma block |
| 4x4 | 1 | 2 | 0 | 0 | Same as corresponding luma block |

Table 2: LUT for intra chroma coefficient scan index selection.

The significance map coding context support for all scan patterns uses JCTVC-D260 context support shown Figure 2.

Figure 2: Significance map coding context support.

# Test Results

## High Efficiency

| Configuration | MDCS D260 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| HE Intra | -0.9 | -1.1 | -1.1 |
| HE Random Access | -0.2 | -0.2 | -0.3 |
| HE Low Delay | -0.1 | -0.1 | 0.0 |

Table 3: BD-rate performance of MDCS+SONY_CTX(D260) for TMuC0.9 anchor.

Table 3 shows the results of Mode Dependent Coefficient Scanning scheme for TMuC 0.9 anchor.

| Configuration | MDCS (D260 context) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| HE Intra | -1.2 | -1.6 | -1.6 |
| HE Random Access | -0.5 | -0.5 | -0.8 |
| HE Low Delay | -0.3 | -0.3 | -0.4 |

Table 4: BD-rate performance of MDCS+SONY_CTX(D260) for TMuC0.9 (DISABLE_HHI_SCAN) anchor.

Table 4 shows the results of Mode Dependent Coefficient Scanning scheme for TMuC 0.9 (DISABLE_HHI_SCAN) anchor.

## Low Complexity

Table 5 shows gains of MDCS in low complexity configuration.

| Configuration | MDCS |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Loco Intra | -0.6 | -0.5 | -0.6 |
| Loco Random Access | -0.3 | -0.1 | -0.2 |
| Loco Low Delay | -0.1 | -0.1 | 0.0 |

Table 3: BD-rate performance of MDCS for TMuC0.9 (JCTVC-D374) anchor.

# Complexity (by Proponent)

Complexity involves two additional scans for 4x4 and 8x8 block, logic to select LUT based on TU, and two LUTs (34x1 for luma and 4x1 for chroma) to map mode to scan.

| Configuration | MDCS |  |
| --- | --- | --- |
|  | Encoder | Decoder |
| HE Intra | 100% | 100% |
| HE Random Access | 100% | 101% |
| HE Low Delay | 101% | 100% |

Table 4: Encoding/decoding time ratios for HE case. (MDCS+SONY_CTX(D260) vs. TMuC0.9 (DISABLE_HHI_SCAN) anchor)

| Configuration | MDCS |  |
| --- | --- | --- |
|  | Encoder | Decoder |
| LoCo Intra | 101% | 100% |
| LoCo Random Access | 100% | 100% |
| LoCo Low Delay | 101% | 101% |

Table 5: Encoding/decoding time ratios for LoCo case. (MDCS vs. TMuC0.9 (JCTVC-D374) anchor.)

# Cross-Check (Texas Instruments, Sony, Samsung)

Software for high efficiency and low complexity has been analyzed and confirmed to match proponents’ description of tool.

- Texas Instruments has verified the High Efficiency Intra coding efficiency results. Some mismatch with encoding/decoding time reported.

- Sony has verified has partially verified Low Complexity results.

- Samsung has verified all Low Complexity results.

# Conclusions

MDCS algorithm is an intra only algorithm that introduces two additional scan patterns that can be computed on the fly. These additional scan patterns are in effect for block sizes 4x4 and 8x8. In addition to the scan patterns, there are 2 LUT (map TU size and intra prediction mode to scan pattern). The significance map coding context pattern is preserves the parallel context processing feature since effectively for block sizes 16x16 and higher only zigzag pattern is used.

# References

- Y. Zheng, M. Coban, J. Sole, R. Joshi, M. Karczewicz, “CE11: Mode Dependent Coefficient Scanning”, JCTVC-D393.

- M. Karczewicz, X. Wang, W.-J. Chien, “CE5: Improved coefficient coding with LCEC”, JCTVC-D374

- C. Auyeung, W. Liu, "Parallel processing friendly simplified context selection of significance map", JCTVC-D260