| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D602 WG11 Number: m19488 |
| --- | --- |

| Title: | Core Experiment 2: Flexible Motion Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng (coordinator), Philippe Bordes, Peisong Chen, Il-Koo Kim (vice-coordinators) | Tel: Email: | +86-755-28422009 zhengxiaozhen@huawei.com philippe.bordes@technicolor.com peisongc@qualcomm.com ilkoo.kim@samsung.com |
| Source: | Huawei & HiSilicon Technologies, Technicolor, Samsung, Qualcomm |  |  |

_____________________________

- Introduction

This Core Experiment (CE) is an ongoing work of previous CE2 – Flexible Motion Partitioning in Error: Reference source not found. Its goal is to further investigate block partition issue.

Several related techniques have been considered in the 4th JCTVC meeting, with promising BD-rate performance, but with some remaining concerns regarding encoding complexity. It should be noted that the decoder complexity is not impacted.

The techniques considered in this CE are:

- Asymmetric Motion Partitioning (AMP), initially proposed in [2];

- Non-rectangular Motion Partitioning (NRMP), proposed in [5], [6];

- Overlap Block Motion Compensation (OBMC), proposed in [3], [4]

Further improvements are required to find the best unified solution of those three methods with the optimal trade-off coding efficiency/encoding complexity.

# Participants List

| Participant | Person, Email address |
| --- | --- |
| Technicolor | Philippe Bordes, philippe.bordes@technicolor.com |
| Qualcomm | Peisong Chen, peisongc@qualcomm.com Liwei Guo, liweig@qualcomm.com |
| Huawei/HiSilicon | Xiaozhen Zheng, zhengxiaozhen@huawei.com Haoping Yu, haopingyu@huawei.com |
| Samsung | Il-Koo Kim, ilkoo.kim@samsung.com |
| HHI | Martin Winken, Martin.Winken@hhi.fraunhofer.de Benjamin Bross, benjamin.bross@hhi.fraunhofer.de |
| NTT Docomo | TK Tan, ttk@pacific.net.sg |
| INRIA | laurent.guillo@inria.fr;ronan.boitard@inria.fr |

# Tools under Test

It has been reported in JCTVC- D368 that OBMC brings coding gain without any penalty in complexity for all considered partitioning sets. Therefore this tool is included in all considered tested tools.

Two related techniques are considered:

- Asymmetric Motion Partitioning (AMP) with Overlapped Block Motion Compensation (OBMC);

- Non-rectangular Motion Partitioning (NRMP) with OBMC;

## Asymmetric Motion Partitioning with OBMC

The related JCTVC contributions are:

- JCTVC-D367 [8], by Samsung & Qualcomm

- JCTVC-D233 [11], by NTT DOCOMO, Inc

Asymmetric motion partition (AMP) was initially proposed in JCTVC-A124 [2], and fast AMP method was proposed in JCTVC-D367 [8]. The tool consists of partitioning a square block into two non-symmetric horizontal or vertical rectangles shapes, as illustrated in Figure 1 for the 64x64 block size. In the current TMuC implementation (version 0.9), the square block is split using AMP in one rectangle of 3/4 of the square width or height, and another rectangle of 1/4 of the square width or height. AMP is used at CU size from 16x16 to 64x64.

Figure 1: Example of asymmetric motion partitioning for a 64x64 block.

For AMP encoding speed up, 4 additional conditions are checked before doing motion estimation for AMP. If the certain conditions are met, additional motion estimation for AMP can be skipped. Pseudo code for speed up is as followings.

Bool bTestAMP_Hor = false, bTestAMP_Ver = false;

if ( rpcBestCU->getPartitionSize(0) == SIZE_2NxN )

{

bTestAMP_Hor = true;

}

else if ( rpcBestCU->getPartitionSize(0) == SIZE_Nx2N )

{

bTestAMP_Ver = true;

}

else if ( rpcBestCU->getPartitionSize(0) == SIZE_2Nx2N && rpcBestCU->getMergeFlag(0) == false && rpcBestCU->isSkipped(0) == false )

{

bTestAMP_Hor = true;

bTestAMP_Ver = true;

}

//! Utilizing the partition size of parent PU

if ( eParentPartSize >= SIZE_2NxnU && eParentPartSize <= SIZE_nRx2N )

{

bTestAMP_Hor = true;

bTestAMP_Ver = true;

}

if ( eParentPartSize == SIZE_2Nx2N )

{

bTestAMP_Hor = false;

bTestAMP_Ver = false;

}

In the pseudo-code, if the value of bTestAMP_Hor is true, SIZE_2NxnU and SIZE_2NxnD are evaluated. If the value of bTestAMP_Ver is true, SIZE_nLx2N and SIZE_nRx2N are evaluated. Otherwise, no AMP modes will be evaluated. The variable eParentPartSize means selected PU size at lower level in CU quad-tree, which contains current PU.

HM2.0 includes PU partial merging of 2NxN and Nx2N rectangles. In JCTVC-D233, the results of asymmetric partial merging with partitions sizes of 2NxnU, 2NxnD, nLx2N and nRx2N were presented.  This experiment will be based on the HM2.0 partial merging where the first PU inferred to be merged, whilst the second PU can either be merged or has motion vector coding.

In OBMC, the overlapped region in the luma component is defined as 2 rows (columns) of pixels on both sides of the horizontal (vertical) partition boundary. For chroma components, the overlapped region is reduced to 1 row (column) of pixels. The weighting factors are (3/4, 1/4) for the pixels which are 1 row (column) apart from the boundary and (7/8, 1/8) for pixels which are 2 row (column) apart from the boundary.

## Non-Rectangular Motion Partitioning with OBMC

The related JCTVC contributions are:

- JCTVC-D230 [5], by Technicolor & Qualcomm

- JCTVC-D297 [5], by Huawei & HiSilicon.

Non-rectangular motion partitioning (NRMP) is a subset of the method proposed at JCTVC-D230 and JCTVC-D297. NRMP enables to partition a square block into 2 regions with non-rectangular shape. Limited sets of diagonal-only partitions (45 degree or 135 degree partitioning) and non-diagonal partitions will be tested..

Figure 2: Example of 6 diagonal partitioning modes

Figure 3: Example of set of 6 not-only diagonal partitioning modes

Similarly to AMP, to reduce artifacts along partition lines, OBMC is used. The details of OBMC for non-rectangular motion partitioning are available in JCTVC-D230 [5]

## Unified solution

The unified solution AMP+NRMP+OBMC is a combination of these two partitioning tools.

To signal the different partitioning modes efficiently, INTER_2Nx2N is used to signal non-rectangular motion partitioning. An additional flag, inserted after the INTER_2Nx2N signaling, allows select INTER_2Nx2N or INTER_NRMP. If INTER_NRMP is selected, additional bits are inserted to signal the INTER_NRMP mode. INTER_2NxN and INTER_Nx2N are used to signal horizontal asymmetric motion partitioning and vertical asymmetric motion partitioning respectively, as initially done in TMuC0.9.

# Experiments

## Tests description

The goal of the tests of CE2 is to identify the best combination and settings and AMP+NRMP.

The reference anchors will be generated using the HM2.0 with the tests conditions as specified in JCTVC-D600 [9].

The selection will be done in two steps.

- Selection of the optimal NRMP configuration NRMPopt*. The following tests will be made:

  - Turn off AMP and test NRMP with 2 diagonal partitioning

  - Turn off AMP and test NRMP with 6 diagonal partitioning

  - Turn off AMP and test NRMP with 6 not-only diagonal partitioning

- Evaluation of AMP and AMP+ NRMPopt. The following tests will be made:

  - Turn on AMP, turn off NRMP partitioning

  - Turn on AMP and NRMPopt, and test this tools combination

*The selection of the best NRMP version will be made by the unified solution proponents based on the trade-off coding gain/encoding complexity.

The following table summarizes the planed tests.

| Test number | AMP | NRMP | Cross-checker |
| --- | --- | --- | --- |
| 1a | No | 2 diagonal | INRIA |
| 1b | No | 6 diagonal | INRIA |
| 1c | No | 6 not-only diagonal | INRIA |
| 2a | Yes | No | HHI |
| 2b | Yes | Best(1a,1b,1c) | HHI |

## Software

The CE tools are going to be implemented into the HM2.0 software, as recommended by the JCT-VC software group at the end of the 4th JCT-VC meeting.

## Test Sequences, Bit Rates and Coding Conditions

The CE tools will be tested and evaluated under the common official configurations recommended by JCT-VC at the conclusion of the 4th JCT-VC meeting in document JCTVC-D600. In particular the reference anchors will be generated using the recommended configuration files provided by the JCT-VC software group.

Restrictions / modifications for this TE:

- No test with intra-only configuration

- High coding efficiency configuration

## Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures and encoding/decoding run time comparisons as defined in document JCTVC-B312 [10].

Following comparisons will be made:

- Proposal against HM2.0 default configuration

# Time-line

- 11 February 2011	Final CE-description

- 12 February 2011	Start tool integration and optimization

- 25 February 2011	Completion of tool integration and optimization, start of testing

- 2 March 2011	 	Start of cross-checking

- 9 March 2011   	Input Documents upload

# References

- E. Francois, P. Chen, X. Zheng, “Description of Core Experiment 2: Flexible Motion Partitioning,” Doc. JCTVC-C099, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, October 2010

- K.McCann, W.-J.Han, I.-K.Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology”, Doc. JCTVC-A124, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Dresden, Germany, April 2010.

- P.Chen, R. Panchal W.-J. Chien, M.Karczewicz , (Qualcomm), “Overlapped block motion compensation in TMuC”, Doc. JCTVC-C251 (m18292), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.

- L. Guo, P.Chen, R. Panchal W.-J. Chien, M.Karczewicz , (Qualcomm), “CE2: Overlapped Block Motion Compensation for Geometry Partition Block”, Doc. JCTVC-D368 (m19143), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

- Edouard Francois, Philippe Bordes (Technicolor), Liwei Guo, Marta Karczewicz (Qualcomm), “CE2: Simplified Geometry Block Partitioning”, Doc. JCTVC-D230 (m18990), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

- X.Zheng (HiSilicon), H.Yu (Huawei), “TE3.2: Huawei & HiSilicon report on flexible motion partitioning coding”, Doc. JCTVC-C099 (m18122), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.

- P.Chen, L.Guo, M.Karczewicz , (Qualcomm), “Geometry motion partition”, Doc. JCTVC-C301 (m18516), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.

- Min-Su Cheon, Il-Koo Kim (Samsung) , L. Guo, P. Chen, M. Karczewicz (Qualcomm), “CE2: Asymmetric motion partition with overlapped block motion compensation”, Doc. JCTVC-D367 (m19142), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-D600 (m19497), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

- K.McCann, “Tool Experiment 12: Evaluation of TMuC Tools”, Doc. JCTVC-B312, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, July 2010.

- Yoshinori Suzuki, TK Tan, “CE9: 3.1 PU Merge & Skip tools and proposed improvements”, Doc. JCTVC-D233, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011