| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 19-28 January, 2011 | Document: JCTVC-C603r2 WG11 Number: m19492 |
| --- | --- |

| Title: | CE3: Interpolation for MC (Luma) |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by JCT-VC |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Elena Alshina | Email: | takeshi.chujoh@toshiba.co.jp elena_a.alshina@samsung.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

The goal of this Core Experiment (CE) is to further investigate interpolation filtering for motion compensation (Luma) technology. Concerning the interpolation filters for motion compensation (Luma) techniques are evaluated according to BD-rate gain, complexity and subjective picture quality.

# Participants

Coordinator: Takeshi Chujoh (TOSHIBA Corporation) and Elena Alshina (Samsung)

| No. | Affiliation | Contact | Email | Remark |
| --- | --- | --- | --- | --- |
| 1 | eBrisk Video | Faouzi Kossentini | faouzi@ebriskvideo.com | Proponent |
| 2 | HHI | Haricharan Lakshman Heiko Schwarz | haricharan.lakshman@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de | Proponent |
| 3 | KDDI | Tomonobu Yoshino | to-yoshino@kddilabs.jp | Proponent |
| 4 | Motorola Mobility | Krit Panusopone Jian Lou | krit@motorola.com lou@motorola.com | Proponent |
| 5 | NTT | Shohei Matsuo | jctvc-te@lab.ntt.co.jp | Proponent |
| 6 | Qualcomm | In Suk Chong Liwei Guo | ichong@qualcomm.com liweig@qualcomm.com | Proponent |
| 7 | Sony | Teruhiko Suzuki Kenji Kondo | teruhikos@jp.sony.com kenji.kondo@jp.sony.com | Proponent |
| 8 | TOSHIBA | Takeshi Chujoh Kazuyo Kanou | takeshi.chujoh@toshiba.co.jp kazuyo.kanou@toshiba.co.jp | Proponent |
| 9 | Nokia | Kemal Ugur | Kemal.Ugur@nokia.com | Cross-checker |
| 10 | Panasonic | Semih Esenlik | semih.esenlik@eu.panasonic.com | Cross-checker |
| 11 | Samsung | Elena Alshina Jianle Chen | elena_a.alshina@samsung.com / elena.alshina@gmail.com jianle.chen@samsung.com | Cross-checker |

# Functionality addressed

- Improvement of coding efficiency by interpolation filter

- Reduction of complexity and reference frame memory access bandwidth generally

# Description of tool

Details are provided in JCTVC-D397, JCTVC-A116, JCTVC-D056, JCTVC-D285, JCTVC-D246, JCTVC-D150, JCTVC-A121, JCTVC-D376, JCTVC-D090 and JCTVC-D154.

# JCTVC-D397 (eBrisk Video) [1]

JCTVC-D397 proposes an Adaptive Interpolation Filtering (AIF) technique for interpolation filtering of video sequences. The proposed technique allows the encoder to switch, for each group of positions, between a fixed interpolation filter specified in HM 2.0 and a newly-generated AIF filter of the same length and precision. The new AIF filter can generated on-line, or it can be determined off-line for a specific application space.

# JCTVC-A116 and JCTVC-D056 (HHI) [2][3]

The method of interpolation using MOMS basis was proposed in JCTVC-A116 and participated in CE3 as JCTVC-D056. In a generalized case, it can be implemented as a combination of an IIR pre-filter and an FIR interpolation filter. This contribution will compare the proposed interpolation scheme with HM2.0 reference scheme in terms of RD performance, complexity and memory bandwidth. An implementation using no IIR pre-filtering will also be provided.

# JCTVC-D285 (KDDI) [4]

KDDI proposes a switching interpolation filter scheme in JCTVC-D285 [4]. This scheme employs separable 8-tap interpolation filter, and up to 8 candidates of filter coefficients are pre-defined. The best interpolation filter among the candidates is decided based on a decision algorithm, which estimates MC predicted error in the current frame and predicts desired interpolation filter characteristics by utilizing the estimated MC prediction error.

# JCTVC-D246 (Motorola Mobility) [5]

JCTVC-D246 proposes the new DCT-based interpolation filters. Though similar ideas are used for filter coefficient derivation, the final integer representations of the filter coefficients are quite different to those of the HM2.0 filters. 6 bit precision filters wil also be provided considering the complexity reduction.

# JCTVC-D150 (NTT) [6]

JCTVC-D150 proposes a region-based adaptive interpolation filter as an extension of AIF. In the proposed method, a frame is segmented into multiple regions, and filter coefficients for each region are derived. The basic idea is that filter coefficients are designed for each region in a frame when the frame consists of multiple regions with different characteristics.

# JCTVC-A121 and JCTVC-D376 (Qualcomm) [7][8]

Qualcomm proposes interpolation filters with different tap sizes, based on the fractional pixel locations as in [8]. The basic idea is to use longer filters for fractional pixel positions that require 1-D filtering and shorter filters for positions that require 2-D filtering.

# JCTVC-D090 (Sony) [9]

The detail description of Bi/Single (BiS) switched interpolation filter is described in JCTVC-D090 [8]. The BiS interpolation filter works as separable 2D filter as same as Samsung’s DCT based interpolation filter (DCTIF). Additionally, the BiS interpolation filter has one function. In order to improve quality of prediction in MC, two pre-defined filter coefficients sets are switched either inter mode is bi-prediction or not. For example, if current MC block mode is bi-prediction, the filter coefficient for bi-prediction is used as shown in [7]. In the table, current filter coefficients are designed as 8 bits. Regarding to precision, in Daegu meeting, it is adopted that interpolation filter uses 6 bit precision DCTIF 8 tap for luma. So, also BiS interpolation filter will support 6 bit precision to equal condition.

# JCTVC-D154 (TOSHIBA) [10]

JCTVC-D154 is an interpolation method that has non-uniform tap length filters for computational complexity reduction. In general, filter with asymmetric coefficients requires a larger number of multiplication operations compared to filter with symmetric coefficients supposing that they are the same tap length N. Then, the worst-case number of operations is able to be reduced by using shorter tap length (M<=N) of filter with coefficients in some cases. For example, in case of inner fractional position, asymmetrical M-tap (M<=N) and symmetrical N-tap filter are used and in case of border fractional position, both asymmetrical and symmetrical N-tap filters are used.

| Proponents | Cross-checkers |
| --- | --- |
| eBrisk Video | Panasonic |
| HHI | eBrisk Video, Samsung |
| KDDI | NTT |
| Motorola Mobility | Samsung |
| NTT | KDDI |
| Qualcomm | TOSHIBA |
| Sony | Nokia, Qualcomm, Samsung |
| TOSHIBA | Motorola Mobility, Sony |

# Expected gains

- Improvement of coding efficiency

- Reduction of complexity and reference frame memory access bandwidth generally

# Core experiments conditions

# Software

HM version 2.0 (version Feb. 11 2011)

# Coding conditions

High coding efficiency anchor:

Random access configuration

Low delay configuration

Low complexity anchor:

Random access configuration

Low delay configuration

Common test condition is specified in JCTVC-600 [11].

# Evaluation criteria

- Measure impact on bitrate/PSNR using provided data. Use 4-point BD-PSNR and BD-Rate [12].

- Complexity (encoding/decoding times, and use a common Excel sheet that counts number of operations instruction cycles and memory bandwidth compared to another)

# Time lines

| Feb.11 2011: | Deadline for sending email to coordinator expressing interest in participating in tool experiment. |
| --- | --- |
| Feb.11 2011: | Deadline of formal description of all proposals. |
| Feb.11 2011: | Distribution of document of Tool experiment. |
| Mar. 3 2011: | Test results representation form announcement and start to cross-check of experimental results and check subjective quality |
| Mar. 9 2011: | Complete cross-checking of experimental results and checking subjective quality |
| Mar.10 2011: | Upload contributions |

# References

[1] F. Kossentini, N. Mahdi and M. A. Ben Ayed (eBrisk Video), “A New Adaptive Interpolation Filtering Technique,” JCTVC-D397, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[2] M. Winken, S. Boße, B. Bross, P. Helle, T. Hinz, H. Kirchhoffer, H. Lakshman, D. Marpe, S. Oudin, M. Preiß, H. Schwarz, M. Siekmann, K. Sühring and T. Wiegand(HHI) , “Description of video coding technology proposal by Fraunhofer HHI,” JCTVC-A116, 1st. JCT-VC Meeting, Dresden, DE, 15-23 April, 2010.

[3] H. Lakshman, H. Schwarz, D. Marpe and T. Wiegand (HHI), “CE3: Luma Interpolation using MOMS,” JCTVC-D056, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[4] T. Yoshino and S. Naito (KDDI), "Switching Interpolation Filter (SIF) scheme with low-complexity decision algorithm," JCTVC-D285, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[5] J. Lou and L. Wang (Motorola Mobility), “New DCT-based interpolation filters,” JCTVC-D246, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[6] S. Matsuo, Y. Bandoh, S. Takamura and H. Jozawa (NTT), “Response to CE3: Region-based Adaptive Interpolation Filter,” JCTVC-D150, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[7] M. Karczewicz, P. Chen, R. Joshi, X. Wang, W.-J. Chien and R. Panchal (Qualcomm), “Video coding technology proposal by Qualcomm Inc.,” JCTVC-A121, 1st. JCT-VC Meeting, Dresden, DE, 15-23 April, 2010.

[8] I.S. Chong, L. Guo and M. Karczewicz , “CE3: Qualcomm’s proposal for luma interpolation filters,” JCTVC-D376, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[9] K. Kondo and T, Suzuki (Sony), “CE3 : Results on Bi/Single MC interpolation filter,” JCTVC-D090, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[10] T. Chujoh, K. Kazuyo and T. Yamakage (TOSHIBA), “CE3: Non-uniform Tap Length Filtering,” JCTVC-D154, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[11] F. Bossen, “Common TM Test Conditions,” JCTVC-D600, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[12] G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves,” VCEG-M33, 13th VCEG Meeting, Austin, USA, April 2001.