| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D604 WG11 Number: m19504 |
| --- | --- |

| Title: | Description of Core Experiment 4: Slice Boundary Processing and Fine Granularity |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Yu-Wen Huang1 Il-Koo Kim2 | Email: | yuwen.huang@mediatek.com ilkoo.kim@samsung.com |
| Source: | MediaTek1, Samsung2 |  |  |

- Introduction

Several input documents proposed to the Daegu meeting are related to slices. They can be divided into three parts. The first part is on slice granularities, the second part is on slice boundary processing, and the third part is on slice common information sharing by picture layer syntax (e.g. picture parameter set). This core experiment (CE) addresses these topics and evaluates the performance of different slice granularities, different slice boundary processing tools, and slice common information sharing by using HM2.0 as the software basis.

# Participants List

| Company | Name | Email Address |
| --- | --- | --- |
| MediaTek | Yu-Wen Huang | yuwen.huang@mediatek.com |
| MediaTek | Chih-Wei Hsu | cw.hsu@mediatek.com |
| MediaTek | Chia-Yang Tsai | chiayang.tsai@mediatek.com |
| MediaTek | Shawmin Lei | shawmin.lei@mediatek.com |
| Fujitsu | Kimihiko Kazui | kazui.kimihiko@jp.fujitsu.com |
| Ericsson | Rickard Sjöberg | rickard.sjoberg@ericsson.com |
| Huawei | Qiu Shen | shenqiu@huawei.com |
| Huawei | Yongbing Lin | yblin@huawei.com |
| Huawei | Changcai Lai | laichangcai@huawei.com |
| Huawei | Jianhua Zheng | zhengjianhua@huawei.com |
| Huawei | Lingzhi Liu | lzliu@huawei.com |
| Huawei | Haoping Yu | haopingyu@huawei.com |
| Samsung | Il-Koo Kim | ilkoo.kim@samsung.com |
| Qualcomm | Peisong Chen | peisongc@qualcomm.com |
| Canon | Masato Shima | shima.masato@canon.co.jp |
| Sharp | Andrew Segall | asegall@sharplabs.com |
| Sharp | Kiran Misra | misrak@sharplabs.com |

# Tools under Test

## Subset1: Slice granularity

The related JCTVC contributions are:

- JCTVC-D127 Error: Reference source not found, by MediaTek

- JCTVC-D312 Error: Reference source not found, by Huawei

- JCTVC-D396 [5], by Qualcomm

- JCTVC-D383 [6], by Docomo

- JCTVC-C154 [7], by Ericsson

### JCTVC-D127 [C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, C.-Y. Chen, C.-M. Fu, and S. Lei (MediaTek)] Leaf CU Aligned Slices

In current HEVC WD, one picture can be partitioned into multiple LCU-aligned slices, so each slice contains an integer number of LCUs as shown in Figure 1(a). The LCU size can be 64x64, which is 16 times of a macroblock in prior video coding standards. Using 64x64 LCUs as basic units of a slice may not be able to provide enough flexibility for rate control. Hence, leaf-CU-aligned slices allow slice to contain a fractional number of LCUs and slice boundaries are aligned with leaf CU boundaries instead of LCU boundaries as shown in Figure 1(b). The combinations of different slice granularities will be tested for target bytes 1500 including different LCU sizes (e.g. 16x16 to 64x64) with both LCU-aligned and leaf-CU-aligned properties. Moreover, removing end_of_slice_flag as described in JCTVC-D383 will also be implemented for test.

- (b)

- (a) Example of the LCU-aligned slices (b) Example of the Leaf-CU-aligned slices

### JCTVC-D312 [Q. Shen, Q. Xie, and H. Yu (Huawei)] Fine Granularity Slice Partition

The slice partition scheme proposed in this contribution includes following points, 1) leaf-CU-aligned slice partitioning, 2) hierarchical signaling of the slice start address in the slice header, 3) hierarchical signaling of the slice end address in the slice header, 4) split flag derivation for CUs in the slice boundaries. The slice partition scheme described above will be evaluated in this CE.

### JCTVC-C154 [R. Sjöberg and P. Wennersten (Ericsson)] Fine Granularity Slices

Fine granularity slices were introduced. The HM specification currently supports slice boundaries on largest coding unit (LCU) resolution. The current common conditions specify the LCU size to be 64x64 which is 16 times larger than traditional 16x16 macroblocks. The problem with such coarse slice boundaries is that it becomes difficult to control the size of slices in terms of bytes. For highly active and hard-to-compress areas, one single LCU may be larger than a target number of slice bytes. By fine granularity slices, the restriction of aligning the slice boundaries to LCU boundaries is removed. The slice address can be signaled using fixed-length coding.

The following test should be done:

Test A: Using LCU and SCU sizes as recommended in JCTVC-D600, slice granularities of sizes ranging from the smallest CU size to the largest CU size should be compared where no slice is allowed to exceed 1500 bytes. Cases in which the slice granularity could not achieve the 1500 byte limit (e.g. a single slice granularity sized block exceeds the 1500 byte limit) should be registered. The result from the test should be in the form of BDR measurements using the common test conditions in JCTVC-D600 together with statistics for slices exceeding the 1500 byte limits. An optional extension of this test is to include packetization overhead during bitrate calculations.

### Crosscheck

There are three proponents in the slice granularity category. The first two are the two-step addressing method in JCTVC-D127 (MediaTek) and JCTVC-D312 (Huawei), and the last one is the one-step addressing method in JCTVC-C154 (Ericsson). The evaluation criterion is BD-rate with JCTVC-D600 anchor used as reference.

| Case | Software | Condition | Crosschecker |
| --- | --- | --- | --- |
| 1 | HM2.1 | LCU=64x64, LCU-aligned, 1500 bytes per slice | MediaTek |
| 2 | HM2.1 | LCU=32x32, LCU-aligned, 1500 bytes per slice | MediaTek |
| 3 | HM2.1 | LCU=16x16, LCU-aligned, 1500 bytes per slice | MediaTek |
| 4 | MediaTek based on HM2.0 | LCU=64x64, leaf-CU-aligned, 1500 bytes per slice, 2-step addressing, use end_of_slice | Qualcomm |
| 5 | MediaTek based on HM2.0 | LCU=64x64, leaf-CU-aligned, 1500 bytes per slice, 2-step addressing, remove end_of_slice | Qualcomm |
| 6 | Huawei based on HM2.0 | LCU=64x64, leaf-CU-aligned, 1500 bytes per slice, 2-step addressing, use end_of_slice | Canon |
| 7 | Huawei based on HM2.0 | LCU=64x64, leaf-CU-aligned, 1500 bytes per slice, 2-step addressing, remove end_of_slice | Canon |
| 8 | Ericsson based on HM2.0 | LCU=64x64, leaf-CU-aligned, 1500 bytes per slice, 1-step addressing, use end_of_slice | Huawei |
| 9 | Ericsson based on HM2.0 | LCU=64x64, leaf-CU-aligned, 1500 bytes per slice, 1-step addressing, remove end_of_slice | Huawei |

In the above cases, only 3 LC configurations in JCTVC-D600 are required for case 5 and case 7, and all 6 configurations in JCTVC-D600 are required for rest cases. Please note that for cases 1-3, if HM2.1 cannot be released by Mar. 1, MediaTek’s slice implementation that can support both LCU-aligned and leaf-CU-aligned slices and is based on HM2.0 will be used for experiments.

## Slice boundary processing

The related JCTVC contributions are:

- JCTVC-D128 Error: Reference source not found, by MediaTek

- JCTVC-D302 [8], by Huawei

### JCTVC-D128 [C.-Y. Tsai, C.-W. Hsu, Y.-W. Huang, C.-Y. Chen, C.-M. Fu, and S. Lei (MediaTek)] Slice Boundary Processing and Picture Layer Raw Byte Sequence Payload

When slice-independent deblocking filter (DF) and slice-independent adaptive loop filter (ALF) are used, each slice in a picture can be independently decoded without using any data from other slices. There might be coding efficiency loss due to the independent nature of slices. Subjective artifacts across slice boundaries can be significant. A slice boundary filter (SBF), which is very similar to DF but only processes slice boundaries, is used to remove possible artifacts across slice boundaries. Experiments will be conducted to show the improvements on reducing boundary artifacts at slice boundaries.

### JCTVC-D302 [Y. Lin, C. Lai, J. Zheng, and L. Liu (HiSilicon)] Intra Coding Improvements for Slice Boundary Blocks

This experiment will be conducted to show the improvements of the coding efficiency for the intra prediction coding on boundary blocks in a slice. In this experiment, two methods will be tested. (1) the efficient intra mode coding method in the slice boundary. For boundary blocks of a slice, the number of candidate intra prediction modes can be reduced according to the availability of the neighbor reference blocks. When the number of the candidate intra prediction modes is reduced to half, one bit can be saved. (2) Padding process for slice boundary blocks. For slice boundary block, the reference pixels located in its unavailable neighbor reference block are padded with the pixel from its available neighbor reference block, instead of being padded with a fixed DC value.

### Crosscheck

There are two proponents in the slice granularity category. One is the slice boundary filter in JCTVC-D128 (MediaTek), and the other is the intra coding improvements in JCTVC-D302 (Huawei). For MediaTek’s proposal, the evaluation criteria are BD-rate with JCTVC-D600 anchor used as reference and visual quality at slice boundaries. For Huawei’s proposal, the evaluation criterion is BD-rate with JCTVC-D600 anchor used as reference.

| Case | Software | Condition | Crosschecker |
| --- | --- | --- | --- |
| 1 | MediaTek based on HM2.1 or HM2.0 | LCU=64x64, LCU-aligned, 1500 bytes per slice, slice-independent DF and ALF, slice boundary filter off | Samsung |
| 2 | MediaTek based on HM2.1 or HM2.0 | LCU=64x64, LCU-aligned, 1500 bytes per slice, slice-independent DF and ALF, slice boundary filter on | Samsung |
| 3 | MediaTek based on HM2.1 or HM2.0 | LCU=64x64, LCU-aligned, two slices per picture, slice-independent DF and ALF, slice boundary filter off | Qualcomm |
| 4 | MediaTek based on HM2.1 or HM2.0 | LCU=64x64, LCU-aligned, two slices per picture, slice-independent DF and ALF, slice boundary filter on | Qualcomm |
| 5 | Huawei based on HM2.1 or HM2.0 | LCU=64x64, LCU-aligned, 1500 bytes per slice, slice-dependent DF and ALF, intra coding improvements off | Canon |
| 6 | Huawei based on HM2.1 or HM2.0 | LCU=64x64, LCU-aligned, 1500 bytes per slice, slice-dependent DF and ALF, intra coding improvements on | Canon |

All 6 configurations in JCTVC-D600 are required for each case. Please note that for cases 1-6, if HM2.1 cannot be released by Feb. 21, MediaTek’s and Huawei’s slice implementations that are based on HM2.0, as shown in Section 3.1.4, will be used for experiments.

## Slice common information sharing

The related JCTVC contributions are:

- JCTVC-D128 Error: Reference source not found, by MediaTek

### JCTVC-D128 [C.-Y. Tsai, C.-W. Hsu, Y.-W. Huang, C.-Y. Chen, C.-M. Fu, and S. Lei (MediaTek)] Slice Boundary Processing and Picture Layer Raw Byte Sequence Payload

There is redundant information among multiple slice headers in a picture. As the number of slices increases, better gain is expected to reduce the redundancy. Picture layer raw byte sequence payload (RBSP) or picture parameter set (PPS) can be used to remove redundancy of syntax elements that have identical values in multiple slices in a picture. Experiments will be conducted to move syntax elements from the slice header to the picture layer RBSP or picture parameter set (PPS) when they are the same for all slices include slice type, slice quantization parameter, entropy coding mode, interpolation filter type, number of reference pictures, ALF coefficients, …etc..

### Crosscheck

The one proponent in the slice common information sharing category is JCTVC-D128 (MediaTek). The evaluation criterion is BD-rate with JCTVC-D600 anchor used as reference.

| Case | Software | Condition | Crosschecker |
| --- | --- | --- | --- |
| 1 | MediaTek based on HM2.1 or HM2.0 | LCU=64x64, LCU-aligned, 1500 bytes per slice, slice common information sharing off | Fujitsu |
| 2 | MediaTek based on HM2.1 or HM2.0 | LCU=64x64, LCU-aligned, 1500 bytes per slice, slice common information sharing on | Fujitsu |

All 6 configurations in JCTVC-D600 are required for each case. Please note that for cases 1-2, if HM2.1 cannot be released by Feb. 21, MediaTek’s slice implementations that are based on HM2.0, as shown in Section 3.1.4, will be used for experiments.

# Experimental Conditions

## Software

The CE tools are going to be implemented into the HM2.0/HM2.1 software.

## Test Sequences, Bit Rates and Coding Conditions

The CE tools will be tested and evaluated under the common official configurations recommended by JCT-VC at the conclusion of the 4th JCT-VC meeting in document JCTVC-D600 Error: Reference source not found. In particular the reference anchors will be generated using the recommended configuration files provided by the JCT-VC software group.

## Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures.

# Time-line

T1:	2011-Feb-11:	Final CE description and upload

T2: 2011-Mar-1: 	Start cross-checking, proponents provide software and results to cross-checker(s)

T3:	2011-Mar-6:	Report proponents’ results to coordinators

T4:	2011-Mar-9:	Report cross-checkers’ results to coordinators

T5:	2011-Mar-10:	Input document uploaded

# References

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D600, Jan. 2011.

- C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, C.-Y. Chen, C.-M. Fu, and S. Lei, “Leaf CU Aligned Slices,” Document of Joint Collaborative Team on Video Coding, JCTVC-D127, Jan. 2011.

- C.-Y. Tsai, C.-W. Hsu, Y.-W. Huang, C.-Y. Chen, C.-M. Fu, and S. Lei, “Slice Boundary Processing and Picture Layer Raw Byte Sequence Payload,” Document of Joint Collaborative Team on Video Coding, JCTVC-D128, Jan. 2011.

- Q. Shen, Q. Xie, and H. Yu, “Fine Granularity Slice Partition,” Document of Joint Collaborative Team on Video Coding, JCTVC-D312, Jan. 2011.

- Y. Chen, P. Chen, and M. Karczewicz, “On Slice Granularity,” Document of Joint Collaborative Team on Video Coding, JCTVC-D312, Jan. 2011.

- F. Bossen, “Simplification of End-of-Slice Coding for Arithmetic Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-D383, Jan. 2011.

- Rickard Sjöberg and Per Wennersten, “Fine Granularity Slices,” Document of Joint Collaborative Team on Video Coding, JCTVC-C154, Oct. 2010.

- Y. Lin, C. Lai, J. Zheng, and L. Liu, “Intra Coding Improvements for Slice Boundary Blocks,” Document of Joint Collaborative Team on Video Coding, JCTVC-D302, Jan. 2011.