| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D607_r6 WG11 Number: m19494 |
| --- | --- |

| Title: | Description of Core Experiment 7: Alternative Transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories Chuohao Yeo Institute for Infocomm Research Rajan Joshi Qualcomm Felix Fernandes Samsung Electronics Co., Ltd. | Email: | cohen@merl.com chyeo@i2r.a-star.edu.sg rajanj@qualcomm.com felix.f@sta.samsung.com |
| Source: | Mitsubishi Electric Research Laboratories, Institute for Infocomm Research, Qualcomm, Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document defines Core Experiment 7 (CE7) on Alternative Transforms. The purpose of this CE is to further investigate the tools related to CE7, as discussed during the January 2011 JCT-VC meeting in Daegu. The conclusions from that CE were that further investigation is needed to evaluate the compression efficiency benefits vs. added complexity of these alternative transforms. Encoding and decoding run times are not sufficient for evaluating complexity, so the CE defined in this document includes additional metrics related to the complexity of the alternative transforms and any additional complexity required by these transforms, such as additional quantization matrices, storage, scan patterns, etc. The list of tools and experiments to be performed are detailed in this document.

# Introduction

For the January 2011 JCT-VC meeting, Core Experiment 7 (CE7) investigated the performance of several proposed alternative transforms for coding Intra slices [1]. These transforms yielded some improvements in compression efficiency, while increasing complexity to some extent. In addition to changing the transform bases, these proposals included various combinations of changes related to coefficient scan orders, quantization matrices, how the transforms are implemented, etc. It was recommended that a new CE be defined to investigate the complexity vs. compression benefit of these alternative transforms, with an emphasis on clarifying the complexity increases with more detail than specifying only encoder and decoder runtimes. In addition, some proposals applied related alternative transforms to Inter slices.

This document defines a new Core Experiment 7 on Alternative Transforms, which will be performed between now and the upcoming March 2011 JCT-VC meeting.

# Participant list

| Name | Affiliation | Email |
| --- | --- | --- |
| Robert Cohen (coordinator) | Mitsubishi Electric Research Labs | cohen@merl.com |
| Chuohao Yeo (coordinator) | Institute for Infocomm Research | chyeo@i2r.a-star.edu.sg |
| Rajan Joshi (coordinator) | Qualcomm | rajanj@qualcomm.com |
| Felix Fernandes (coordinator) | Samsung | felix.f@sta.samsung.com |
| Akiyuki Tanizawa | Toshiba | akiyuki.tanizawa@toshiba.co.jp |
| Jun Yamaguchi | Toshiba | jun2.yamaguchi@toshiba.co.jp |
| Elena Alshina | Samsung | elena_a.alshina@samsung.com |
| Ankur Saxena | Samsung | asaxena@sta.samsung.com |
| Il-Koo Kim | Samsung | ilkoo.kim@samsung.com |
| Haitao Yang | Huawei | haitaoyang@huawei.com |
| Jin Song | Huawei | songjinmail@huawei.com |
| Haoping Yu | Huawei | haopingyu@huawei.com |
| Xin Zhao | Peking Univ. | xzhao@jdl.ac.cn |
| Li Zhang | Peking Univ. | zhanglili@jdl.ac.cn |
| Siwei Ma | Peking Univ. | swma@jdl.ac.cn |
| Youji Shibahara | Panasonic | shibahara.youji@jp.panasonic.com |
| Takahiro Nishi | Panasonic | nishi.takahiro@jp.panasonic.com |
| Atsuro Ichigaya | NHK | ichigaya.a-go@nhk.or.jp |
| Shinichi Sakaida | NHK | sakaida.s-gq@nhk.or.jp |
| Feng Zou | HKUST | fengzou@ust.hk |
| Xing Wen | HKUST | wxxab@ust.hk |
| Marta Mrak | BBC | Marta.Mrak@bbc.co.uk |

# Description of tools

The following tools shall be evaluated for CE 7:

| Tool # | Proponent(s) | Related Docs. | Tool Abstract | Cross-checkers |
| --- | --- | --- | --- | --- |
| 1 | Qualcomm Toshiba I2R Huawei | JCTVC-D392 | D392 proposed a mode dependent transform and coefficient scanning for coding of intra prediction residuals. The transform is a combination of two 1-D transforms, which are DCT and DST. The choice of the transform is mode dependent without need for any additional R-D. But the 8x8 DST does not have a fast implementation. In this core experiment, alternative transforms that have faster implementations or reuse part of the core transform shall be explored. Additional results based on mode dependent scans will also be presented. | 1. BBC 2. Samsung 3. Panasonic |
| 2 | Samsung | JCTVC-D033 | The proposed tool applies the conventional Discrete Cosine Transform (DCT), or DST (separably along the vertical or horizontal directions) based on the intra prediction direction. The proposed DCT/DST transform does not require any additional signaling information or Rate-Distortion search during encoding, and works in a single-pass. No training was required to derive the DST transform. The compression performance and complexity of the proposed tool will also be studied. | 1. I2R |
| 3 | Peking Univ. | JCTVC-D284 | The proposed tools apply mode-dependent reordering, transform and coefficient scanning for directional intra prediction and coding in HM. For mode-dependent transform, a single transform matrix is employed for all the directional intra prediction modes defined in HM, and the original DCT method is maintained for DC prediction residue. Moreover, for 4x4 transform, the applied transform is a simplified 4x4 DST featured by multiplierless arithmetic and low dynamic range of internal results. | Withdrawn |
| 4 | Samsung | JCTVC-D357 JCTVC-D360 JCTVC-C096 | The Rotational Transform (ROT) is an adaptive transform for efficient residual coding (especially for Intra residuals). From JCTVC-D357, the encoder/decoder complexity was increased when ROT was used in the Intra scenario. A lifting implementation will address this complexity issue and guarantee perfect ROT invertibility (JCTVC-C096). ROT with lifting implementation will be tested in this CE. Additionally, JCTVC-D360 proposed a scan modification. The synergy between the ROT and an Intra-prediction, mode-dependent scan will be studied in this CE and reported as supplementary results. | 1. Qualcomm 2. Panasonic |
| 5 | Panasonic | JCTVC-D151 | This contribution proposes a unification of MDDT and ROT, in which mode dependent 1-step transform is used for 4x4 transform block and mode dependent 2-step transforms are used for larger ones. The same 4x4 mode dependent KLT as MDDT is used for 4x4 transform block. In JCTVC-D151, the first of the 2-step transforms was either AVC transform or Chen-DCT in the TMuC0.9-HM, and the second one was 4x4 KLT which was partially applied to the lower frequency components. Application of the second transform depends on the intra prediction mode of the corresponding prediction unit. | 1. Toshiba 2. Samsung |
| 6 | NHK | JCTVC-D182 | The DCT (Discrete Cosine Transform) and DST (Discrete Sine Transform) type II are used adaptively in this method. The technique in the method shares the same core transform matrices between DCT and DST by changing the sign of alternate inputs to the DCT in order to implement the DST. This method does not affect the complexity of a decoder and can also adopt fast mode decision algorithm to reduce the encoding complexity. | 1. MERL |
| 7 | HKUST | JCTVC-D276 | This contribution proposed a rate distortion optimized transform scheme for intra block transform coding in HM. The transform matrices are trained offline that can minimize the total RD cost. The trade-off between efficiency and complexity of the rate-distortion optimized transform will be investigated further. | 1. ? |

# Core Experiment tasks

The objectives for this Core Experiment are to:

- Specify the software platform, common software configurations and common test conditions for these experiments;

- Evaluate compression efficiency and complexity of the alternative transforms studied in this CE;

- Provide informal comments regarding subjective differences, if any.

- Investigate interactions between alternative transforms and coefficient scanning schemes;

- Report the results and conclusions of these experiments.

# Test conditions

The following test conditions shall be used for this CE:

## Software

HM version 2.0 shall be used for these experiments, as specified in JCTVC-D600 [2].

## Test configurations

This CE shall use the same encoder and decoder configurations as described in JCTVC-D600. Experiments shall be run for the Intra High-Efficiency, Intra Low-Complexity, Random Access High-Efficiency, and Random Access Low-Complexity coding configurations. Alternative transforms that operate on Inter slices shall also use the Low-Delay configurations.

## Test sequences

The test sequences as described in JCTVC-D600 shall be used for these tests.

## Training of transforms (if applicable)

The test sequences used from Section 5.3 shall not be used for training any proposed alternative transforms. Proponents shall report which sequences they use for training of their transforms, if applicable.

# Evaluation Criteria

Results for both compression efficiency and complexity evaluation shall be entered into the accompanying spreadsheet “JCTVC-D607.xls”. If a proposed transform is applied on top of the core transform (for example, tools 4, 5, and 8), the number of operations of each alternative and core transform shall be reported separately. Operations counts for the core transform will be provided by the CE coordinators in the accompanying spreadsheet. If a proposed transform includes scan orders not already included in HM, simulation results at a minimum shall be reported for the alternative transform without these additional scan orders. Results with the additional scan orders may be reported as well. This is done to help JCT-VC determine the benefits of these additional scans. Proponents are welcome to include additional results at their discretion.

## Compression efficiency

To obtain objective rate-distortion measurements, four point BD-Rate and BD-PSNR metrics [3] shall be used to evaluate coding efficiency. Rates and PSNR values obtained for all test points shall be provided as well. Compression efficiency and encoder/decoder run time results shall also be reported with the common spreadsheet template that is being used for most JCT-VC contributions. The references for these computations shall be generated from the unmodified HM version 2.0 software.

## Complexity evaluation

### Tool description

In the JCT-VC text document associated with each proposed tool, the abstract shall include a brief text description of the proposed alternative transform, along with a brief summary of the compression performance and complexity evaluation.

### Alternative transform description

The alternative transforms for each transform size shall be described in such a way that makes it clear what operations are being performed. Pseudo-code or data-flow diagrams are preferred.

### Arithmetic operations

For each transform size of each alternative transform, the following statistics associated with multiplies, adds, and shifts shall be reported. These counts shall be based upon the pseudo-code or data-flow diagrams.

- Average number of arithmetic operations per block

- Average number of arithmetic operations per pixel

- Worst-case number of arithmetic operations per block

- Worst-case number of arithmetic operations per pixel

- Table showing: Mults, Adds, Shifts, Total per block and per pixel, for 4x4, 8x8, …

### Bit precision

Report the maximum bit precision needed for each transform size.

### Memory requirements

For each transform size, report any additional memory requirements in bytes.

### Quantization

Report the presence and number of additional quantization matrices, for each transform size.

Any additional bit-depth required by use of these additional quantization matrices shall also be reported.

### Scan orders

The number of additional scan orders shall be reported for each transform size.

The Intra prediction mode mapping table, if applicable, shall be reported for each transform size.

Report any additional complexity related to adaptive scanning that is introduced by the alternative transform.

### R-D optimization

If any additional rate-distortion loops or iterations are being performed in the encoder or decoder, their presence shall be indicated in the spreadsheet as “yes” or “no”. The reasons for doing this is that some participants may not be able to report encode or decode runtimes due to variations in computing platforms.

### Encoder/Decoder run times

Encoder and decoder run times shall be reported as described above.

### Other statistics

If present, memory transfers, reordering, and other data operations not included above shall be reported.

## Subjective effects

If sequences coded using a proposed alternative transform exhibit subjective improvements or degradations, proponents and/or cross-verifiers shall describe these subjective differences. These descriptions are to be considered as informal comments, as formal subjective testing is not required by CE7 at this time.

# Timeline

February 11, 2011: HM 2.0 software expected to be available

February 11, 2011: Final CE7 description uploaded

March 3, 2011: Cross-verification begins by this date

March 9, 2011: Proponents report results to CE7 coordinators

March 10, 2011: Input documents uploaded

# References

- R. Cohen, C. Yeo, R. Joshi, “CE7: Alternative Transforms – Summary Report,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D042, 4th JCT-VC Meeting, Daegu, Korea, Jan., 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C600, 4th JCT-VC Meeting, Daegu, Korea, Jan. 2011.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, VCEG-M33, 13th VCEG Meeting, Austin, USA, April 2001.