| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D608_r1 WG11 Number: m19503 |
| --- | --- |

| Title: | Description of CE8: Non-deblocking loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 (Primary Coordinator) Yu-Wen Huang2 In Suk Chong3 (Coordinator) Tomohiro Ikai4 Matthias Narroschke5 (Coordinator) Wang L. Lai6 Ali Tabatabai7 Madhukar Budagavi8 Faouzi Kossentini9 |  | tomoo.yamakage@toshiba.co.jp yuwen.huang@mediatek.com ichong@qualcomm.com ikai.tomohiro@sharp.co.jp Matthias.Narroschke@eu.panasonic.com wlai@sta.samsung.com Ali.Tabatabai@am.sony.com madhukar@ti.com faouzi@ebriskvideo.com |
| Source: | Toshiba1, MediaTek2, Qualcomm3, SHARP4, Panasonic5, Samsung6, Sony7, TI8, eBrisk Video9 |  |  |

_____________________________

# Introduction

The goal of this Core Experiment (CE) is to enhance non-deblocking loop filtering techniques in the HEVC Test Model.  This is not limited to improving coding efficiency, but also reducing complexity and harmonization of the various schemes that are technically feasible based on the understanding that the current situation of JCT-VC activity is in “collaborative phase”.

Techniques are evaluated according to BD-rate gain, complexity and/or subjective picture quality. The following items are categorized as subtests:

- Single- and multi-input schemes are evaluated. i.e. 1-input, 2-inputs and 3-inputs schemes (Toshiba (JCTVC-D192), Sharp (JCTVC-D115), Panasonic (JCTVC-D217)).

- Filter adaptation schemes are evaluated. i.e. Block level filter adaptation (MediaTek/Qualcomm/Toshiba (JCTVC-D119, JCTVC-D119), Sharp (JCTVC-D116), Samsung (JCTVC-D221)).

- Enhancement schemes by combining some other in-loop processing are evaluated (MediaTek/Qualcomm/Toshiba (JCTVC-D119), Samsung (JCTVC-D221)).

- Effectiveness of adaptive loop filtering scheme are evaluated for low complexity conditions (Sony (JCTVC-D270)).

- Complexity reduction and/or quality improvement are evaluated by modifying shapes of filters is evaluated. i.e. Shapes such a diamond, a square, a rectangular, a cross, including symmetry properties and/or different sizes or number of taps, etc. (eBrisk Video (JCTVC-D385), TI (JCTVC-D039), Samsung (JCTVC-D221)).

The software basis for this CE is HM 2.0, and the experiments are conducted based on the test conditions in JCTVC-D600.

# Participants

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 | Chia-Yang Tsai Chih-Ming Fu Ching-Yeh Chen Shawmin Lei Xun Guo Yu-Wen Huang Matthias Narroschke (coordinator) Thomas Wedi Semih Esenlik Marta Karczewicz In Suk Chong (coordinator) Wei-Jung Chien Wang (PoLin) Lai Felix C. Fernandes Tomohiro Ikai Tomoyuki Yamamoto Takeshi Chujoh Tomoo Yamakage (Primary coordinator) Andrey Norkin Faouzi Kossentini Madhukar Budagavi Shun-ichi Sekiguchi Kazuo Sugimoto Ali Tabatabai Ehsan Maani | MediaTek MediaTek MediaTek MediaTek MediaTek MediaTek Panasonic Panasonic Panasonic Qualcomm Qualcomm Qualcomm Samsung Samsung SHARP SHARP Toshiba Toshiba Ericsson eBrisk Video TI Mitsubishi Mitsubishi Sony Sony | chiayang.tsai@mediatek.com chihming.fu@mediatek.com chingyeh.chen@mediatek.com shawmin.lei@mediatek.com xun.guo@mediatek.com yuwen.huang@mediatek.com Matthias.Narroschke@panasonic.de Thomas.Wedi@eu.panasonic.com Semih.Esenlik@eu.panasonic.com martak@qualcomm.com ichong@qualcomm.com wchien@qualcomm.com wlai@sta.samsung.com ffernandes@sta.samsung.com ikai.tomohiro@sharp.co.jp yamamoto.tomoyuki@sharp.co.jp takeshi.chujoh@toshiba.co.jp tomoo.yamakage@toshiba.co.jp andrey.norkin@ericsson.com faouzi@ebriskvideo.com madhukar@ti.com Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp Ali.Tabatabai@am.sony.com Ehsan.Maani@am.sony.com | Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent |

# Experimental Conditions

## Software

HM 2.0 shall be used, which adopts Nx7 filter proposed in JCTVC D039 (TI).  (Will be provided on Feb. 11, 2011.)

## Test Sequences, Bit Rates and Coding Conditions

Follow the recommended sequences, configurations and test cases described in JCTVC-D600. This includes newly added sequences in Class A.

All High Efficiency cases shall be tested.

If a proposal recommends using ALF in low complexity conditions (i.e. subtest 4), the proposal shall report all six test cases.

## Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures and/or complexity.

In subtests 1, 2 and 3, encoding technique shall follow the techniques in HM 2.0 as much as possible (e.g. 16 passes for designing filters). In subtest 1, an adjustment of deblocking parameter may be tested under the efforts of proponents.

In subtest 3, encoding technique should follow the technique in the anchor as much as possible.

If enhancements of encoding technique are further applied, it is encouraged to report results by the enhancement in a separate manner and to describe the possibility of applying the similar concept to the other proposals.

For the complexity measurement, the HM 2.0 software with the anchor configuration and the proposal implemented on the HM 2.0 software should be executed on the same machine, and the computational time will be measured for each test sequence and test case. Relative computational time is calculated against the anchor. Maximum number of vertical taps of filtering is also measured.

# Subtest 1: Number of inputs

## Functionality Addressed

- Identifying an appropriate number of inputs to Wiener-based filter

## Expected Gains

- Better understanding of trade-off between coding efficiency improvement and number of inputs

- Better understanding of parallel processing capability with deblocking filtering

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Number of inputs

- Maximum number of vertical taps for each input

- Parallel processing capability with deblocking filter

Note: In subtest 1, it is only allowed to modify number of inputs from the HM 2.0 anchor. In addition, spatial filtering operation on pre-deblocked pixels instead of post-deblocked pixels is evaluated for parallel processing capability proposed in JCTVC-D115 (Sharp).

No change of chroma processing is allowed for this category.

## Tool Description

### Deblocking parameter adjustment for 1-input ALF (JCTVC-D192, Toshiba)

This evaluates the anchor (QC_ALF), which adopts 1-input (deblocked pixels) for filtering, with different settings of deblocking parameters ( and/or tc). The parameters are fixed for all High Efficiency test cases. Since the text of WD 1 is not consistent with the HM software, some modification of bitstream organization to signal tc offset instead of  offset is needed.

### CE 8: Results for Adaptive loop filter using prediction and residual (3-Input-ALF) (JCTVC-D217, Panasonic)

#### General Concept

The adaptive loop filter using the prediction and residual aims at reducing quantization noise. Based on the prediction signals, the quantized prediction errore', and the deblocked signals'', the following linear filter operations are performed:

s'''=i=1M−2wi⋅si''+wM−1⋅s+wM⋅e'+c

All filter coefficientswi, with i=1,…,M as well as the offset c are estimated once per frame by minimizing the mean squared error between the original input signalsand the filtered signals''':

D=Es−s'''2→min

It is noted that the size of the filters that are designed for prediction and quantized prediction error signals are1×1 in size. In other words, the prediction and quantized prediction error signals are merely weighted during the calculation of the output signal. The estimated filter coefficients are coded and transmitted to the decoder. The filter size M is determined by minimization of the Lagrangian cost of overall data rate R and mean squared errorD:

R⋅λ+D→min

#### Bitstream organization

Additional information transmitted within the bitstream: 
Coefficient information to filter the prediction signal and the quantized prediction error signal.

### DF-combined adaptive loop filter (JCTVC-D115, Sharp)

Sharp’s 2 inputs system is evaluated using plural of post-deblocked pixels on HM design.

In addition, Parallel processing system, which use only one or so small pixels for post-deblock pixels, will be also evaluated.

#### Bitstream organization

The signal to indicate the number of inputs is added.

### Participants

| Participant | Contact |
| --- | --- |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| Panasonic | Matthias Narroschke Matthias.Narroschke@panasonic.com |
| Qualcomm | In Suk Chong ichong@qualcomm.com |
| SHARP | Tomohiro Ikai ikai.tomohiro@sharp.co.jp |
| Toshiba | Tomoo Yamakage tomoo.yamakage@toshiba.co.jp |
| Ericsson | Andrey Norkin andrey.norkin@ericsson.com |

Proponent(s)						Cross-checker(s)

Toshiba (JCTVC-D192)					Ericsson

Panasonic (JCTVC-D217)				Toshiba

Sharp (JCTVC-D115)					Toshiba (2 patterns)

# Subtest 2: Filter adaptation

## Functionality Addressed

- Complexity reduction and coding efficiency enhancement for filter adaptation

## Expected Gains

- Better understanding of trade-off between coding efficiency and unit of filter adaptation adaptive

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Number of operations

- Maximum number of vertical taps

- Possibility of scanning whole frame for encoding and/or decoding

Note: In subtest 2, it is suggested to limit the modification part from the HM 2.0 anchor as small as possible to evaluate the proposed schemes.

No change of chroma processing is allowed for this category.

## Tool Description

### Improving the Adaptive Loop Filter in TMuC0.9 by MediaTek, Qualcomm, and Toshiba (JCTVC-D119, MediaTek/Qualcomm/Toshiba)

Current HM design (QC_ALF) adopts pixel-base adaptation of filter coefficients. This evaluates block-level adaptation of filter coefficients. Also harmonization with other block adaptive schemes (i.e., directional filtering) may be considered.

### Improving the Adaptive Loop Filter in TMuC0.9 by MediaTek, Qualcomm, and Toshiba (JCTVC-D119, MediaTek/Qualcomm/Toshiba)

Segmentation of sets of filter coefficients for pixel/block level adaptation is made based on Filter Unit that consists of one or more of LCUs, which forms a part of the contribution JCTVC-D119. This does not include AO part. Pixel/block level adaptation of filter selection is adaptively turned off block by block to perform Single Filter mode operation.

#### Bitstream organization

Information on Filter Unit size, selection of pixel adaptation mode or one of single filter modes, FU filter set index is added.

### Region-based adaptive loop filter using two-dimensional feature (JCTVC-D116, Sharp)

Region-based adaptive loop filter using two-dimensional feature utilizes two-dimensional feature activity and direction to catch image correlation more properly and up to xxx sets of filter coefficients are sent. On top of this, hierarchical syntax is used to achieve flexible division patterns and relatively small signal amount.

#### Bitstream organization

The signal to decide division pattern(“alf_filter_pattern”) and the prediction method of filter coefficients are extended to accommodate to two-dimensional feature

### Loop filter with directional similarity mapping (DSM) (JCTVC-D221, Samsung)

#### General Concept

This experiment will evaluate filter adaptation (selection) using directions of local gradients, and directional symmetry constraints in the directional filter design, which were proposed in parts of JCTVC-D221. This experiment will perform pixel / block-based directional adaption for directional filters. This experiment does not include the non-linear mapping function. CU synchronous control map will be used to indicate on/off of the filter operations.

#### Bitstream organization

Flag to indicate whether to use loop filtering. Filter coefficients. CU synchronous control map.

## Participants

| Participant | Contact |
| --- | --- |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| Qualcomm | In Suk Chong ichong@qualcomm.com |
| SHARP | Tomohiro Ikai ikai.tomohiro@sharp.co.jp |
| Toshiba | Tomoo Yamakage tomoo.yamakage@toshiba.co.jp |
| Samsung | Wang (PoLin) Lai wlai@sta.samsung.com |

Proponent(s)						Cross-checker(s)

MediaTek/Qualcomm/Toshiba (JCTVC-D119, block)	Samsung, Sharp

MediaTek/Qualcomm/Toshiba (JCTVC-D119, FU)	Panasonic

Sharp (JCTVC-D116)					Qualcomm

Samsung (JCTVC-D221, adaption for directional filter)	Toshiba, Sony

# Subtest 3: Enhancement schemes of loop filtering process

## Functionality Addressed

- Coding efficiency enhancement and/or complexity reduction by new filter operation schemes

## Expected Gains

- Coding efficiency

- Complexity reduction

- Trade-offs between coding efficiency and complexity

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Maximum number of vertical taps

- Number of operations, if a proposal introduces a non-linear feature, such branch operation shall also be reported

Note: In subtest 3, encoding technique shall follow the technique in the anchor as much as possible. If a proposal consists of plural of technical areas, it is requested to report incremental results by each scheme.

No change of chroma processing is allowed for this category.

## Tool Description

### Improving the Adaptive Loop Filter in TMuC0.9 by MediaTek, Qualcomm, and Toshiba (JCTVC-D119, MediaTek/Qualcomm/Toshiba)

This evaluates simplified harmonization of QC_ALF and AO that forms a part of the contribution JCTVC-D119. This does not include FU nor SF part.

### Loop filter with directional similarity mapping (DSM) (JCTVC-D221, Samsung)

#### General Concept

This experiment will evaluate directional filter adaption, directional filters with similarity mapping which includes non-linear enhancement, in parts of JCTVC-D221. This experiment will include the reduction of filter window size as described in Sebtest 5, Section 8.4.3. CU synchronous control map will be used to indicate on/off of the filter operations.

#### Bitstream organization

Flag to indicate whether to use loop filtering. Filter coefficients. CU synchronous control map.

## Participants

| Participant | Contact |
| --- | --- |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| Qualcomm | In Suk Chong ichong@qualcomm.com |
| Samsung | Wang (PoLin) Lai wlai@sta.samsung.com |
| Toshiba | Tomoo Yamakage tomoo.yamakage@toshiba.co.jp |

Proponent(s)						Cross-checker(s)

MediaTek/Qualcomm/Toshiba (JCTVC-D119)		Samsung

Samsung (JCTVC-D221)				MediaTek

# Subtest 4: Adaptation for Low Complexity design

## Functionality Addressed

- Complexity reduction by sets of fixed filters

- Maximum number of vertical taps

## Expected Gains

- Complexity reduction

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Number of sets of filter coefficients (to be test in encoder)

Note: In subtest 4, encoding technique will be different from that in HM 2.0, since designing filter coefficients is performed off-line and needs not be done during the encoding process.

## Tool Description

### Low Complexity Parametric Adaptive Loop Filter (JCTVC-D270, Sony)

Alternative type of Adaptive Loop Filter (ALF) intended to remove coding errors and improve compression efficiency is introduced. In this approach, a set of fixed filters is used instead of the traditional online-trained Weiner filters. The encoder chooses the best (in RD sense) filter for a Coding Unit (CU) and transmits the index of the filter in the bit stream to the decoder. The selection of the best filter at the encoder requires only a single-pass processing for each Large CU (LCU), therefore, reducing the encoder delay significantly compared to traditional ALF approaches which require one frame delay. The classification of the filters is 2D and it is based on direction and bandwidth parameters.

In this CE it is further investigated about the potential of these kind of filters as a set of default fixed ALF filters with lower memory access and delay.

CE will include compression efficiency tradeoffs between filter shapes, computational bit depth etc..

## Participants

| Participant | Contact |
| --- | --- |
| Sony | Ali Tabatabai Ali.Tabatabai@am.sony.com Ehsan Maani Ehsan.Maani@am.sony.com |
| Qualcomm | In Suk Chong ichong@qualcomm.com |
| Mitsubishi | Shun-ichi Sekiguchi Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp |

Proponent(s)						Cross-checker(s)

Sony (JCTVC-D270)					Qualcomm, Mitsubishi

# Subtest 5: Filter size and shape

## Functionality Addressed

- Complexity reduction and improved coding efficiency

- Maximum number of vertical taps

- Maximum number of coefficients

## Expected Gains

- Reduction in worst case computational complexity and/or memory requirements

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Maximum number of vertical taps

- Maximum number of coefficients

## Tool Description

### Adaptive Loop Filtering Using Multiple Filter Shapes (JCTVC-D385, eBrisk Video)

This contribution presents the advantages of utilizing different filter shapes in the Adaptive Loop Filtering part (that is applied to the luminance samples) of the TMuC 0.9 encoder/decoder. More specifically, it was reported that the simultaneous use of two Nx7 shapes yields lower worst-case computational requirements and lower memory requirements while maintaining or increasing the same coding efficiency levels.

### ALF decode with reduced vertical filter size (JCTVC-D039, TI)

JCTVC-D039 reported that vertical size of ALF filter determines the memory bandwidth/line buffer requirements in ALF decode and that reducing the vertical size of the filter leads to reduction in memory bandwidth/line buffer requirements. In this experiment, filters with reduced vertical size (e.g. Nx5) will be investigated to reduce memory bandwidth/line buffer requirements.

### Loop filter with directional similarity mapping (DSM) (JCTVC-D221, Samsung)

#### General Concept

This experiment will evaluate directional filter adaption, directional symmetry constraints in the directional filter design, as parts of JCTVC-D221, with reduction of the filter window size. This experiment does not include the non-linear mapping function. CU synchronous control map will be used to indicate on/off of the filter operations.

#### Bitstream organization

Flag to indicate whether to use loop filtering. Filter coefficients. CU synchronous control map.

## Participants

| Participant | Contact |
| --- | --- |
| eBriskVideo | Faouzi Kossentini faouzi@ebriskvideo.com |
| TI | Madhukar Budagavi madhukar@ti.com |
| Samsung | Wang (PoLin) Lai wlai@sta.samsung.com |

Proponent(s)						Cross-checker(s)

eBrisk Video (JCTVC-D385)				TI

TI (JCTVC-D039)					eBrisk Video

Samsung (JCTVC-D221)				TI, Qualcomm

# Time-line and Responsibilities

T1:	2011-Feb-11:	Final CE description and upload

T2: 2011-Mar-3:	Start cross-checking, proponents to provide software and results to cross-checker(s)

T3:	2011-Mar-4:	Report proponents’ results to Coordinators

T4:	2011-Mar-9:	Report cross-checkers’ results to Coordinators

T5:	2011-Mar-11:	Input document upload

# Previous Contributions

JCTVC-D039 [M. Budagavi, V. Sze, M. Zhou (TI)] ALF decode complexity analysis and reduction

JCTVC-D115 [T. Ikai, T. Yamazaki (Sharp)] CE8: DF-combined adaptive loop filter

JCTVC-D116 [T. Ikai, Y. Yasugi (Sharp)] Region-based adaptive loop filter using two-dimensional feature

JCTVC-D119 [C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, M. Karczewicz, I. S. Chong, T. Yamakage, T. Chujoh, T. Watanabe (MediaTek/Qualcomm/Toshiba)] CE8 Subtest2: A Joint Proposal on Improving the Adaptive Loop Filter in TMuC0.9 by MediaTek, Qualcomm, and Toshiba

JCTVC-D192 [T. Yamakage, S. Asaka, A. Tanizawa, T. Watanabe, T. Chujoh (Toshiba)] Analysis on the interaction between deblocking filtering and in-loop filtering

JCTVC-D217 [S. Esenlik, M. Narroschke, T. Wedi (Panasonic)] CE 8: Results for adaptive loop filter using prediction and residual (3-Input-ALF)

JCTVC-D221 [P. Lai, F. C. Fernandes (Samsung)] Loop filter with directional similarity mapping (DSM)

JCTVC-D270 [E. Maani, W. Liu and L. Dong (Sony)] Low Complexity Parametric Adaptive Loop Filter

JCTVC-D385 [F. Kossentini, N. Mahdi, H. Guermazi, M. A. BenAyed (eBrisk Video)] Adaptive Loop Filtering Using Multiple Filter Shapes

# References

[1]	“Common test conditions and software reference configurations,” JCTVC-D600, Daegu, January 2011.

[2]	“High Efficiency Video Coding (HEVC) Test Model 2 (HM 2) Encoder Description”, JCTVC-D502, Daegu, January 2011.

[3]	“High Efficiency Video Coding (HEVC) text specification Working Draft 2”, JCTVC-D503, Daegu, January 2011.