| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D609 WG11 Number: m19489 |
| --- | --- |

| Title: | Description of Core Experiment 9: MV Coding and Skip/Merge operations |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Tool Experiment Report |  |  |
| Author(s) or Contact(s): | J. Jung 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com |
|  | B. Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin |  | +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
|  | P. Chen Qualcomm 5775 Morehouse Drive San Diego, CA, USA |  | +1 858 845 1347 peisongc@qualcomm.com |
|  | W.-J. Han Samsung Electronics Suwon, Korea |  | +82 10 3329 6393 wjhan.han@samsung.com |
| Source: | CE coordinators |  |  |

____________________________

# Introduction

At the Daegu meeting, some modifications have been applied to the HM design regarding the Inter, the Skip, and the Merge modes. As the major modification, the Direct mode has been removed from the HM design, and a PU Merge mode is now available.

Some input documents were proposed to modify further the set of predictors, and their derivation. This CE has two major goals that are to evaluate these modifications with the new HM design, and try as much as possible to unify the set of predictors and their derivation.

Adaptive motion vector resolution experiments are also part of this CE

# Participants List

| Participant | Person, Email address | P | C |
| --- | --- | --- | --- |
| Canon | P. Onno (patrice.onno@crf.canon.fr) G. Laroche (guillaume.laroche@crf.canon.fr) |  | x |
| HHI | B. Bross (benjamin.bross@hhi.fraunhofer.de) | x | x |
| Institute for Infocomm Research (I2R) | Y.H. Tan (yhtan@i2r.a-star.edu.sg) | x | x |
| LG | J.Y. Park (jy.park@lge.com) S.W. Park (seungwook.park@lge.com) |  |  |
| Mediatek | C.-W. Hsu (cw.hsu@mediatek.com) J.-L. Lin (jl.lin@mediatek.com) Y.-W. Huang (yuwen.huang@mediatek.com) Y.-W. Chen (yiwen.chen@mediatek.com) S. Lei (shawmin.lei@mediatek.com) X. Guo (xun.guo@mediatek.com) |  |  |
| NTT DOCOMO | T.K. Tan (ttk@pacific.net.sg) F. Bossen (bossen@docomolabs-usa.com) Y. Suzuki (suzukiyos@rd.nttdocomo.co.jp) A. Fujibayashi (fujibayashi@nttdocomo.co.jp) |  | X |
| Orange Labs | J. Jung (joelb.jung@orange-ftgroup.com) G. Clare (gordon.clare@orange-ftgroup.com) |  | X |
| Panasonic | T. Sugio (sugio.toshiyasu@jp.panasonic.com) T. Nishi (nishi.takahiro@jp.panasonic.com) | x |  |
| Qualcomm | Peisong Chen (peisongc@qualcomm.com) Wei-jung Chien (wchien@qualcomm.com) Y. Zheng (zhengy@qualcomm.com) L. Guo (liweig@qualcomm.com) | x |  |
| Samsung | W.-J. Han (wjhan.han@samsung.com) I.-K. Kim (ilkoo.kim@samsung.com) |  |  |
| Sejong Univ., Sungkyunkwan Univ., SK Telecom | J. O. Lee (jolee@sju.ac.kr) Joo-Hee Moon (jhmoon@sejong.ac.kr) Haekwang Kim (hkkim@sejong.ac.kr) B. Jeon (bjeon@skku.edu) Jeongyeon Lim (jeongyeon@sktelecom.com) |  |  |
| Sharp | A. Segall (asegall@sharplabs.com) |  |  |
| Sony | K. Sato (Kazushi.Sato@jp.sony.com) |  |  |
| Sungkyunkwan University | B. Jeon (bjeon@skku.edu) J. Yang (binbak1@ece.skku.ac.kr) |  | X |
| Texas Instruments | M. Zhou (zhou@ti.com) | x |  |

# List of experiments and testers

| Experiment | Tester1 | Tester2 |
| --- | --- | --- |
| anchor | X | x |
| A | Panasonic | HHI |
| B | HHI | Orange Labs |
| C | HHI | Panasonic |
| D | HHI | I2R |
| E | TI | NTT DOCOMO |
| F | TI | NTT DOCOMO |
| G | NTT DOCOMO |  |
| H | NTT DOCOMO |  |
| I | Panasonic |  |
| J | Panasonic |  |
| L | I2R |  |
| M01 | Mediatek | Samsung |
| M02 | Mediatek | Samsung |
| M03 | Mediatek | Samsung |
| M04 | Mediatek | Samsung |
| M05 | Mediatek | Samsung |
| M06 | Mediatek | Samsung |
| M07 | Mediatek | LG |
| M08 | Mediatek | LG |
| M09 | Mediatek | LG |
| M10 | Mediatek | LG |
| M11 | Mediatek | LG |
| M12 | Mediatek | LG |
| M13 | Mediatek | Sony |
| M14 | Mediatek | Sony |
| M15 | Mediatek | Sony |
| M16 | Mediatek | Sony |
| M17 | Mediatek | Sony |
| M18 | Mediatek | Sony |
| N | LG | MediaTek |
| O | Qualcomm |  |
| P | HHI | Orange Labs |
| Q | HHI |  |

# Merge/Skip and Inter Motion Vector Derivation

## Related Contributions

### JCTVC-D125 [MediaTek] Improved AMVP

Four changes to AMVP were proposed:

- First, in general cases, six spatial motion vector prediction (MVP) candidates and four temporal MVP candidates are used in the MVP candidate set for motion vector competition (MVC); in low complexity configurations, when the picture order count (POC) difference between the current picture and the co-located picture is equal to 1 and the co-located MV exists, the MVP candidates are reduced to one spatial MVP candidate and four temporal MVP candidates.

- Second, the order of the co-located temporal MVP is modified according to the POC difference.

- Next, when each spatial MVP candidate is derived, a predefined priority is used to select one motion vector (MV) from list 0 and list 1 MVs of the spatial neighboring block, and the chosen MV is scaled to the same reference picture of the current prediction unit (PU) as the spatial MVP candidate for MVC.

- Last, when each temporal MVP candidate is derived, a predefined priority is used to select one MV from list 0 and list 1 MVs of two co-located blocks, and the chosen MV is scaled to the same reference picture of the current PU as the temporal MVP candidate for MVC.

The four improvements can also be applied for the new Skip and Merge that are adopted at the Daegu meeting.

The following tests are proposed.

- Test different numbers of MVP candidates (Section 2.1)

- Test adaptive order of the co-located temporal MVP (Section 2.2)

- Test predefined priority and MV scaling for deriving each spatial MVP candidate (Section 2.3)

- Test predefined priority and MV scaling for deriving each temporal MVP candidate. (Section 2.4)

- Test combination of the above. (Section 2.1 – 2.4 in JCTVC-D125)

### JCTVC-D095 [LG and Qualcomm] Improvements on median motion vectors of AMVP

One changes on spatial neighboring MVs for median vectors in AMVP was proposed.

In AMVP, MVs with the same reference index and same reference list are regarded as spatial neighboring MV. But in the proposed method, MVs with a different reference index or a different reference list are also considered. The first available MV is chosen with pre-determined order, and it is scaled and inverted according to temporal distances and prediction directions. It is then used to calculate median MV predictors.

The proposed method can be also applied to other spatial MV candidates as well.

One experiment in this CE is proposed: test predefined priority and MV scaling for deriving each spatial MVP candidate.

Based on this proposal, tests are also conducted to verify the performance of current skip mode, merge mode, and AMVP mode in HM-2.0, which suggests some directions for unifying the skip, merge, and inter mode in motion vector prediction.

### JCTVC-D055 (TI)

The contribution JCTVC-D055 advocates simplification of upper/left spatial motion vector predictor calculation by using up to two neighboring PUs instead of up to 16 PUs as currently defined in the test model, no coding loss has been reported by this simplification. It is recommended to evaluate the impact of this simplification on the coding efficiency and complexity. In this experiment, the following two simplifications of upper/left spatial motion vector predictor calculation will be evaluated. Only number of PU to be searched will be changed in this evaluation.

Anchor: Current default setting(HM2.0)

In the HM2, as shown in Figure 3.1.3a, the spatial MVP candidates (left, upper) of the current prediction unit (PU) are calculated from the motion vectors of its neighboring PUs, and has the following steps:

Figure 3.1.3a

- MVP candidate from left neighbors:

  - Scanning the left neighbors (PU) from bottom to top (A0 … Am,Am+1; maximum 17 PUs), find the

first PU that meets the following conditions:

    - It is available

    - It is inter-coded

    - It has a MV for the same direction (L0 or L1)

    - It uses the same reference index

  - Otherwise, left predictor is set to (0,0) and marked unavailable.

- MVP candidate from upper neighbors :

  - Do the same procedure with one following additional condition (as that of left predictor) for the upper

neighbors, scanning from right to left (B-1,B0 … Bn,Bn+1, maximum 18 PUs)

    - It is not same to left predictor

Note that m and n can be 0 if the current PU has the minimum vector block size allowed by the HEVC standard.

JCTVC-D055- b : Reduction of search partition in left/top PMV candidate detection (JCTVC-D055)

As shown in Figure 3.1.3b, for the search of left predictor, instead of scanning all the left neighboring PUs from bottom to top, only up to three neighboring PUs, Am+1, Am,A0 are used. Similarly, For the search of top predictor, only up to four neighboring PUs,Bn+1,Bn,B0,B-1 ,are used. Other steps to select MVP candidate are unchanged.

Figure 3.1.3b

JCTVC-D055-c : Further reduction of search partition in left/top PMV candidate detection

As shown in Figure 3.1.3c, left/upper MVP candidate calculation is further simplified. For the search of left predictor, instead of scanning all the left neighboring PUs from bottom to top, only up to two neighboring PUs, Am+1,Am, are used. Similarly, For the search of top predictor, only up to three neighboring PUs, Bn+1,Bn ,B-1　are used. Other steps to select MVP candidate are unchanged.

Figure 3.1.3c

| Experiment | Inter | MERGE | SKIP |  |  |
| --- | --- | --- | --- | --- | --- |
|  | candidates | candidates | Candidates | ref_idx | inter_pred_idc |
| E | JCTVC-D055-b | HM2.0 | JCTVC-D055-b | HM2.0 | HM2.0 |
| F | JCTVC-D055-c | HM2.0 | JCTVC-D055-c | HM2.0 | HM2.0 |

Table X: Summary of the JCTVC-D055 related experiments.

### JCTVC-D274 (Panasonic)

The contribution JCTVC-D274 by Panasonic reports modified derivation process and usage of predicted motion vector for skip mode. It includes 1) uni-prediction for skip mode in GPB frames, 2) enhanced reference index decision for skip mode, 3) enhanced temporal motion vector predictor. It was recommended to evaluate the impact of this modification on the coding efficiency and complexity.

| Experiment | Inter | MERGE |  | SKIP |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | candidates | candidates | inter_pred_idc | Candidates | ref_idx | inter_pred_idc |
| I | HM2.0 | MERGE | JCTVC-D274 | HM2.0 | JCTVC-D274 | JCTVC-D274 |
| J | JCTVC-D274 | MERGE | JCTVC-D274 | JCTVC-D274 | JCTVC-D274 | JCTVC-D274 |

Table X: Summary of the JCTVC-D274 related experiments.

### JCTVC-D051 (I2R)

JCTVC-D051 presented a coding scheme that aimed to simplify the Merge/Skip/Direct mechanisms within the HM1 design by reducing the degrees of freedom of the Skip mode and removing the Direct mode. One characteristic of the proposed design is the possibility of coding a merged CU with a single bin. In this experiment, in view of the adoption of various changes to the Merge/Skip/Direct/Inter modes, it is planned to investigate a simpler design of the remaining Merge/Skip modes using the same approach as in D051.

JCTVC-D165 (Mediatek) CU-level Directional Merge Mode

In this contribution, a directional merge method was proposed to improve the CU-level merge mode. The syntaxes of two merge methods are exactly the same. Their semantics to utilize the motion information from more neighboring blocks is modified to increase the chance of sharing motion parameters from its neighboring blocks. The technique ist o be evaluated with the new PU merge partitioning.

JCTVC-D314 PU merging (HM 2.0 MERGE) with motion vector difference

Additionally to the PU merging index, a motion vector difference can be transmitted. The AMVP is adapted accordingly.

## Experiments related to Merge & Skip motion derivation

Reference of the CE is the HM anchor.

The contribution JCTVC-D441 summarizes the activities of BoG on motion vector coding in CE9. It recommended adopting PU-level merging with AMVP skip on CU-level (3.1.t) with the following Partial Merging restrictions when a CU is split into 2NxN or Nx2N PUs (JCTVC-D233):

- The first PU is inferred to be merged

- The second PU signaling follows 3.1.t (allows either merge for MV coding)

Due to the fact that 3.1.e and 3.1.t from CE9 are very similar, it was recommended to study more in depth their motion derivation process. The difference between 3.1.e and 3.1.t is that in 3.1.t skip is enabled with derivation of the MV from MV Coding, in 3.1.e, there is no skip mode but there is a merge with no residual.

In the BoG activity, it was found that there are three notable differences between two designs:

- Test 3.1.1 Motion vector candidate list

  - JCTVC-D231 in AMVP skip and inter AMVP (HM 2.0 AMVP)

  - JCTVC-D314 in PU merging (HM 2.0 MERGE)

- Test 3.1.2 ref_idx derivation

  - Set to zero in AMVP skip

  - Derived from neighbors in PU merging

  - Derived from neighbors in AMVP skip in JCTVC-D274

- Test 3.1.3 inter_pred_idc derivation

  - Set according to slice type in AMVP skip

  - Derived from neighbors in PU merging

Since some components in two designs are not actually contradictory each other, it was suggested to test all those components by a combinational way.

## Experiments related to Inter AMVP predictors derivation

In HM, there are two ways to construct the motion vector candidate list while they seem to be similar. The possibility of the unification could be beneficial for the consistent design of HM.

- 3.1.4Motion vector candidate list

  - JCTVC-D231 in AMVP skip and inter AMVP (HM 2.0 AMVP)

  - JCTVC-D314 in PU merging (HM 2.0 MERGE)

  - JCTVC-D314 PU merging (MERGE2) with motion vector difference

It should be noted that Test 3.1.4 could be synchronized to Test 3.1.1.as in tests C and D.

| Experiment | Inter | MERGE | SKIP |  |  |
| --- | --- | --- | --- | --- | --- |
|  | candidates | candidates | Candidates | ref_idx | inter_pred_idc |
| anchor | AMVP (HM2.0) | MERGE (HM2.0) | AMVP (HM2.0) | 0 (HM2.0) | slice dependent (HM2.0) |
| A | HM2.0 | HM2.0 | HM2.0 | JCTVC-D274 | HM2.0 |
| B | HM2.0 | HM2.0 | MERGE | derived | derived |
| C | HM2.0 | AMVP | HM2.0 | derived | derived |
| D | MERGE | HM2.0 | MERGE | derived | derived |
| E | JCTVC-D055-b | HM2.0 | JCTVC-D055-b | HM2.0 | HM2.0 |
| F | JCTVC-D055-c | HM2.0 | JCTVC-D055-c | HM2.0 | HM2.0 |
| G | JCTVC-D055-b | HM2.0 | MERGE | derived | derived |
| H | JCTVC-D055-c | HM2.0 | MERGE | derived | derived |
| I | HM2.0 | MERGE | HM2.0 | JCTVC-D274 | JCTVC-D274 |
| J | JCTVC-D274 | MERGE | JCTVC-D274 | JCTVC-D274 | JCTVC-D274 |
| L | HM2.0 | HM2.0 | MERGE (reduced candidate set JCTVC-D051) | derived | derived |
| M01 | JCTVC-D125 Section 2.1 | HM2.0 | JCTVC-D125 Section 2.1 | HM2.0 | HM2.0 |
| M02 | HM2.0 | JCTVC-D125 Section 2.1 | HM2.0 | HM2.0 | HM2.0 |
| M03 | JCTVC-D125 Section 2.1 | JCTVC-D125 Section 2.1 | JCTVC-D125 Section 2.1 | HM2.0 | HM2.0 |
| M04 | JCTVC-D125 Section 2.2 | HM2.0 | JCTVC-D125 Section 2.2 | HM2.0 | HM2.0 |
| M05 | HM2.0 | JCTVC-D125 Section 2.2 | HM2.0 | HM2.0 | HM2.0 |
| M06 | JCTVC-D125 Section 2.2 | JCTVC-D125 Section 2.2 | JCTVC-D125 Section 2.2 | HM2.0 | HM2.0 |
| M07 | JCTVC-D125 Section 2.3 | HM2.0 | JCTVC-D125 Section 2.3 | HM2.0 | HM2.0 |
| M08 | HM2.0 | JCTVC-D125 Section 2.3 | HM2.0 | HM2.0 | HM2.0 |
| M09 | JCTVC-D125 Section 2.3 | JCTVC-D125 Section 2.3 | JCTVC-D125 Section 2.3 | HM2.0 | HM2.0 |
| M10 | JCTVC-D125 Section 2.4 | HM2.0 | JCTVC-D125 Section 2.4 | HM2.0 | HM2.0 |
| M11 | HM2.0 | JCTVC-D125 Section 2.4 | HM2.0 | HM2.0 | HM2.0 |
| M12 | JCTVC-D125 Section 2.4 | JCTVC-D125 Section 2.4 | JCTVC-D125 Section 2.4 | HM2.0 | HM2.0 |
| M13 | JCTVC-D125 Section 2.1, 2.2, 2.3, 2.4 | HM2.0 | JCTVC-D125 Section 2.1, 2.2, 2.3, 2.4 | HM2.0 | HM2.0 |
| M14 | HM2.0 | JCTVC-D125 Section 2.1, 2.2, 2.3, 2.4 | HM2.0 | HM2.0 | HM2.0 |
| M15 | JCTVC-D125 Section 2.1, 2.2, 2.3, 2.4 | JCTVC-D125 Section 2.1, 2.2, 2.3, 2.4 | JCTVC-D125 Section 2.1, 2.2, 2.3, 2.4 | HM2.0 | HM2.0 |
| M16 | HM2.0 | JCTVC-D165 | HM2.0 | HM2.0 | HM2.0 |
| M17 | HM2.0 | HM2.0 | JCTVC-D165 | HM2.0 | HM2.0 |
| M18 | HM2.0 | JCTVC-D165 | JCTVC-D165 | HM2.0 | HM2.0 |
| N | JCTVC-D095 | HM2.0 | JCTVC-D095 | HM2.0 | HM2.0 |
| O | JCTVC-D095 | JCTVC-D095 | JCTVC-D095 | Derived | derived |
| P | MERGE2 | HM2.0 | MERGE | derived | derived |
| Q | MERGE2 | AMVP | HM2.0 | derived | derived |

Table 1: Summary of the experiments.

Softwares availability:

Experiment I: 23rd of February.

Adaptive MV Resolution

JCTVC-D394 (Qualcomm)

The contribution JCTVC-D394 by Qualcomm reports an adaptive motion vector resolution method (AMVRES). In the proposed AMVRES, the motion accuracy can be adaptively selected to be 1/4th pixel or 1/8th pixel for each motion vector. It was recommended to evaluate the impact of this modification on the coding efficiency and complexity.

JCTVC-A113 (Sejong Univ., Sungkyunkwan Univ., SK telecom)

In this technique, the motion vector accuracy is adaptively chosen between 1/4 and 1/8 pel. The motion vector resolution is signaled as a flag in coding unit (CU) or a prediction unit (PU). The flag in a CU-level signals a choice among 1/4-pel, 1/8-pel, or PU-level signaling. The selective flag for a PU-level signals a choice between 1/4-pel and 1/8-pel.

Test Sequences / Test Points / SW Configuration

Simulation results will be generated for sequences and coding conditions specified in the document JCTVC-D600. Latest version of the HM (HM 2.0), available on February 11th will be used for evaluation.

# Definition of Performance Measurement Criteria

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BDRate).

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the default configuration)

- Decoding time (compared to the default configuration)

# Time-line

| T1 | 2011-February 11 | Final CE description and description of proposals HM2.0 available |
| --- | --- | --- |
| T2 | 2011-February 23 | Completion of tool integration, test configuration announcement Start of cross-check |
| T3 | 2011-March 9 | Cross-check complete Reporting of all results to CE9 coordinators complete |
| T4 | Documents registration deadline | Input documents uploaded |
| T5 | Documents registration deadline + 1 day | CE9 summary report uploaded |