| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D610 WG11 Document: m19491 |
| --- | --- |

| Title: | CE10: Core transform design |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by JCT-VC |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala FastVDO LLC Madhukar Budagavi Texas Instruments Arild Fuldseth Cisco Rajan Joshi Qualcomm Il-Koo Kim Samsung Electronics | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1-443-455-0394 pankajtva@yahoo.com madhukar@ti.com arilfuld@cisco.com rajanj@qualcomm.com ilkoo.kim@samsung.com |
| Source: | FastVDO LLC, Texas Instruments, Cisco, Qualcomm, Samsung Electronics |  |  |

_____________________________

# Objective

This CE shall investigate the design of core spatial transforms, and study the tradeoff between coding efficiency and complexity, return evidence and a recommendation for design selection. The anchor for comparison shall be the transforms in the HM. Proposals that use dynamic range control methods (such as clipping) should incorporate them into the complexity analyses below.

# Participant list

| Name | Affiliation | Email |
| --- | --- | --- |
| Pankaj Topiwala | FastVDO LLC | pankajtva@yahoo.com |
| Madhukar Budagavi | Texas Instruments | madhukar@ti.com |
| Arild Fuldseth | Cisco | arilfuld@cisco.com |
| Rajan Joshi | Qualcomm | rajanj@qualcomm.com |
| Il-Koo Kim | Samsung Electronics | ilkoo.kim@samsung.com |
| Dzung Hoang | Zenverge | Dzung.Hoang@Zenverge.com |
| David Flynn | BBC | davidf@rd.bbc.co.uk |
| Louie Kerofsky | Sharp | lkerofsky@sharplabs.com |
| Jian Lou | Motorola | lou@motorola.com |

# Description of tools

JCTVC-D036: Matrix multiplication specification for HEVC transforms (Texas Instruments)

This contribution is a continuation of prior work in JCTVC-C226 and presents more details and software optimization results. JCTVC-C226 proposed the use of matrix multiplication to specify HEVC transforms while eliminating the existing norm correction quantization matrices in HM. In this contribution, DCT/IDCT was implemented using even-odd decomposition optimization (which uses a partial butterfly structure).

JCTVC-D224: Unified transform design for HEVC with 16 bit intermediate data representation (Cisco)

The proposed tool uses a set of unified transforms covering all block sizes from 4x4 to 32x32. The unified transforms have the following properties; 16 bit data representation before and after each transform stage, no need for correction of different norms of basis vectors during quantization/dequantization, all transform sizes above 4x4 can reuse arithmetic operations for smaller transform sizes, and implementations can use both pure matrix multiplication and a combination of matrix multiplication and butterfly.

JCTVC-D256: Efficient 16 and 32-point transforms (Qualcomm)

16 and 32-point transforms were proposed in D256. The proposed transforms are scaled orthogonal integer transforms and support a recursive factorization structure. The proposed 16 and 32-point transforms use 36 and 92 multiplications, respectively, compared to 44 and 116 multiplications for the corresponding transforms in the existing test model. A multiplier-less implementation of these transforms is also provided. In this core experiment, the design will be extended to 4-point and 8-point transforms. The use of clipping to limit the dynamic range at certain points in the inverse transform stages will also be explored.

JCTVC-D339: Fast Integer Transforms for the HM (FastVDO)

The current HM utilizes integer transforms of sizes 4-pt to 32-pt. 16-pt and 32-pt transforms are designed using Chen's factorization of the DCT. JCTVC-D339 highlights the introduction of integer transforms based on Loeffler’s factorization..  The current 4-pt and 8-pt transforms are initially retained, while the more complex 16-pt and 32-pt transforms are systematically simplified, even including lifting-based implementation and multiplication-free designs. These designs provide identical performance to the designs with multiplications.

JCTVC-D365: Fast Integer Transforms for the HM, and Complexity Analysis (FastVDO/Samsung)

The current HM (TMuC-0.9) utilizes integer transforms of sizes 4-pt to 32-pt, designed using Chen's factorization of the DCT.  This contribution summarizes the performance and the complexity of the existing transform cores and introduces new integer transforms and one variant using lifting.  The current 4-pt and 8-pt transforms are initially retained, while the more complex 16-pt and 32-pt transforms are systematically simplified, including multiplication-free designs. The multiplication-free designs afford exact invertibility in limited bitdepth arithmetic, yet have identical performance. In addition, we have developed a simplified approach to quantization matrices – just replacing them with scalar values.  This can be done for both the existing HM transforms, as well as the new transforms in our design.

# Core Experiment Tasks

| Proponents | Cross-checkers |
| --- | --- |
| Cisco/TI |  |
| FastVDO/Samsung | Motorola |
| Qualcomm |  |

B.1. Coding efficiency

B.1.1. Measure the RD performance of candidate transform designs using common conditions D600 and HM 2.0 software. If a transform design uses clipping etc. on intermediate variables, this clipping should be included in coding efficiency tests.

B.1.2. Also test at low and high QP values.   Provide BD-Rate results for the following QP sets: 1, 5, 9, 13 and 36, 42, 47, 51 to see if there is unusual behavior at these uncommon test points.  Run this test on 2 sec of video to speed up this test. Number of frames to encode is provided in the table below.

|  |  | Number of frames to encode |
| --- | --- | --- |
| S01 | Traffic | 65 |
| S02 | PeopleOnStreet | 65 |
| S03 | Kimono | 49 |
| S04 | ParkScene | 49 |
| S05 | Cactus | 97 |
| S06 | BasketballDrive | 97 |
| S07 | BQTerrace | 129 |
| S08 | BasketballDrill | 97 |
| S09 | BQMall | 12 |
| S10 | PartyScene | 97 |
| S11 | RaceHorses (C) | 65 |
| S12 | BasketballPass | 97 |
| S13 | BQSquare | 129 |
| S14 | BlowingBubbles | 97 |
| S15 | RaceHorses (D) | 65 |
| S16 | Vidyo1 | 129 |
| S17 | Vidyo3 | 129 |
| S18 | Vidyo4 | 129 |

B.2. Complexity

Analyze the complexity of candidate transform designs using the following parameters:

- Arithmetic operations

- Software execution speed

- Dynamic range

- Parallelizability on SIMD

- Number of sequential operations

- HW

Proponents are encouraged to provide additional complexity analyses voluntarily.

Discussion:

Complexity is multidimensional, and it is important to capture the essence of this space, but at the same time, keep the analysis space limited in size. A number of complexity metrics are listed below, and it was the sense of the group to do more, rather than less, testing.  These tests are constructed to explore suitability of designs in both software and hardware.  Some of these tests are "paper" tests, while others require actual coding and optimization. At this time, some tests are considered required, while others are optional. This is based on the fact that some tests require optimization, in software or hardware, which it may be unreasonable to require proponents to produce.  Nevertheless, it is the intent of the group to conduct even the optional tests, by recruiting volunteers. It is recognized that we are dealing with an exceptionally short meeting cycle (6 weeks), which makes completing all tasks in one meeting cycle challenging.  It is hoped to collect all relevant evidence in at most two meeting cycles.

B.2.1. Arithmetic Operations

Report the number of arithmetic operations needed for implementing the 2D inverse transform, in terms of multiplications, additions, shifts, compares. If the design allows flexibility in the way it is implemented, report the range of arithmetic operations available (for example, methods with and without multiplications).  Include in the count TPE and final rounding if used.  [Paper study]

B.2.2. Software Execution Speed

Develop a software model of candidate (transform + quantization modules) in single-thread C++ compatible with current HM software.  Develop a common test environment for testing all candidates.  In software tests below, run the tests N times, for N sufficiently large to smooth variations (e.g., N = 100K). Report total execution times.

B.2.2.1. IBDI Off

Develop a test input frame of a random noise (9-bit, [-255, 255]), and run N trials of the modules:

[forward transform, quantization, dequantization, inverse transform].

B.2.2.2. IBDI On

Develop a test input frame of a random noise (11-bit, [-1020, 1020]), and run N trials of the modules:

[forward transform, quantization, dequantization, inverse transform].

(TI/Cisco volunteer to develop a common test environment.)

B.2.3. Dynamic Range Analysis

B.2.3.1. (Optional) Report if the candidate transform design is exactly invertible in fixed-point arithmetic, and if so, at what bitdepth.    [Paper study]

B.2.3.2. In the software module developed for B.2.2., test the dynamic range of the data at four points: prior to dequantization, prior to inverse transform, after first-stage inverse transform, and after second-stage inverse transform.  Example input may be data consisting of +/- 255, either random, or Hadamard-like. (TBD)

B.2.4. Parallelizability on SIMD

Develop an optimized implementation for a target SIMD architecture (e.g., Intel Core i7 platform) for candidate designs [forward transform, quantization, dequantization, inverse transform]. Combine source codes into one environment (machine, OS, compiler, SW package) for comparative testing.

Volunteers:

David Flynn (BBC) volunteers to conduct this test - which is greatly appreciated by the group.  Recommend that this be done in a common environment so anyone can test, crosscheck.

B.2.5. Number of sequential operations

Report the number of cascaded operators (multipliers/adders) in the longest execution path.  [Paper study]

B.2.6. HW [Optional]

B.2.6.1. Develop optimized hardware models (RTL) of candidate transform designs, and determine the silicon area required, where optimization is for a common hardware condition.  Details to be determined.

B.2.6.2. Determine the hardware speed.  From area and speed, power consumption can be estimated. This subtask is intended to be a paper study, given B.2.6.1.

TBD: specific tasks, common hardware conditions, distribution of task responsibilities.

Remarks:

TI / Samsung plan to implement three proposals (Cisco/TI, Samsung/FastVDO, Qualcomm) in hardware, and cross-verify, for this test.

# Timeline

Feb. 11, 2011: HM 2.0 software expected to be available.

Feb. 14, 2011: Final CE10 description uploaded

Feb. 18, 2011: Common test framework for stand-alone testing in B.2.2 and B.2.3 will be made available.

Feb. 23, 2011: Cross-verification begins by this date

Mar. 10, 2011: Reporting of all results to CExx coordinators complete

Mar. 10, 2011: Input documents uploaded