| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D611 WG11 Number: m19499 |
| --- | --- |

| Title: | Description of Core Experiment (CE11): Coefficient scanning and coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Vivienne Sze (coordinator) Texas Instruments Krit Panusopone Motorola Mobility Jianle Chen Samsung Tung Nguyen Fraunhofer HHI Muhammed Coban Qualcomm | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1-214-567-4005 sze@ti.com +1-858-404-3618 krit@motorola.com +82-31-279-8826 jianle.chen@samsung.com +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de +1-858-658-3973 mcoban@qualcomm.com |
| Source: | Texas Instruments, Motorola Mobility, Samsung, Fraunhofer HHI, Qualcomm |  |  |

_____________________________

# Introduction

Various methods have been proposed for coding the transform coefficients of the residual to reduce complexity and/or improve coding efficiency. This core experiment evaluates the coding efficiency and complexity impact of

- Alternative binarization and context modeling/selection for syntax elements related to transform coefficients. (i.e. significiant_coeff_flag, last_signifiicant_coeff_flag, and abs_coeff_level_minus1)

- transform coefficient scanning order methods

The objectives of this CE include:

- Investigation of the impact of alternative binarization and context selection schemes for transform coefficient coding on the coding efficiency and complexity

- Coding efficiency and complexity measurement of transform coefficient scanning order method when combining with context selection

Complexity measures are described in Section 6.2

# Participants List

| Company | Participants |
| --- | --- |
| TI | Vivienne Sze, sze@ti.com Madhukar Budagavi, madhukar@ti.com |
| HHI | Tung Nguyen, tung.nguyen@hhi.fraunhofer.de Heiko Schwarz, heiko.schwarz@hhi.fraunhofer.de |
| Panasonic | Hisao Sasai, sasai.hisao@jp.panasonic.com |
| Qualcomm | Muhammed Coban, mcoban@qualcomm.com Joel Sole, joels@qualcomm.com Yunfei Zheng, zhengy@qualcomm.com |
| Samsung | Jianle Chen, jianle.chen@samsung.com Vadim Seregin, V.seregin@samsung.com |
| Huawei | Jin Song, andy.songjin@huawei.com Haitao Yang, haitao.yang@huawei.com Haoping Yu, haopingyu@huawei.com |
| RIM | Dake He, dhe@rim.com |
| Sony | Cheung Auyeung, Cheung.Auyeung@am.sony.com |
| DOCOMO USA Labs | Frank Bossen, bossen@docomolabs-usa.com |
| Sharp | Andrew Segall, asegall@sharplabs.com Yukinobu Yasugi, yasugi.yukinobu@sharp.co.jp Tomoyuki Yamamoto, yamamoto.tomoyuki@sharp.co.jp |
| Motorola Mobility | Krit Panusopone, krit@motorola.com Jian Lou, lou@motorola.com |

`

# Tools under Test

The following tools will be considered.

| Proponent | Document | Tool description | Cross-checker |
| --- | --- | --- | --- |
| HHI | JCTVC-D336 | Reduced-complexity entropy coding of transform coefficient levels using a combination of VLC and CABAC/PIPE | RIM, TI, DOCOMO |
| Panasonic | JCTVC-D185 | Simplified Context modeling for Transform Coefficient Coding | Qualcomm |
| Qualcomm | JCTVC-D262 | Parallel Context Processing for the significance map in high coding efficiency | Panasonic, Sony |
| Samsung | JCTVC-D360 | Low-complexity adaptive coefficients scanning | Huawei |
| Huawei | JCTVC-D311 | Adaptive coefficients scanning for inter-frame coding | HHI |
| Qualcomm | JCTVC-D374 | Adaptive Coefficient Scanning for LCEC | Motorola, Sharp |

## Sub Experiment CE.A: Context Modeling/Selection and Binarization for Transform Coefficient related syntax elements

### JCTVC-D336: "Reduced-complexity entropy coding of transform coefficient levels using a combination of VLC and CABAC/PIPE ", HHI

JCTVC-D336 proposes a reduced-complexity method for the coding of absolute transform coefficient levels by increasing the number of bins coded in bypass mode of CABAC/PIPE. The main underlying idea of this proposal is to allow the mixing of structured VLCs and PIPE/CABAC coded bits. Compared to the current method implemented in HM 1.0, the same coding efficiency is achieved while the computational complexity is reduced, especially for the high bitrate case. Also, the upper limit on the number of bins to be parsed by PIPE/CABAC can be reduced by at least a factor of 3 compared to the current method.

### JCTVC-D185: " Simplified Context modeling for Transform Coefficient Coding ", Panasonic

JCTVC-D185 proposes that a simplified context modeling for significant map and last flag. With the simplification of the context model, some context is shared with several transform block sizes and neighborhood of coefficient position. This core experiment will verify the coding efficiency of the simplification of context model and complexity impact.

### JCTVC-D262: "Parallel Context Processing for the significance map in high coding efficiency", Qualcomm

JCTVC-D262 proposes a technique for the parallelization of context processing. The method separates syntax elements for parallel processing and simplifies contexts. Significance coefficient flag and last significant coefficient flag are separated by encoding the position of the last significant coefficient before the position of the other significant coefficients within a block.

## Sub Experiment CE.B: Coefficient Scanning Methods

### JCTVC-D360: "Low-complexity adaptive coefficients scanning", Samsung

JCTVC-D360 proposes a technique for adaptive coefficients scanning. The method applies mode dependent LUT table to derive scanning pattern for intra block. Explicit signaling is used to indicate scanning patterns for inter blocks.

### JCTVC-D311: "Adaptive coefficients scanning for inter-frame coding", Huawei

JCTVC-D311 proposes an adaptive scanning method for inter blocks. Three scanning orders are used, and for every Transform Unit (TU) the best scanning order is chosen based on the texture direction of reference block. The encoder and decoder follow the same rules to decide the best scanning order, so explicit signaling is avoided.

### JCTVC-D374: “Adaptive Coefficient Scanning for LCEC”, Qualcomm

In adaptive scans three scans are used: horizontal, vertical and zigzag. The scan order for first few coefficients (up to 64) is adaptively adjusted based on previously coded coefficients (as described in the document JCTVC-C250). For large blocks with a size bigger than 16x16, they share the same scan with 16x16 block.

# Software and Configuration

## Software

The CE will use the HM 2.0 software that is recommended by the Software Ad Hoc group.

## Test Sequences, Bit Rates and Coding Conditions

The CE will use the test sequences, configuration and conditions that are recommended by the Software Ad Hoc group.

# Time-line and Responsibilities

HM2.0 software available Feb 11, 2011

[SOM=Start of next meeting, numbers are counting weeks]

- Feb. 11, 2011:	Final CE-description

- SOM-3 (Feb. 23, 2011)	Completion of tool integration, start of testing

- SOM-3 (Feb. 23, 2011):	Cross-check

- SOM-1 (Mar. 9, 2011):	Complete cross-check; Input Document upload

# Definition of Performance Measurement Criteria

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are considered to be used in this core experiment.

## Coding Performance Measurements

Objective rate-distortion measurements using 4-point BD-PSNR and BD-Rate [1] for all tools in section 3. Proposals in sub experiment CE. B relating to coefficient scanning methods should be tested with both high efficiency and low complexity configurations.

## Complexity Measurements

For the complexity measurement, the anchor, the reference software and the reference software with the tool implemented will be executed on the same machine and the computational time will be measured. A time factor will then be calculated between the reference software and the reference software with the tool implemented.

However, the complexity analysis does not solely rely on the encoding and decoding time measurements of a particular implementation (reference software) of the working draft. The impact on optimized implementations in hardware and software are taken into account.

Core experiment participants who investigate parallelization aspects are requested to provide a description showing how the parallelization is improved and which architecture(s) would see the benefit of the parallelization. Average and worst case throughput gain from parallelized methods should be considered.

As the number of bin impact overall throughput, the following needs to be reported

- the total number of bins and bin reduction/increase due to proposed technique

- a breakdown of the number of bins coded in regular and bypass mode for CABAC

- a breakdown of the number of bins for sig flag, last flag and absolute coefficient level

The size of the context memory (i.e. number of contexts used) should be reported.

In addition, for sub experiment CE.B

Scheme of selecting the scan pattern during encoding needs to be described.

Scheme of updating the scan pattern during parsing needs to be described.

# References

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.